3GPP TSG-RAN WG3 Meeting #116-e	R3-223414

# 9-19 May 2022

# Online

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0803 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ASN.1 Correction on RAT Restriction Information and Extended RAT Restriction Information for TS 38.413 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current NGAP specification, the explicit access restriction for all types of NR RATs based on satellite constellations, i.e. NR(LEO), NR(MEO), NR(GEO) and NR(OTHERSAT) have been introduced in the IEs RAT Restriction Information and Primary RAT Restriction, while in the corresponding ASN.1, the above explicit access restriction has not reflected. Similar issue is found in the IE Secondary RAT Restriction. RATRestrictionInformation ::= BIT STRING (SIZE(8, ...)) ExtendedRATRestrictionInformation ::= SEQUENCE { primaryRATRestriction BIT STRING (SIZE(8, ...)), secondaryRATRestriction BIT STRING (SIZE(8, ...)), iE-Extensions ProtocolExtensionContainer { {ExtendedRATRestrictionInformation-ExtIEs} } OPTIONAL, ... } |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the access restrictions explicitly in the IEs Primary RAT Restriction and Secondary RAT Restriction in the ASN.1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ASN.1 for IEs RAT Restriction Information and Extended RAT Restriction Information is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>> START OF CHANGE <<<

ExtendedRATRestrictionInformation ::= SEQUENCE {

primaryRATRestriction		BIT STRING (e-UTRA (0),nR (1), nR-unlicensed (2), nR-LEO (3), nR-MEO (4), nR-GEO (5), nR-OTHERSAT (6)} SIZE(8, ...)),

secondaryRATRestriction		BIT STRING (e-UTRA (0),nR (1), e-UTRA-unlicensed (2), nR-unlicensed (3)} SIZE(8, ...)),

iE-Extensions		ProtocolExtensionContainer { {ExtendedRATRestrictionInformation-ExtIEs} }	OPTIONAL,

...

}

ExtendedRATRestrictionInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

>>>NEXT CHANGE <<<

RATRestrictionInformation ::= BIT STRING (e-UTRA (0),nR (1), nR-unlicensed (2), nR-LEO (3), nR-MEO (4), nR-GEO (5), nR-OTHERSAT (6)} SIZE(8, ...))

>>> END OF CHANGE <<<