3GPP TSG-RAN WG3 Meeting # 116-e	R3-223425

# Online,  May 9th 2022 - May 19th 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | xxxx | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 38.401 corrections for CCO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Qualcomm Incorporated, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Section 7.9.1 (General) does not clearly indicate what a CCO issue is and that CCO function can be used to resolve (in addition to mitigate) a CCO issue. In section 7.9.3 (Dynamic coverage configuration changes): - It is not clear that a gNB-CU action is done to mitigate or resolve a CCO issue. - there is a reference to CCO configuration, which is not a clear term - the term “SSB” is missing in relation to “beams” - it is not clear that gNB-DU can send per SSB-beam coverage state to gNB-CU |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some text is added to section 7.9.1 (General) to introduce the term “CCO issue” and to indicate that CCO function can be used to resolve (in addition to mitigate) a CCO issue. In section 15.5.5.3 (Dynamic coverage configuration changes): - a clarification is added to indicate that the action of gNB-CU is to mitigate or resolve a CCO issue - the reference to CCO configuration is replaced with “coverage configuration”, a term already used in the paragraph - the term “SSB” is added in relation to “beams” - it is added that gNB-DU can send per SSB-beam coverage state to gNB-CU Impact Analysis The CR has limited impact on the previous version of the specification. No ASN.1 impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear what a CCO issue is and that CCO function can be used to resolve it. It is also unclear what a CCO configuration is. It is not said that gNB-DU can send SSB beam coverage state to gNB-CU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.9.1, 7.9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------  Start of Changes ---------------------------------------------------------

## 7.9	Support for CCO

### 7.9.1		General

The NR Capacity and Coverage Optimization (CCO) Function in non-split gNB case is specified in TS 38.300 [2]. The objective of this function is to detect and resolve or mitigate CCO issues, e.g. coverage and cell edge interference issues.

### 7.9.2	OAM requirements

Each gNB-DU may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contain relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted.

### 7.9.3	Dynamic coverage configuration changes

In case of split gNB architecture, CCO detection function is located at the gNB-CU. The gNB-CU, to mitigate or resolve a CCO issue, signals to the gNB-DU the CCO issue and the affected cells and SSB beams. The gNB-DU resolves or mitigates the CCO issue concerning own served cells by local action within the OAM configured limits. The gNB-DU may also take into account information received for other cells when adopting the modified coverage configuration. The gNB-DU informs the gNB-CU of the new cell coverage states adopted and optionally of the new SSB beams coverage states adopted.

-----------------------------------------------  End of Changes ---------------------------------------------------------