3GPP TSG-RAN WG3 Meeting # 116-e	R3-223426

# Online,  May 9th 2022 - May 19th 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0805 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NGAP corrections for Inter-System Load Balancing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The semantic description of NR Composite Available Capacity IE refers to 38.423, whilst the following agreement, taken at RAN#114-bis-e: Encode CAC as in LTE in both directions, and no further discussion in R17. indicates to use encoding according to 36.423. The IE description for NR Composite Available Capacity is not aligned with the tabular content and refers to SSB area. The IE description for Inter-system Resource Status Reply IE does not refer to inter-system load reporting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The reference to 38.423 is replaced with a reference to 36.423 in the semantic description of NR Composite Available Capacity IE. The IE description for NR Composite Available Capacity is aligned with the tabular content, and the reference to SSB area is removed. Reference to inter-system load reporting is added to the IE description for Inter-system Resource Status Reply IE. Impact Analysis The CR has limited impact on the previous version of the specification. No ASN.1 impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IE description and the semantic description for NR Composite Available Capacity IE are not accurate. The IE description for Inter-system Resource Status Reply IE is not accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.3.60, 9.3.3.61, 9.3.1.201 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------  Start of Changes ---------------------------------------------------------

#### 9.3.3.60	Inter-system Resource Status Report

This IE contains the Inter-system load report.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting System |  |  |  |  |
| >E-UTRAN |  |  |  |  |
| >>E-UTRAN Cell To Report List |  | 1 |  |  |
| >>>E-UTRAN Cell To Report Item |  | 1..<maxnoofReportedCells> |  |  |
| >>>>Cell ID | M |  | E-UTRA CGI 9.3.1.9 |  |
| >>>>E-UTRAN Composite Available Capacity Group | M |  | 9.3.1.196 | Corresponds to the Composite Available Capacity Group IE as defined in TS 36.423 [40]. |
| >>>>Number of Active UEs | O |  | INTEGER (0..16777215, …) | Corresponds to the Number of Active UEs IE as defined in TS 36.314 [49]. |
| >>>>RRC Connections | O |  | INTEGER (1..65536, …) | Corresponds to the RRC Connections IE as defined in TS 38.423 [24]. |
| >>>>E-UTRAN Radio Resource Status | O |  | 9.3.1.200 | Corresponds to the Radio Resource Status IE as defined in TS 36.423 [40]. |
| >NG-RAN |  |  |  |  |
| >>NG-RAN Cell To Report List |  | 1 |  |  |
| >>>NG-RAN Cell To Report Item |  | 1..<maxnoofReportedCells> |  |  |
| >>>>Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |
| >>>>NR Composite Available Capacity Group | M |  | 9.3.1.201 | Corresponds to the Composite Available Capacity Group IE as defined in TS 36.423 [40]. |
| >>>>Number of Active UEs | O |  | INTEGER (0..16777215, …) | Corresponds to the Number of Active UEs IE as defined in TS 38.423 [24]. |
| >>>>RRC Connections | O |  | INTEGER (1..65536, …) | Corresponds to the RRC Connections IE as defined in TS 38.423 [24]. |
| >>>>NR Radio Resource Status | O |  | 9.3.1.205 | Corresponds to the DL/UL GBR PRB usage for MIMO IE, DL/UL non-GBR PRB usage for MIMO IE and DL/UL Total PRB usage for MIMO IE, as defined in TS 38.423 [24]. |

| Range bound | Explanation |
| --- | --- |
| maxnoofReportedCells | Maximum no. of cells that can be reported. Value is 256. |

-----------------------------------------------  Next Change ---------------------------------------------------------

#### 9.3.3.61	Inter-system Resource Status Reply

This IE indicates for which cell(s) the inter-system load reporting was successfully initiated.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Reporting System |  |  |  |  |
| >E-UTRAN |  |  |  |  |
| >>E-UTRAN Cell To Report List |  | 1 |  |  |
| >>>E-UTRAN Cell To Report Item |  | 1..<maxnoofReportedCells> |  |  |
| >>>>Cell ID | M |  | E-UTRA CGI 9.3.1.9 |  |
| >NG-RAN |  |  |  |  |
| >>NG-RAN Cell To Report List |  | 1 |  |  |
| >>>NG-RAN Cell To Report Item |  | 1..<maxnoofReportedCells> |  |  |
| >>>>Cell ID | M |  | NG-RAN CGI 9.3.1.73 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReportedCells | Maximum no. of cells that can be reported. Value is 256. |

-----------------------------------------------  Next Change ---------------------------------------------------------

#### 9.3.1.201	NR Composite Available Capacity Group

This IE indicates the overall available resource level per cell in the cell in downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Composite Available Capacity Downlink | M |  | NR Composite Available Capacity 9.3.1.202 | For the downlink |
| Composite Available Capacity Uplink | M |  | NR Composite Available Capacity 9.3.1.202 | For the uplink |

-----------------------------------------------  End of Changes ---------------------------------------------------------