3GPP TSG-RAN WG3 Meeting #116-e															R3-223482

E-meeting, 09 May – 19 May 2022

Title: 	Corrections to MRO for mobility enhancement

Source: 	Huawei, CMCC, Qualcomm

Agenda item:	9.1.1.1

Document for:	Discussion

# Introduction

This paper contains a set of corrections for MRO for mobility enhancements.

- Discussion

## DAPS HO in stage2

The case “if DAPS HO is configured but an RLF is detected in the source cell with successful DAPS HO” is classified as Intra-system Too Late Handover.

But in RAN2, The case “if DAPS HO is configured but an RLF is detected in the source cell with successful DAPS HO” is classified into SHR. The UE will not record the RLF report for this case. Therefore, it is not needed to consider the detection mechanism for this case.

Remove this case from the detection mechanism for intra-system Too Late Handover.

This can instead be added to the sections about succesful handover report. This sections does however not exist in 38.300, and may be discussed by RAN2 at their next meeting. The author would volounteer to provide such a CR already now if the group prefers.

## CHO Candidate Cell

For the CHO Candidate Cell, the IE CHO Candidate Cell ID includes the GCI while the MeasOject Container includes the frequency and PCI. The CHO Candidate Cell info is used for the source cell to identify the CHO candidate cell. It seems not meaning to include both of the above info.

Delete the MeasObject Container and update the related ASN.1 part.

## ReportConfg Container

The ReportConfig Container refers to the ReportConfigToAddMob, which actually includes the ReportConfigId and all the possible ReportConfig IEs as follows.

ReportConfigToAddMod ::=            SEQUENCE {

reportConfigId                      ReportConfigId,

reportConfig                        CHOICE {

reportConfigNR                      ReportConfigNR,

...,

reportConfigInterRAT                ReportConfigInterRAT,

reportConfigNR-SL-r16               ReportConfigNR-SL-r16

}

}

For the MeasObjectId, this is not needed. Besides, CHO is only applied for intra-NR mobility. Therefore, only reportConfigNR should be included in the ReportObject Container.

Modify the semantics description of ReportConfig Container to ReportConfigNR.

# Conclusion

We propose to agree to the above proposals. The corresponding changes can be found in the related CRs.