3GPP TSG-RAN WG3 Meeting #116-e	R3-223497

Electronic Meeting, May 9th – 19th, 2022

Agenda item:	9.1.5.1

Source: 	Intel Corporation

Title: 	Correction for positioning measurement during INACTIVE

Document for:	Discussion and Decision

# Introduction

For positioning measurement during INACTIVE, RAN2 defined the SRS-PosRRC-InactiveConfig-r17 IE in RRC spec [1] that is configured by suspend configuration via RRCRelease message. To make the positioning measurement during INACTIVE work flawlessly, we believe that F1AP change is inevitable.

In this contribution, we discuss this issue in more depth and propose to correct F1AP [2] to complete Rel-17 Positioning enhancement for INACTIVE.

# Discussion

For positioning measurement during INACTIVE, RAN2 defined the SRS-PosRRC-InactiveConfig-r17 IE in RRC spec [1] that is configured by suspend configuration via RRCRelease message:

SuspendConfig ::=                   SEQUENCE {

fullI-RNTI                          I-RNTI-Value,

shortI-RNTI                         ShortI-RNTI-Value,

ran-PagingCycle                     PagingCycle,

ran-NotificationAreaInfo            RAN-NotificationAreaInfo                                            OPTIONAL,   -- Need M

t380                                PeriodicRNAU-TimerValue                                             OPTIONAL,   -- Need R

nextHopChainingCount                NextHopChainingCount,

...,

[[

sl-ServingCellInfo-r17              SL-ServingCellInfo-r17                                              OPTIONAL, -- Cond L2RemoteUE

sdt-Config-r17                      SetupRelease { SDT-Config-r17 }                                     OPTIONAL,   -- Need M

srs-PosRRC-InactiveConfig-r17       SRS-PosRRC-InactiveConfig-r17                                       OPTIONAL,   -- Need M

ran-ExtendedPagingCycle-r17         ExtendedPagingCycle-r17                                             OPTIONAL    -- Need R

]]

}

////////////////////////////////irrelevant operations skipped///////////////////////////////////////

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfig-r17                       SRS-PosConfig-r17,

bwp-r17                                 BWP                                                                 OPTIONAL,    -- Need S

srs-TimeAlignmentTimer-r17              TimeAlignmentTimer                                                  OPTIONAL,    -- Need R

inactivePosSRS-RSRP-changeThresh-r17    RSRP-ChangeThresh-r17                                               OPTIONAL,    -- Need R

srs-NrofSS-BlocksToAverage-r17          INTEGER (1..ffsUpperLimit)                                          OPTIONAL,    -- Need R

-- FFS upper limit

inactivePosSRS-AbsThreshSS-BlocksConsolidation-r17  RSRP-Range                                              OPTIONAL     -- Need R

}

As we all know, suspend configuration for INACTIVE is putting into RRCRelease message by CU for a UE. But looking at all the five subfields within SRS-PosRRC-InactiveConfig-r17, we believe they all are under the responsibility of DU where DU should generate for a UE and supply to CU to be configured to the UE by RRCRelease. However, currently there is no such transport over F1AP [2] and we are not sure whether RAN3 was aware of this in the past positioning discussions.

Observation 1: For positioning measurement during INACTIVE, RAN2 defined the SRS-PosRRC-InactiveConfig-r17 in RRC spec that is configured by suspend configuration via RRCRelease message.

Observation 2: Suspend configuration for INACTIVE is putting into RRCRelease message by CU for a UE. But looking at all the five subfields within SRS-PosRRC-InactiveConfig-r17, we believe they all are under the responsibility of DU where DU should generate for a UE and supply to CU to be configured to the UE by RRCRelease.

Observation 3: However, currently there is no such transport over F1AP from DU and we are not sure whether RAN3 was aware of this in the past positioning discussions.

In the following subsections, we investigate each sub-field one by one and what RAN3 should do and what should be changed in F1AP [2] to make the positioning measurement during INACTIVE work flawlessly.

## srs-PosConfig-r17

According to RRC specification [1], srs-PosConfig-r17 consists of four sub-fields where each is a list of Rel-16 IEs, i.e. SRS-PosResourceSet and SRS-PosResource:

SRS-PosConfig-r17 ::=               SEQUENCE {

srs-PosResourceSetToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16 OPTIONAL,-- Need N

srs-PosResourceSetToAddModList-r17  SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16  OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r17    SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16      OPTIONAL,-- Need N

srs-PosResourceToAddModList-r17     SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16        OPTIONAL -- Need N

}

Checking in our F1AP [2], currently those Rel-16 IEs are part of 9.3.1.192 SRS Configuration:

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

9.3.1.192		SRS Configuration

This information element contains the SRS configuration configured by the gNB-CU for the UE.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| SRS Carrier List |  | 1..<maxnoSRS-Carriers> |  |  |
| >Point A | M |  | INTEGER (0..3279165) | NR ARFCN |
| >Uplink Channel BW-PerSCS-List |  | 1..<maxnoSCSs> |  | SCS-SpecificCarrier TS 38.331 [8] |
| >>Offset To Carrier | M |  | INTEGER(0..2199,…) | First usable RB to Point A in the number of PRBs |
| >>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…) |  |
| >>Carrier Bandwidth | M |  | INTEGER(1..275,…) |  |
| >Active UL BWP | M |  |  | Only the configuration in the active UL BWP is needed. |
| >>Location And Bandwidth | M |  | INTEGER(0..37949,…) | BWP TS 38.331 [8] |
| >>Subcarrier Spacing | M |  | ENUMERATED(kHz15, kHz30, kHz60, kHz120,…) |  |
| >>Cyclic Prefix | M |  | ENUMERATED(Normal, Extended) |  |
| >>Tx Direct Current Location | M |  | INTEGER(0..3301,…) |  |
| >>Shift7dot5kHz | O |  | ENUMERATED(true,…) |  |
| >>SRS Config | M |  |  | SRS-Config as defined in TS 38.331 [8] |
| >>>SRS Resource List |  | 0..<maxnoSRS-Resources> |  |  |
| >>>>SRS Resource | M |  | 9.3.1.193 | SRS-Resource as defined in TS 38.331 [8] |
| >>>Positioning SRS Resource List |  | 0..<maxnoSRS-PosResources> |  |  |
| >>>>Positioning SRS Resource | M |  | 9.3.1.194 | SRS-PosResource-r16 as defined in TS 38.331 [8] |
| >>>SRS Resource Set List |  | 0..<maxnoSRS-ResourceSets> |  |  |
| >>>>SRS Resource Set | M |  | 9.3.1.195 | SRS-ResourceSet as defined in TS 38.331 [8] |
| >>>Positioning SRS Resource Set List |  | 0..<maxnoSRS-PosResourceSets> |  |  |
| >>>>Positioning SRS Resource Set | M |  | 9.3.1.196 | SRS-PosResourceSet-r16 as defined in TS 38.331 [8] |
| >PCI | O |  | INTEGER (0..1007) | Physical Cell ID of the cell that contains the SRS carrier |

/////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////////

where 9.3.192 SRS Configuration is used in the Positioning Information Exchange, Positioning Information Update, Positioning Measurement, and Positioning Measurement Update procedures. Looking at their descriptions, we found several reasonable evidence to believe they are under the responsibility of DU:

### 8.13.9 Positioning Information Exchange #### 8.13.9.1 General The Positioning Information Exchange procedure is initiated by the gNB-CU to indicate to the gNB-DU the need to configure the UE to transmit SRS signals and to retrieve the SRS configuration from the gNB-DU. The procedure uses UE-associated signalling. ////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////// ### 8.13.16 Positioning Information Update #### 8.13.16.1 General The Positioning Information Update procedure is initiated by the gNB-DU to indicate to the gNB-CU that a change has occurred in the SRS configuration, or to report UE Tx TEG information. The procedure uses UE-associated signalling. ////////////////////////////////////////////////////////////irrelevant operations skipped///////////////////////////////////////////////////////////////////

Also in RRC [1], we cross-checked that those SRS-PosResourceSet and SRS-PosResource IEs are included in CellGroupConfig which is generated by DU and conveyed to CU [3].

From the above evidence, it looks clear that srs-PosConfig-r17 is under the responsibility of DU.

Observation 4: srs-PosConfig-r17 consists of Rel-16 IEs (SRS-PosResourceSet, SRS-PosResource) where from F1AP and RRC it looks clear that those Rel-16 IEs are under the responsibility of DU.

## bwp-r17

We cross-checked that bwp-r17 includes BWP configuration which in earlier release was generated by DU and conveyed to CU via CellGroupConfig [3]. So, it looks clear that bwp-r17 is under the responsibility of DU.

Observation 5: bwp-r17 includes BWP configuration which in earlier releases was generated by DU and conveyed to CU via CellGroupConfig. It looks clear that bwp-r17 is under the responsibility of DU.

## Other parameters

The remaining sub-fields are srs-TimeAlignmentTimer-r17, inactivePosSRS-RSRP-changeThresh-r17, srs-NrofSS-BlocksToAverage-r17, and inactivePosSRS-AbsThreshSS-BlocksConsolidation-r17.

According to RRC field descriptions [1], they are all related to the timing alignment validation:

SRS-PosRRC-InactiveConfig field descriptions

bwp BWP configuration for SRS for Positioning during the RRC_INACTIVE state. If the field is absent UE is configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP.

inactivePosSRS-AbsThreshSS-BlocksConsolidation absolute RSRP threshold for determining the set of SSBs for derivation of downlink pathloss reference for TA validation.

inactivePosSRS-NrofSS-BlocksToAverage number of SSBs with highest RSRPs for derivation of downlink pathloss reference for TA validation

inactivePosSRS-RSRP-changeThresh RSRP threshold for the increase/decrease of RSRP for time alignment validation

srs-TimeAlignmnetTimer TA timer for SRS for positioning transmission during RRC_INACTIVE State. Editor's Note: Range to be taken from SDT

Also, their details are described by NR MAC specification [4]:

## 5.2 Maintenance of Uplink Time Alignment RRC configures the following parameters for the maintenance of UL time alignment: - timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned; - inactivePosSRS-TimeAlignmentTimer which controls how long the MAC entity considers the Positioning SRS transmission in RRC_INACTIVE in clause 5.26 to be uplink time aligned; - cg-SDT-TimeAlignmentTimer which controls how long the MAC entity considers the uplink transmission for CG-SDT to be uplink time aligned. ## ////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////// ## 5.26 Positioning SRS transmission in RRC_INACTIVE ### 5.26.1 General Periodic and semi-persistent Positioning SRS can be configured for Positioning SRS transmission in RRC_INACTIVE. RRC configures the following parameter for TA validation of the Positioning SRS transmission in RRC_INACTIVE: - inactivePosSRS-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation. The MAC entity shall, if the TA of the configured Positioning SRS is valid according to clause 5.26.1: - transmit Positioning Periodic SRS or Semi-Persistent SRS defined in TS 38.214 [7]. ### 5.26.2 TA validation for SRS transmission in RRC_INACTIVE RRC configures the following parameters for validation for SRS transmission in RRC_INACTIVE: - inactivePosSRS-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation; - inactivePosSRS-NrOfSS-BlocksToAverage: number of SSBs with highest RSRPs for derivation of downlink pathloss reference for TA validation; - inactivePosSRS-AbsThreshSS-BlocksConsolidation: absolute RSRP threshold for determining the set of SSBs for derivation of downlink pathloss reference for TA validation. The MAC entity shall: 1> if inactivePosSRS-NrOfSS-BlocksToAverage is not configured; or 1> if inactivePosSRS-AbsThreshSS-BlocksConsolidation is not configured or the highest beam measurement quantity value is below or equal to inactivePosSRS-AbsThreshSS-BlocksConsolidation, if configured: 2> derive the downlink pathloss reference RSRP for TA validation for SRS transmission in RRC_INACTIVE as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [24]. 1> else: 2> derive the downlink pathloss reference RSRP for TA validation for SRS transmission in RRC_INACTIVE as the linear average of the power values of up to inactivePosSRS-NrOfSS-BlocksToAverage of the highest beam measurement quantity values above inactivePosSRS-AbsThreshSS-BlocksConsolidation, where each beam measurement quantity is described in TS 38.215 [24]. The MAC entity shall: 1> if the UE is configured with measObject for the Serving Cell where the UE receives configuration for SRS transmission in RRC_INACTIVE: 2> store the RSRP of the downlink pathloss reference derived based on the measObject configured for the Serving Cell as in TS 38.331. 1> else if Timing Advance Command MAC CE is received for inactivePosSRS-TimeAlignmentTimer as in clause 5.2: 2> update the stored downlink pathloss reference with the current RSRP value of the downlink pathloss reference. The MAC entity shall consider the TA to be valid when the following condition is fulfilled: 1> compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference has not increased/decreased by more than inactivePosSRS-RSRP-ChangeThreshold, if configured.

MAC protocol is under DU and the management of uplink timing alignment is also the responsibility of DU. Based on the above, it looks clear that those remaining sub-fields are also under the responsibility of DU.

Observation 6: The remaining sub-fields of SRS-PosRRC-InactiveConfig-r17 are related to uplink timing alignment validation, where their details are described in RAN2 MAC specification, and the management of uplink timing alignment is under the responsibility of DU. It looks clear that those remaining sub-fields are also under the responsibility of DU.

## RAN3 impacts

Based on the above investigations and observations, it looks quite clear that DU is responsible for all the sub-fields of SRS-PosRRC-InactiveConfig-r17. Therefore, it is reasonable to conclude that SRS-PosRRC-InactiveConfig-r17 included in the suspend configuration of RRCRelease message for a UE should be coming from DU as a whole.

Observation 7: Based on the above investigations and observations, it looks clear that DU is responsible for all the sub-fields under SRS-PosRRC-InactiveConfig-r17.

Observation 8: As a result, it is reasonable to conclude that SRS-PosRRC-InactiveConfig-r17 included in the suspend configuration of RRCRelease message for a UE should be coming from DU as a whole.

For such transport from DU (which is currently missing in F1AP), we think that the famous DU to CU RRC Information IE is the right place. The DU to CU RRC Information IE has been used as a way to transport UE-specific RRC IE or container from DU to be used by CU for compiling RRC messages. The IE has been included in the UE context management related procedures to serve the purpose. The SRS-PosRRC-InactiveConfig-r17 that we have discussed so far is also for a specific UE that should be coming from DU and to be included by CU into RRCRelease message, so we believe it is very reasonable to define the corresponding IE container in the DU to CU RRC Information IE.

Observation 9: For such transport from DU (which is currently missing in F1AP), the DU to CU RRC Information IE is the right place which has been used to transport UE-specific RRC IE or container from DU to be used by CU for compiling RRC messages. The SRS-PosRRC-InactiveConfig-r17 discussed so far is UE-specific and coming from DU to be included by CU into RRCRelease.

Proposal 1: RAN3 to include the corresponding RRC IE container for SRS-PosRRC-InactiveConfig-r17 in the DU to CU RRC Information IE.

One more thing to discuss is about CU's retrieval of this SRS-PosRRC-InactiveConfig-r17 configuration from DU. Since CU is the one who decides to move the UE to INACTIVE state and also decides positioning measurements during INACTIVE, this configuration should be ready in CU before sending RRCRelease message and CU should be able to query it from DU.

For Rel-17 CG-SDT works, RAN3 did the similar job for SDT-MAC-PHY-CG-Config-r17 which is generated by DU and included in the suspend configuration of RRCRelease message. For that, RAN3 defined the CG-SDT Query Indication IE in the UE CONTEXT MODIFICATION REQUEST message to enable CU to retrieve such configuration [5]. For this SRS-PosRRC-InactiveConfig-r17 which is generated by DU and shares the same story, we should do the same thing. So, we also propose to add the Positioning INACTIVE Query Indication IE in the UE CONTEXT MODIFICATION REQUEST message with an appropriate description.

Observation 10: Moreover, CU (who decides moving the UE to INACTIVE and decides positioning measurements during INACTIVE) should be able to query SRS-PosRRC-InactiveConfig-r17 and retrieve it from DU before sending RRCRelease message.

Observation 11: For Rel-17 CG-SDT works, RAN3 did the similar job for SDT-MAC-PHY-CG-Config-r17 generated by DU and included in the suspend configuration by CU. For that, RAN3 defined the CG-SDT Query Indication IE in the UE CONTEXT MODIFICATION REQUEST message to enable CU to retrieve such configuration. For this SRS-PosRRC-InactiveConfig-r17 which is generated by DU and shares the same story, the same query mechanism should also in place in F1AP.

Proposal 2: RAN3 to add a query indicator IE (e.g. "Positioning INACTIVE Query Indicator") in the UE CONTEXT MODIFICATION REQUEST message to enable CU to retrieve SRS-PosRRC-InactiveConfig-r17 from DU.

# Conclusion

In the present contribution we make the following observations:

Observation 1: For positioning measurement during INACTIVE, RAN2 defined the SRS-PosRRC-InactiveConfig-r17 in RRC spec that is configured by suspend configuration via RRCRelease message:

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfig-r17                       SRS-PosConfig-r17,

bwp-r17                                 BWP                                                                 OPTIONAL,    -- Need S

srs-TimeAlignmentTimer-r17              TimeAlignmentTimer                                                  OPTIONAL,    -- Need R

inactivePosSRS-RSRP-changeThresh-r17    RSRP-ChangeThresh-r17                                               OPTIONAL,    -- Need R

srs-NrofSS-BlocksToAverage-r17          INTEGER (1..ffsUpperLimit)                                          OPTIONAL,    -- Need R

-- FFS upper limit

inactivePosSRS-AbsThreshSS-BlocksConsolidation-r17  RSRP-Range                                              OPTIONAL     -- Need R

}

Observation 2: Suspend configuration for INACTIVE is putting into RRCRelease message by CU for a UE. But looking at all the five subfields within SRS-PosRRC-InactiveConfig-r17, we believe they all are under the responsibility of DU where DU should generate for a UE and supply to CU to be configured to the UE by RRCRelease.

Observation 3: However, currently there is no such transport over F1AP from DU and we are not sure whether RAN3 was aware of this in the past positioning discussions.

Observation 4: srs-PosConfig-r17 consists of Rel-16 IEs (SRS-PosResourceSet, SRS-PosResource) where from F1AP and RRC it looks clear that those Rel-16 IEs are under the responsibility of DU.

Observation 5: bwp-r17 includes BWP configuration which in earlier releases was generated by DU and conveyed to CU via CellGroupConfig. It looks clear that bwp-r17 is under the responsibility of DU.

Observation 6: The remaining sub-fields of SRS-PosRRC-InactiveConfig-r17 are related to uplink timing alignment validation, where their details are described in RAN2 MAC specification, and the management of uplink timing alignment is under the responsibility of DU. It looks clear that those remaining sub-fields are also under the responsibility of DU.

Observation 7: Based on the above investigations and observations, it looks clear that DU is responsible for all the sub-fields under SRS-PosRRC-InactiveConfig-r17.

Observation 8: As a result, it is reasonable to conclude that SRS-PosRRC-InactiveConfig-r17 included in the suspend configuration of RRCRelease message for a UE should be coming from DU as a whole.

Observation 9: For such transport from DU (which is currently missing in F1AP), the DU to CU RRC Information IE is the right place which has been used to transport UE-specific RRC IE or container from DU to be used by CU for compiling RRC messages. The SRS-PosRRC-InactiveConfig-r17 discussed so far is UE-specific and coming from DU to be included by CU into RRCRelease.

Observation 10: Moreover, CU (who decides moving the UE to INACTIVE and decides positioning measurements during INACTIVE) should be able to query SRS-PosRRC-InactiveConfig-r17 and retrieve it from DU before sending RRCRelease message.

Observation 11: For Rel-17 CG-SDT works, RAN3 did the similar job for SDT-MAC-PHY-CG-Config-r17 generated by DU and included in the suspend configuration by CU. For that, RAN3 defined the CG-SDT Query Indication IE in the UE CONTEXT MODIFICATION REQUEST message to enable CU to retrieve such configuration. For this SRS-PosRRC-InactiveConfig-r17 which is generated by DU and shares the same story, the same query mechanism should also in place in F1AP.

Based on the discussion in the present contribution and the observations above we propose:

Proposal 1: RAN3 to include the corresponding RRC IE container for SRS-PosRRC-InactiveConfig-r17 in the DU to CU RRC Information IE.

Proposal 2: RAN3 to add a query indicator IE (e.g. "Positioning INACTIVE Query Indicator") in the UE CONTEXT MODIFICATION REQUEST message to enable CU to retrieve SRS-PosRRC-InactiveConfig-r17 from DU.

The proposed changes for F1AP are provided in [6].

# Reference

[1] TS 38.331, NR; Radio Resource Control (RRC) protocol specification (Release 17)

[2] TS 38.473, NG-RAN; F1 application protocol (F1AP) (Release 17)

[3] R2-2205824, "[I512] [I010] SRS Positioning configuration provided for SDT", Intel Corporation

[4] TS 38.321, NR; Medium Access Control (MAC) protocol specification (Release 17)

[5] R3-222963, "CG-SDT BLCR to TS 37.473", Samsung, ZTE, Nokia, Nokia Shanghai Bell, Intel Corporation, CATT

[6] R3-223498, "Rel-17 ePos correction on positioning measurement during INACTIVE for SRS-PosRRC-InactiveConfig-r17", Intel Corporation