3GPP TSG-RAN WG3 Meeting #116-e	R3-223537

# E-meeting, 09 May – 19 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0015 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NR MBS in E1AP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR fixed some errors in latest versino of E1AP specifications - The criticality of the gNB-CU-UP MBS E1AP ID IE in BC BEARER CONTEXT SETUP FAILURE message should be ignore, since the gNB-CU-UP can ignore this IE if not comprehend. - RAN2 agreed to extend MRB ID space beyond current 32 limit and up to 512, in case there are limited MBS services within a cell or a DU, the same MRB ID can be used for all the UEs for the same MBS service, note that during HO, the UE may need to be re-configured to change the MRB ID used in the new cell. Thus, the value of MRB ID should be INTEGER (1.. 512, ...) in E1AP specification. - The MBS NG-U Information at NG-RAN IE is used to provide the shared DL NG-U tunnel information, in which the UP Transport Layer Information is generated by gNB for shared NG-U unicast transport. Thus, the "UL" may be a typo, and the description about shared NG-U multicast unicast transport in BC Bearer Context NG-U TNL Info at NG-RAN IE should be removed. - In the current version specification, the presence of the BC Bearer Context NG-U TNL Info at NG-RAN IE and the MC Bearer Context NG-U TNL Info at NG-RAN IE are "C-ifunicast", which conditions are expressed as that this IE shall be present if the MBS NG-U Information at NG-RAN IE within the MC/BC Bearer Context NG-U TNL Info at NG-RAN IE contains unicast TNL information. However, in the MC/BC Bearer Context NG-U TNL Info at NG-RAN IE, there is no alternative other than unicast TNL information. So, determine the the presence of the MC/BC Bearer Context NG-U TNL Info at NG-RAN IE denpend on whether including unicast TNL information is unfeasibly. It can be simply changed to optional. In ASN.1 part, the notes should be removed. - in current MC Bearer Context Modification related messages, both the gNB-CU-CP/UP MBS E1AP ID pair and the MBS Multicast F1-U Context Descriptor IE are included in all the messages or the included IEs. When the F1 Context setup, the MBS Multicast F1-U Context Descriptor IE is included in MC Bearer Context F1-U TNL Info at DU IE. As the gNB- gNB-CU-CP/UP MBS E1AP ID pair value is there, and the value of the MBS Multicast F1-U Context Descriptor IE should not be updated, it is better to only keep the MBS Multicast F1-U Context Descriptor IE in the MC Bearer Context F1-U TNL Info at DU IE message, and remove it in other message and IEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the criticality of the gNB-CU-UP MBS E1AP ID IE to ignore, in BC BEARER CONTEXT SETUP FAILURE message. - Update the value of MRB ID from INTEGER (1.. 32, ...) to INTEGER (1.. 512, ...). - Update "Shared NG-U UL Transport Layer Information" to "Shared NG-U DL Transport Layer Information" in MBS NG-U Information at NG-RAN IE, and remove the description about shared NG-U multicast unicast transport in BC Bearer Context NG-U TNL Info at NG-RAN IE and MC Bearer Context NG-U TNL Info at NG-RAN IE. - Update the presence of the BC Bearer Context NG-U TNL Info at NG-RAN IE in BC Bearer Context To Setup Response IE and the MC Bearer Context NG-U TNL Info at NG-RAN IE in MC Bearer Context To Setup Response IE from “C-ifunicast” to “O”, and remove the conditions, explanations and notes in ASN.1 part for “ifunicast”. - Remove the MBS Multicast F1-U Context Descriptor IE in the MC Bearer Context To Modify, MC Bearer Context To Modify Response, MC Bearer Context To Modify Required and MC Bearer Context To Modify Confirm IEs and MC BEARER CONTEXT MODIFICATION FAILURE message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The above NR MBS related errors remain in E1AP specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.5.1.3, 9.2.5.2.6, 9.3.1.16a, 9.3.1.116, 9.3.1.117, 9.3.1.121, 9.3.3.27, 9.3.3.33, 9.3.3.34, 9.3.3.35, 9.3.3.36, 9.3.3.37, 9.4.4, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------Start of the First Change-------------

##### 9.2.5.1.3	BC BEARER CONTEXT SETUP FAILURE

This message is sent by the gNB-CU-UP to indicate that the setup of the broadcast MBS Session Resource context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.106 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | O |  | 9.3.1.107 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

-----------Start of the Next Change-------------

##### 9.2.5.2.6	MC BEARER CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-CU-UP to indicate that the modification of the multicast MBS Session Resource context was unsuccessful.

Direction: gNB-CU-UP  gNB-CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP MBS E1AP ID | M |  | 9.3.1.106 |  | YES | reject |
| gNB-CU-UP MBS E1AP ID | M |  | 9.3.1.107 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

-----------Start of the Next Change-------------

#### 9.3.1.16a	MRB ID

This IE uniquely identifies an MRB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MRB ID | M |  | INTEGER (1.. 512, ...) |  |

-----------Start of the Next Change-------------

#### 9.3.1.116	BC Bearer Context NG-U TNL Info at NG-RAN

This IE contains NG-RAN NG-U TNL information for an MBS Session for both, shared NG-Uunicast transport. It may also contain per Area Session ID NG-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS NG-U Information at NG-RAN | M |  | 9.3.1.117 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS NG-U Information at NG-RAN |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.111 |  |
| >>MBS NG-U Information at NG-RAN | M |  | 9.3.1.117 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

#### 9.3.1.117	MBS NG-U Information at NG-RAN

This IE contains NG-RAN TNL information for a single shared NG-U tunnel.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS NG-U Transport | M |  |  |  |
| >unicast |  |  |  |  |
| >>Shared NG-U DL Transport Layer Information | M |  | UP Transport Layer Information 9.3.2.1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

-----------Start of the Next Change-------------

#### 9.3.1.121	MC Bearer Context NG-U TNL Info at NG-RAN

This IE contains NG-RAN NG-U TNL information for an MBS Session for both, shared NG-Uunicast transport. It may also contain per Area Session ID NG-U TNL information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE MBS Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS NG-U Information at NG-RAN | M |  | 9.3.1.117 |  |
| >location dependent |  |  |  |  |
| >>Location dependent MBS NG-U Information at NG-RAN |  | 1..<maxnoofMBSAreaSessionIDs> |  |  |
| >>>MBS Area Session ID | M |  | 9.3.1.111 |  |
| >>>MBS NG-U Information at NG-RAN | M |  | 9.3.1.117 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSAreaSessionIDs | Maximum no. of MBS Area Session IDs. Value is 256. |

-----------Start of the Next Change-------------

#### 9.3.3.27	BC Bearer Context To Setup Response

This IE contains MBS session resource related information used to confirm BC Bearer Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| BC Bearer Context NG-U TNL Info at NG-RAN | O |  | 9.3.1.116 |  |
| BC MRB Setup Response List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >MBS QoS Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| >BC Bearer Context F1-U TNL Info at CU | M |  | 9.3.1.118 |  |
| BC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >Cause | M |  | 9.3.1.2 |  |
| Available BC MRB Configuration | O |  | BC MRB Setup Configuration 9.3.1.114 | In case the shared MBS NG-U termination had a different MRB Configuration applied. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

-----------Start of the Next Change-------------

#### 9.3.3.33	MC Bearer Context To Setup Response

This IE contains MBS session resource related information used to confirm MC Bearer Context Context Setup.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at NG-RAN | O |  | 9.3.1.121 |  |
| MC MRB Setup Response List |  | 1..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >MBS QoS Flow Setup List | M |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| MC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >Cause | M |  | 9.3.1.2 |  |
| Available MC MRB Configuration | O |  | MC MRB Setup Configuration 9.3.1.120 | In case the shared MBS NG-U termination had a different MRB Configuration applied. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

#### 9.3.3.34	MC Bearer Context To Modify

This IE contains MBS session resource related information used to request a modification of a multicast MC Bearer Context.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at 5GC | O |  | 9.3.1.122 |  |
| MC Bearer Context NG-U TNL Info at NG-RAN Request | O |  | 9.3.1.123 | To request NG-U TNL information from the gNB-CU-UP, if not yet available at gNB-CU-CP |
| MC MRB To Setup or Modify List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >MC Bearer Context F1-U TNL Info at DU | O |  | 9.3.1.124 |  |
| >SDAP Configuration | O |  | 9.3.1.39 |  |
| >MBS PDCP Configuration | O |  | PDCP Configuration 9.3.1.38 | Editor’s Note: along running RRC CR |
| >MBS QoS Flows Information To Be Setup | O |  | QoS Flow QoS Parameters List 9.3.1.25 |  |
| >MRB QoS | O |  | QoS Flow Level QoS Parameters 9.3.1.26 | Indicates the MRB QoS when more than one QoS Flow is mapped to the MRB. |
| MC MRB To Remove List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

#### 9.3.3.35	MC Bearer Context To Modify Response

This IE contains MBS session resource related information used to confirm a MC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC Bearer Context NG-U TNL Info at NG-RAN Modify Response | O |  | 9.3.1.127 |  |
| MC MRB Setup or Modify Response List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >MBS QoS Flow Setup List | O |  | QoS Flow List 9.3.1.12 |  |
| >MBS QoS Flow Failed List | O |  | Flow Failed List 9.3.1.45 |  |
| >MC Bearer Context F1-U TNL Info at CU | O |  | UP Transport Layer Information 9.3.2.1 |  |
| MC MRB Failed List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >Cause | M |  | 9.3.1.2 |  |
| Available MC MRB Configuration | O |  | MC MRB Setup Configuration 9.3.1.120 | In case the shared MBS NG-U termination had a different MRB Configuration applied. |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

#### 9.3.3.36	MC Bearer Context To Modify Required

This IE contains MBS session resource related information used to request MC Bearer Context Modification.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC MRB To Remove List Required |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for a UE. Value is 32. |

#### 9.3.3.37	MC Bearer Context To Modify Confirm

This IE contains MBS session resource related information used to confirm a MC Bearer Context Modification.

NOTE:	In the current version of this specification, this IE does not contain any information.

-----------Start of the Next Change-------------

-----------Start of the Next Change-------------

### 9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

#### -- Elementary Procedure definitions

--

-- **************************************************************

E1AP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-PDU-Descriptions (0) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

//skip unchanged part

-- **************************************************************

--

-- BC BEARER CONTEXT SETUP FAILURE

--

-- **************************************************************

BCBearerContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextSetupFailureIEs } },

...

}

BCBearerContextSetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY ignore	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE optional	}|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BC BEARER CONTEXT MODIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- BC BEARER CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

BCBearerContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextModificationRequestIEs } },

...

}

BCBearerContextModificationRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-BCBearerContextToModify			CRITICALITY reject	TYPE 	BCBearerContextToModify				PRESENCE mandatory	}|

{ ID id-GNB-DU-ID						CRITICALITY ignore	TYPE 	GNB-DU-ID							PRESENCE optional },

...

}

-- **************************************************************

--

-- BC BEARER CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

BCBearerContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextModificationResponseIEs } },

...

}

BCBearerContextModificationResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-BCBearerContextToModifyResponse	CRITICALITY reject	TYPE 	BCBearerContextToModifyResponse	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

-- BC BEARER CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

BCBearerContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextModificationFailureIEs } },

...

}

BCBearerContextModificationFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BC BEARER CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

-- **************************************************************

--

-- BC BEARER CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

BCBearerContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextModificationRequiredIEs } },

...

}

BCBearerContextModificationRequiredIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-BCBearerContextToModifyRequired	CRITICALITY reject	TYPE 	BCBearerContextToModifyRequired	PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- BC BEARER CONTEXT MODIFICATION CONFIRM

--

-- **************************************************************

BCBearerContextModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextModificationConfirmIEs } },

...

}

BCBearerContextModificationConfirmIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory }|

{ ID id-BCBearerContextToModifyConfirm	CRITICALITY reject	TYPE 	BCBearerContextToModifyConfirm	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BC BEARER CONTEXT RELEASE

--

-- **************************************************************

-- **************************************************************

--

-- BC BEARER CONTEXT RELEASE COMMAND

--

-- **************************************************************

BCBearerContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextReleaseCommandIEs } },

...

}

BCBearerContextReleaseCommandIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- BC BEARER CONTEXT RELEASE COMPLETE

--

-- **************************************************************

BCBearerContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextReleaseCompleteIEs } },

...

}

BCBearerContextReleaseCompleteIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- BC BEARER CONTEXT RELEASE REQUEST

--

-- **************************************************************

-- **************************************************************

--

-- BC BEARER CONTEXT RELEASE REQUEST

--

-- **************************************************************

BCBearerContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { BCBearerContextReleaseRequestIEs } },

...

}

BCBearerContextReleaseRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

#### -- MC BEARER CONTEXT SETUP

--

-- **************************************************************

-- **************************************************************

--

-- MC BEARER CONTEXT SETUP REQUEST

--

-- **************************************************************

MCBearerContextSetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextSetupRequestIEs } },

...

}

MCBearerContextSetupRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GlobalMBSSessionID				CRITICALITY reject	TYPE 	GlobalMBSSessionID					PRESENCE mandatory	}|

{ ID id-MCBearerContextToSetup			CRITICALITY reject	TYPE 	MCBearerContextToSetup				PRESENCE mandatory	}|

{ ID id-GNB-DU-ID						CRITICALITY ignore	TYPE 	GNB-DU-ID							PRESENCE optional		},

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT SETUP RESPONSE

--

-- **************************************************************

MCBearerContextSetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextSetupResponseIEs } },

...

}

MCBearerContextSetupResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-MCBearerContextToSetupResponse	CRITICALITY reject	TYPE 	MCBearerContextToSetupResponse	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT SETUP FAILURE

--

-- **************************************************************

MCBearerContextSetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextSetupFailureIEs } },

...

}

MCBearerContextSetupFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY ignore	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE optional	}|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MC BEARER CONTEXT MODIFICATION

--

-- **************************************************************

-- **************************************************************

--

-- MC BEARER CONTEXT MODIFICATION REQUEST

--

-- **************************************************************

MCBearerContextModificationRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextModificationRequestIEs } },

...

}

MCBearerContextModificationRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-MCBearerContextToModify			CRITICALITY reject	TYPE 	MCBearerContextToModify				PRESENCE mandatory	}|

{ ID id-GNB-DU-ID						CRITICALITY ignore	TYPE 	GNB-DU-ID							PRESENCE optional },

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT MODIFICATION RESPONSE

--

-- **************************************************************

MCBearerContextModificationResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextModificationResponseIEs } },

...

}

MCBearerContextModificationResponseIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-MCBearerContextToModifyResponse	CRITICALITY reject	TYPE 	MCBearerContextToModifyResponse	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT MODIFICATION FAILURE

--

-- **************************************************************

MCBearerContextModificationFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextModificationFailureIEs } },

...

}

MCBearerContextModificationFailureIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID				CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID					PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID				CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID					PRESENCE mandatory }|

{ ID id-MBSMulticastF1UContextDescriptor	CRITICALITY reject	TYPE 	MBSMulticastF1UContextDescriptor	PRESENCE optional	 }|

{ ID id-Cause								CRITICALITY ignore	TYPE 	Cause									PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	 },

...

}

-- **************************************************************

--

#### -- MC BEARER CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

-- **************************************************************

--

-- MC BEARER CONTEXT MODIFICATION REQUIRED

--

-- **************************************************************

MCBearerContextModificationRequired ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextModificationRequiredIEs } },

...

}

MCBearerContextModificationRequiredIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory	}|

{ ID id-MCBearerContextToModifyRequired	CRITICALITY ignore	TYPE 	MCBearerContextToModifyRequired	PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT MODIFICATION CONFIRM

--

-- **************************************************************

MCBearerContextModificationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextModificationConfirmIEs } },

...

}

MCBearerContextModificationConfirmIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID				PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID				PRESENCE mandatory }|

{ ID id-MCBearerContextToModifyConfirm	CRITICALITY reject	TYPE 	MCBearerContextToModifyConfirm	PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE 	CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MC BEARER CONTEXT RELEASE

--

-- **************************************************************

-- **************************************************************

--

-- MC BEARER CONTEXT RELEASE COMMAND

--

-- **************************************************************

MCBearerContextReleaseCommand ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextReleaseCommandIEs } },

...

}

MCBearerContextReleaseCommandIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- MC BEARER CONTEXT RELEASE COMPLETE

--

-- **************************************************************

MCBearerContextReleaseComplete ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextReleaseCompleteIEs } },

...

}

MCBearerContextReleaseCompleteIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics				PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- MC BEARER CONTEXT RELEASE REQUEST

--

-- **************************************************************

-- **************************************************************

--

-- MC BEARER CONTEXT RELEASE REQUEST

--

-- **************************************************************

MCBearerContextReleaseRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MCBearerContextReleaseRequestIEs } },

...

}

MCBearerContextReleaseRequestIEs E1AP-PROTOCOL-IES ::= {

{ ID id-GNB-CU-CP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-CP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-GNB-CU-UP-MBS-E1AP-ID			CRITICALITY reject	TYPE 	GNB-CU-UP-MBS-E1AP-ID			PRESENCE mandatory }|

{ ID id-Cause							CRITICALITY ignore	TYPE 	Cause							PRESENCE mandatory	},

...

}

END

-- ASN1STOP

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

//skip unchanged part

##### -- BCBearerContextToSetupResponse

BCBearerContextToSetupResponse ::= SEQUENCE {

bcBearerContextNGU-TNLInfoatNGRAN			BCBearerContextNGU-TNLInfoatNGRAN			OPTIONAL,

bcMRBSetupResponseList					BCMRBSetupResponseList,

bcMRBFailedList							BCMRBFailedList							OPTIONAL,

availableBCMRBConfig						BCMRBSetupConfiguration					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {BCBearerContextToSetupResponse-ExtIEs} }	OPTIONAL,

...

}

BCBearerContextToSetupResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

//skip unchanged part

##### -- MCBearerContextToSetupResponse

MCBearerContextToSetupResponse ::= SEQUENCE {

mcBearerContextNGU-TNLInfoatNGRAN			MCBearerContextNGU-TNLInfoatNGRAN			OPTIONAL,

mcMRBSetupResponseList					MCMRBSetupResponseList,

mcMRBFailedList							MCMRBFailedList							OPTIONAL,

availableMCMRBConfig						MCMRBSetupConfiguration					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToSetupResponse-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToSetupResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextNGU-TNLInfoatNGRAN::= CHOICE {

locationindpendent				MBSNGUInformationAtNGRAN,

locationdependent				LocationDependentMBSNGUInformationAtNGRAN,

choice-extension	ProtocolIE-SingleContainer	{{MCBearerContextNGU-TNLInfoatNGRAN-ExtIEs}}

}

MCBearerContextNGU-TNLInfoatNGRAN-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

MCMRBSetupResponseList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBSetupResponseList-Item

MCMRBSetupResponseList-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

qosflow-setup						QoS-Flow-List,

qosflow-failed						QoS-Flow-Failed-List		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupResponseList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBFailedList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBFailedList-Item

MCMRBFailedList-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

cause								Cause,

iE-Extensions		ProtocolExtensionContainer { {MCMRBFailedList-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBFailedList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToModify

MCBearerContextToModify ::= SEQUENCE {

mcBearerContextNGUTNLInfoat5GC					MCBearerContextNGUTNLInfoat5GC				OPTIONAL,

mcBearerContextNGUTnlInfoatNGRANRequest			MCBearerContextNGUTnlInfoatNGRANRequest		OPTIONAL,

mcMRBToSetupModifyList							MCMRBSetupModifyConfiguration				OPTIONAL,

mcMRBToRemoveList								MCMRBRemoveConfiguration						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModify-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModify-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextNGUTNLInfoat5GC ::= SEQUENCE {

mbsNGUInformationAt5GC			MBSNGUInformationAt5GC,

mbsAreaSession-ID				MBSAreaSessionID		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextNGUTNLInfoat5GC-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextNGUTNLInfoat5GC-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextNGUTnlInfoatNGRANRequest ::= SEQUENCE {

ngRANNGUTNLRequested				ENUMERATED {requested, ...},

mbsAreaSession-ID				MBSAreaSessionID		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextNGUTnlInfoatNGRANRequest-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextNGUTnlInfoatNGRANRequest-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBSetupModifyConfiguration ::= SEQUENCE {

mrb-ID							MRB-ID,

f1uTNLatDU						MCBearerContextF1UTNLInfoatDU		OPTIONAL,

sdap-config						SDAP-Configuration				OPTIONAL,

mbs-pdcp-config					PDCP-Configuration				OPTIONAL,

qoS-Flow-QoS-Parameter-List		QoS-Flow-QoS-Parameter-List		OPTIONAL,

mrbQoS							QoSFlowLevelQoSParameters		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupModifyConfiguration-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupModifyConfiguration-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextF1UTNLInfoatDU ::= SEQUENCE {

mbsF1UInfoatDU						UP-TNL-Information,

mbsMulticastF1UContextDescriptor	MBSMulticastF1UContextDescriptor,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextF1UTNLInfoatDU-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextF1UTNLInfoatDU-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSMulticastF1UContextDescriptor ::= CHOICE {

du					NULL,

mbsAreaSession		MBSAreaSessionID,

cell				MCDUCellReference,

ptp-retransmission	MBSPTPUEReference,

ptp-only-MRB			MBSPTPUEReference,

choice-extension		ProtocolIE-SingleContainer	{{MBSMulticastF1UContextDescriptor-ExtIEs}}

}

MBSMulticastF1UContextDescriptor-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

MCDUCellReference ::= SEQUENCE {

du-CellIndex			INTEGER (1..512),

nr-CGI				NR-CGI,

iE-Extensions		ProtocolExtensionContainer { {MCDUCellReference-ExtIEs} }	OPTIONAL,

...

}

MCDUCellReference-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MBSPTPUEReference::= SEQUENCE {

ueReferenceID		INTEGER (0..65535, ...),

du-CellIndex			INTEGER (1..512),

iE-Extensions		ProtocolExtensionContainer { {MBSPTPUEReference-ExtIEs} }	OPTIONAL,

...

}

MBSPTPUEReference-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBRemoveConfiguration ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF DRB-ID

##### -- MCBearerContextToModifyResponse

MCBearerContextToModifyResponse ::= SEQUENCE {

mcBearerContextNGU-TNLInfoatNGRANModifyResponse	MCBearerContextNGU-TNLInfoatNGRANModifyResponse		OPTIONAL,

mcMRBModifySetupResponseList						MCMRBSetupModifyResponseList							OPTIONAL,

mcMRBFailedList									MCMRBFailedList										OPTIONAL,

availableMCMRBConfig								MCMRBSetupConfiguration								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModifyResponse-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModifyResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCBearerContextNGU-TNLInfoatNGRANModifyResponse ::= SEQUENCE {

mbs-NGU-InfoatNGRAN	MBSNGUInformationAtNGRAN,

mbsAreaSession		MBSAreaSessionID					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextNGU-TNLInfoatNGRANModifyResponse-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextNGU-TNLInfoatNGRANModifyResponse-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MCMRBSetupModifyResponseList ::= SEQUENCE (SIZE(1..maxnoofMRBs)) OF MCMRBSetupModifyResponseList-Item

MCMRBSetupModifyResponseList-Item ::= SEQUENCE {

mrb-ID								MRB-ID,

qosflow-setup						QoS-Flow-List			OPTIONAL,

qosflow-failed						QoS-Flow-Failed-List		OPTIONAL,

mcBearerContextF1UTNLInfoatCU		UP-TNL-Information		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCMRBSetupModifyResponseList-Item-ExtIEs} }	OPTIONAL,

...

}

MCMRBSetupModifyResponseList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToModifyRequired

MCBearerContextToModifyRequired ::= SEQUENCE {

mcMRBToRemoveRequiredList						MCMRBRemoveConfiguration						OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModifyRequired-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModifyRequired-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

##### -- MCBearerContextToModifyConfirm

MCBearerContextToModifyConfirm ::= SEQUENCE {

iE-Extensions		ProtocolExtensionContainer { {MCBearerContextToModifyConfirm-ExtIEs} }	OPTIONAL,

...

}

MCBearerContextToModifyConfirm-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MDTPollutedMeasurementIndicator ::= ENUMERATED {

iDC,

no-IDC,

...

}

MRB-ID	::=	INTEGER (1..512, ...)

-----------End of the Change-------------