3GPP TSG-RAN3 Meeting #116-e	R3-223547

09 – 19 May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0942 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of pre-confiugred measurement gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS-Core |  |  |  |  |  | Date: |  |  | 2022-04-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 16) Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) |  |
| Reason for change: |  | - current spec doesn’t support the gNB-CU obtain the preconfigured MGs information from gNB-DU - gNB-CU is not aware of the preconfigured measurement gap that is (de)activated by UE initiation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add PosMeasGapPreConfigList IE MEASUREMENT PRECONFIGURATION CONFIRM messaeg. - Add new procedure Measurement Activation Notify |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The pre-configured measuremeng gap cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.13.18, 9.2.12.31, 8.13.x, 9.2.12.x, 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.455 CR |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Proposal 1, gNB-DU should notify gNB-CU PosMeasGapPreConfig so that the gNB-CU can send preconfigured measurement gaps information to UE.

==== Start of change for proposal 1====

8.13.18	Measurement Preconfiguration

8.13.18.1	General

The Measurement Preconfiguration procedure allows the gNB-CU to provide necessary information to the serving gNB-DU and request the gNB-DU to configure measurement gap or PRS processing window of the UE.

8.13.18.2	Successful Operation

Figure 8.13.18.2-1: Measurement Preconfiguration procedure, successful operation

The gNB-CU initiates the procedure by sending a MEASUREMENT PRECONFIGURATION REQUIRED message.

If the gNB-DU is able to configure measurement gap or PRS processing window, it shall reply with the MEASUREMENT PRECONFIGURATION CONFIRM message.

If the TRP PRS Information List IE is included in the MEASUREMENT PRECONFIGURATION REQUIRED message, the gNB-DU shall generate the preconfigured measurement gaps based on the received TRP PRS information. Then the gNB-DU shall send the preconfigured measurement gaps information to the gNB-CU in the PosMeasGapPreConfigList IE that is included in the MEASUREMENT PRECONFIGURATION COMFIRM message.

==== Next Change ====

9.2.12.31	MEASUREMENT PRECONFIGURATION CONFIRM

This message is sent by an gNB-DU to gNB-CU to confirm successful configuration of measurement gap or PRS processing window of the UE.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| PosMeasGapPreConfigList | O |  | OCTET STRING | PosMeasGapPreConfigToAddModList-r17 and/or PosMeasGapPreConfigToRemoveList-r17 as defined in TS 38.331 [8]. | YES | ignore |

==== End of change for proposal 1 ====

Proposal 2, gNB-DU should notify the gNB-CU the activated preconfigured MG by UE initiation.

==== Start of change for proposal 2 ====

8.13.x	Measurement Activation Notify

8.13.x.1	General

The purpose of the Measurement Activation Notify procedure is to enable the gNB-DU to inform the gNB-CU that the pre-configured measuremernt gap is activated or deactivated by UE initiation. The procedure uses UE-associated signalling.

8.13.x.2	Successful Operation

Figure 8.13.x.2-1: Measurement Activation Notify procedure, successful operation

The gNB-DU initiates the procedure by sending a MEASUREMENT ACTIVATION NOTIFY message.

The MEASUREMENT ACTIVATION NOTIFY message shall contain the pre-configured measurement gap information that is activated or deactivated by UE initiation.

Upon reception of the MEASUREMENT ACTIVATION NOTIFY message, the gNB-CU shall consider the pre-confiured measurement gap is activated or deactivated by UE initiation.

8.13.x.3	Unsuccessful Operation

Not Applicable.

==== Next Change ====

#### 9.2.12.x	MEASUREMENT ACTIVATION NOTIFY

This message is sent by the gNB-DU to indicate the gNB-CU the preconfigured measurement gap activated or dactivated by UE initiation.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Measurement Activation Indication | M |  | ENUMERATED (Active, Deactivate, ...) |  | YES | reject |
| PosMeasGapPreConfig | O |  | OCTET STRING | PosMeasGapPreConfig as defined in TS 38.331 [8], indicates preconfigured measurement gap configuration that is activated or deaciveted by UE initiation | YES | ignore |

==== ASN.1 Change for proposal 1 ====

UEPagingCapability,

GNBDUUESliceMaximumBitRateList,

MDTPollutedMeasurementIndicator,

PosMeasGapPreConfigList

==== next change ====

id-PEIPSAssistanceInfo,

id-UEPagingCapability,

id-GNBDUUESliceMaximumBitRateList,

id-PosMeasGapPreConfigList,

maxCellingNBDU,

==== next change ====

-- **************************************************************

--

-- Positioning Preconfiguration Confirm

--

-- **************************************************************

MeasurementPreconfigurationConfirm ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { MeasurementPreconfigurationConfirm-IEs} },

...

}

MeasurementPreconfigurationConfirm-IEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID	CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID	CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID			PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics	CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional }|

{ ID id-PosMeasGapPreConfigList	CRITICALITY ignore	TYPE PosMeasGapPreConfigList		PRESENCE optional },

...

}

==== next change ====

PosMeasGapPreConfigList ::= OCTET STRING

==== next change ====

id-SIB17-message									ProtocolIE-ID ::= 625

id-GNBDUUESliceMaximumBitRateList					ProtocolIE-ID ::= 626

id-PosMeasGapPreConfigList	                    	ProtocolIE-ID ::= xxx

==== end of ANS.1 change for proposal 1 ====

==== ASN.1 Change for proposal 2 ====

MeasurementPreconfigurationRequired,

MeasurementPreconfigurationConfirm,

MeasurementPreconfigurationRefuse,

MeasurementActivation,

QoEInformationTransfer,

MeasurementActivationNotify

==== next change ====

id-measurementPreconfiguration,

id-measurementActivation,

id-QoEInformationTransfer,

id-MeasurementActivationNotify

==== next change ====

F1AP-ELEMENTARY-PROCEDURES-CLASS-2 F1AP-ELEMENTARY-PROCEDURE ::= {

errorIndication							|

uEContextReleaseRequest					|

dLRRCMessageTransfer					|

uLRRCMessageTransfer					|

uEInactivityNotification				|

privateMessage							|

initialULRRCMessageTransfer				|

systemInformationDelivery				|

paging									|

notify									|

pWSRestartIndication					|

pWSFailureIndication					|

gNBDUStatusIndication					|

rRCDeliveryReport						|

networkAccessRateReduction				|

traceStart								|

deactivateTrace							|

dUCURadioInformationTransfer			|

cUDURadioInformationTransfer			|

resourceStatusReporting					|

accessAndMobilityIndication				|

referenceTimeInformationReportingControl|

referenceTimeInformationReport			|

accessSuccess							|

cellTrafficTrace						|

positioningAssistanceInformationControl		|

positioningAssistanceInformationFeedback	|

positioningMeasurementReport				|

positioningMeasurementAbort					|

positioningMeasurementFailureIndication		|

positioningMeasurementUpdate				|

positioningDeactivation						|

e-CIDMeasurementFailureIndication			|

e-CIDMeasurementReport						|

e-CIDMeasurementTermination					|

positioningInformationUpdate				|

multicastGroupPaging						|

broadcastContextReleaseRequest				|

multicastContextReleaseRequest				|

pDCMeasurementReport						|

measurementActivation						|

qoEInformationTransfer						|

measurementActivationNotify,

...

}

==== next change ====

qoEInformationTransfer F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		QoEInformationTransfer

PROCEDURE CODE			id-QoEInformationTransfer

CRITICALITY				ignore

}

measurementActivationNotify F1AP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		MeasurementActivationNotify

PROCEDURE CODE			id-MeasurementActivationNotify

CRITICALITY				ignore

}

==== next change ====

id-PEIPSAssistanceInfo,

id-UEPagingCapability,

id-GNBDUUESliceMaximumBitRateList,

id-MeasurementActivationIndication,

id-PosMeasGapPreConfig,

maxCellingNBDU,

==== next change ====

-- **************************************************************

--

-- Measurement Activation Notify

--

-- **************************************************************

MeasurementActivationNotify ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ MeasurementActivationNotifyIEs}},

...

}

MeasurementActivationNotifyIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID					CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID						PRESENCE mandatory	}|

{ ID id-MeasurementActivationIndication		CRITICALITY reject	TYPE MeasurementActivationIndication	PRESENCE mandatory	}|

{ ID id-PosMeasGapPreConfig					CRITICALITY ignore	TYPE PosMeasGapPreConfig					PRESENCE optional	   },

...

}

MeasurementActivationIndication ::= ENUMERATED (Active, Deactivate, ...)

PosMeasGapPreConfig ::= OCTET STRING

==== next change ====

id-SIB17-message									ProtocolIE-ID ::= 625

id-GNBDUUESliceMaximumBitRateList					ProtocolIE-ID ::= 626

id-MeasurementActivationIndication, 				ProtocolIE-ID ::= yyy

id-PosMeasGapPreConfig	                    		ProtocolIE-ID ::= zzz

==== end of ANS.1 change for proposal 1 ====