3GPP TSG-RAN WG3 Meeting #116-e	R3-223582

# 9 May – 19 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 835 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enable configuration of Network Slice Groups |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Slice |  |  |  |  |  | Date: |  |  | 2022-4-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At post email discussion, RAN2 achieve consensus on Slice list and priority information for cell reselection in [R2-2203933]. Based on the requirements, SA2#150 also agreed to support the feature. It is necessary to introduce Slice group configuration in specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce Network Slice Groups IE in Slice Support List IE and Extended Slice Support List in NGAP NG SETUP REQUEST and RAN CONFIGURATION UPDATE message. The CR has no BC impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 Slice Grouping and Slice Priority can not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.1.2, 8.7.2.2, 9.3.1.17, 9.3.1.191, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR833 TS 38.473 CR946 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the NG SETUP REQUEST message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

--Skip unchanged part

If the Network Slice Groups IE in Slice Support List IE or in Extended Slice Support List is included in the NG SETUP REQUEST message, the AMF shall take it into account for configuration of Network Slice Groups as specified in TS 23.501 [9].

Next Change

8.7.2.2	Successful Operation

Figure 8.7.2.2-1: RAN configuration update: successful operation

The NG-RAN node initiates the procedure by sending a RAN CONFIGURATION UPDATE message to the AMF including an appropriate set of updated configuration data that it has just taken into operational use. The AMF responds with a RAN CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the RAN CONFIGURATION UPDATE message, the AMF shall interpret that the corresponding configuration data is not changed and shall continue to operate the NG-C interface with the existing related configuration data.

If the Supported TA List IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall overwrite the whole list of supported TAs and the corresponding list of supported slices for each TA, and use them for subsequent registration area management of the UE. If the Network Slice Groups IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall take it into account for configuration of Network Slice Groups as specified in TS 23.501 [9].

--Skip unchanged part

Next Change

9.3.1.17	Slice Support List

This IE indicates the list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Slice Support Item |  | 1..<maxnoofSliceItems> |  |  | - |  |
| >S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| >Network Slice Groups |  | 0..<maxnoofNetworkSliceGroups> |  |  | Yes | ignore |
| >>Network Slice Group ID | M |  | OCTET STRING (SIZE(1)) | This IE indicates the network slice group ID for either cell reselection or RACH purpose of the S-NSSAI. This IE is encoded in the same format as theSliceGroupID-r17 IE as defined in TS 38.331 [18]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNetworkSliceGroups | Maximum no. of network slicing groups. Value is 2. |

Next Change

9.3.1.191	Extended Slice Support List

This IE indicates a list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Slice Support Item |  | 1..<maxnoofExtSliceItems> |  |  | - |  |
| >S-NSSAI | M |  | 9.3.1.24 |  | - |  |
| >Network Slice Groups |  | 0..<maxnoofNetworkSliceGroups> |  |  | Yes | ignore |
| >>Network Slice Group ID | M |  | OCTET STRING (SIZE(1)) | This IE indicates the network slice group ID for either cell reselection or RACH purpose of the S-NSSAI. This IE is encoded in the same format as theSliceGroupID-r17 IE as defined in TS 38.331 [18]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofExtSliceItems | Maximum no. of signalled slice support items. Value is 65535. |
| maxnoofNetworkSliceGroups | Maximum no. of network slicing groups. Value is 2. |

Next Change

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-Alternative-SharedNG-U-Multicast-TNL-Information,

id-AlternativeQoSParaSetList,

id-BurstArrivalTimeDownlink,

id-Cause,

id-CNPacketDelayBudgetDL,

id-CNPacketDelayBudgetUL,

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

--skip unchanged part

id-UserLocationInformationTNGF,

id-UserLocationInformationTWIF,

id-UserLocationInformationW-AGF,

id-NetworkSliceGroups,

--Skip unchanged part

maxnoofTargetS-NSSAIs,

maxnoofTimePeriods,

maxnoofTNLAssociations,

maxnoofUEAppLayerMeas,

maxnoofUEsforPaging,

maxnoofWLANName,

maxnoofXnExtTLAs,

maxnoofXnGTP-TLAs,

maxnoofXnTLAs,

maxnoofNetworkSliceGroups

Next Change

SliceSupportList ::= SEQUENCE (SIZE(1..maxnoofSliceItems)) OF SliceSupportItem

SliceSupportItem ::= SEQUENCE {

s-NSSAI				S-NSSAI,

iE-Extensions		ProtocolExtensionContainer { {SliceSupportItem-ExtIEs} }	OPTIONAL,

...

}

SliceSupportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ ID id-NetworkSliceGroups	CRITICALITY ignore	EXTENSION NetworkSliceGroups		PRESENCE optional },

...

}

NetworkSliceGroups ::= SEQUENCE (SIZE(1..maxnoofNetworkSliceGroups)) OF NetworkSliceGroupID

NetworkSliceGroupID ::= OCTET STRING (SIZE(1))

Next Change

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

--skip unchanged part

maxnoofXnGTP-TLAs					INTEGER ::= 16

maxnoofXnTLAs						INTEGER ::= 2

maxnoofCandidateCells				INTEGER ::= 32

maxnoofTargetS-NSSAIs				INTEGER ::= 8

maxNRARFCN							INTEGER ::= 3279165

maxnoofCellIDforQMC					INTEGER ::= 32

maxnoofPLMNforQMC					INTEGER ::= 16

maxnoofUEAppLayerMeas				INTEGER ::= 16

maxnoofSNSSAIforQMC					INTEGER ::= 16

maxnoofTAforQMC						INTEGER ::= 8

maxnoofNetworkSliceGroups			INTEGER ::= 2

--skip unchanged part

id-PagingCauseIndicationForVoiceService					ProtocolIE-ID ::= 343

id-PEIPSassistanceInformation							ProtocolIE-ID ::= 344

id-FiveG-ProSeAuthorized								ProtocolIE-ID ::= 345

id-FiveG-ProSeUEPC5AggregateMaximumBitRate				ProtocolIE-ID ::= 346

id-FiveG-ProSePC5QoSParameters							ProtocolIE-ID ::= 347

id-NetworkSliceGroups							        ProtocolIE-ID ::= 3xx

END

-- ASN1STOP

End of Change