3GPP TSG-RAN WG3 Meeting #116-e	R3-223583

# 9 May – 19 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 946 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enable configuration of Network Slice Groups |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Slice |  |  |  |  |  | Date: |  |  | 2022-4-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At post email discussion, RAN2 achieve consensus on Slice list and priority information for cell reselection in [R2-2203933]. Based on the requirements, SA2#150 also agreed to support the feature. It is necessary to introduce Slice group configuration in specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce Network Slice Groups IE in Slice Support List IE and Extended Slice Support List IE. The CR has no BC impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 Slice Grouping and Slice Priority can not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.137, 9.3.1.165, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.423 CR833 TS 38.413 CR835 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

9.3.1.37	Slice Support List

This IE indicates the list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Slice Support Item IEs |  | 1..<maxnoofSliceItems> |  |  |  |  |
| >S-NSSAI | M |  | 9.3.1.38 |  |  |  |
| >Network Slice Groups |  | 0..<maxnoofNetworkSliceGroups> |  |  | Yes | ignore |
| >>Network Slice Group ID | M |  | OCTET STRING (SIZE(1)) | This IE indicates the network slice group ID for either cell reselection or RACH purpose of the S-NSSAI. This IE is encoded in the same format as theSliceGroupID-r17 IE as defined in TS 38.331 [8]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNetworkSliceGroups | Maximum no. of network slicing groups. Value is 2. |

Next Change

9.3.1.165	Extended Slice Support List

This IE indicates a list of supported slices.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Slice Support Item IEs |  | 1..<maxnoofExtSliceItems> |  |  |  |  |
| >S-NSSAI | M |  | 9.3.1.38 |  |  |  |
| >Network Slice Groups |  | 0..<maxnoofNetworkSliceGroups> |  |  | Yes | ignore |
| >>Network Slice Group ID | M |  | OCTET STRING (SIZE(1)) | This IE indicates the network slice group ID for either cell reselection or RACH purpose of the S-NSSAI. This IE is encoded in the same format as theSliceGroupID-r17 IE as defined in TS 38.331 [8]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofExtSliceItems | Maximum no. of signalled slice support items. Value is 65535. |
| maxnoofNetworkSliceGroups | Maximum no. of network slicing groups. Value is 2. |

Next Change

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

--skip unchanged part

id-TransmissionStopIndicator,

id-SrsFrequency,

id-EstimatedArrivalProbability,

id-TRPType,

id-SRSSpatialRelationPerSRSResource,

id-NetworkSliceGroups,

--skip unchanged part

maxnoSRS-Carriers,

maxnoSCSs,

maxnoSRS-Resources,

maxnoSRS-PosResources,

maxnoSRS-PosResourceSets,

maxnoSRS-PosResourcePerSet,

maxnoofPRS-ResourceSets,

maxnoofPRS-ResourcesPerSet,

maxNoOfMeasTRPs,

maxnoofPRSresourceSets,

maxnoofPRSresources,

maxnoofNetworkSliceGroups

Next Change

SliceSupportList ::= SEQUENCE (SIZE(1.. maxnoofSliceItems)) OF SliceSupportItem

SliceSupportItem ::= SEQUENCE {

sNSSAI	SNSSAI,

iE-Extensions				ProtocolExtensionContainer { { SliceSupportItem-ExtIEs } }	OPTIONAL

}

SliceSupportItem-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NetworkSliceGroups	CRITICALITY ignore	EXTENSION NetworkSliceGroups		PRESENCE optional },

...

}

NetworkSliceGroups ::= SEQUENCE (SIZE(1..maxnoofNetworkSliceGroups)) OF NetworkSliceGroupID

NetworkSliceGroupID ::= OCTET STRING (SIZE(1))

Next Change

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

--skip unchanged part

maxnoofPRS-ResourceSets					INTEGER ::= 2

maxnoofPRS-ResourcesPerSet				INTEGER ::= 64

maxNoOfMeasTRPs							INTEGER ::= 64

maxnoofPRSresourceSets					INTEGER ::= 8

maxnoofPRSresources						INTEGER ::= 64

--skip unchanged part

id-SCGIndicator										ProtocolIE-ID ::= 432

id-EstimatedArrivalProbability						ProtocolIE-ID ::= 433

id-TRPType											ProtocolIE-ID ::= 434

id-SRSSpatialRelationPerSRSResource					ProtocolIE-ID ::= 435

id-NetworkSliceGroups							    ProtocolIE-ID ::= 4xx

END

-- ASN1STOP

End of Change