3GPP TSG-RAN WG3 Meeting #116-e	R3-223584

# 9 May – 19 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 833 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enable configuration of Network Slice AS Groups |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Slice |  |  |  |  |  | Date: |  |  | 2022-4-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | At post email discussion, RAN2 achieve consensus on Slice list and priority information for cell reselection in [R2-2203933]. Based on the requirements, SA2#150 also agreed to support the feature. It is necessary to introduce Slice group configuration in specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce Cell Reselection Network Slice Group ID IE in S-NSSAI IE. The CR has no BC impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 Slice Grouping and Slice Priority can not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.21, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR835 TS 38.473 CR946 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change

9.2.3.21	S-NSSAI

This IE contains the S-NSSAI as defined in TS 23.003 [22] and the associated network slice group ID.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SST | M |  | OCTET STRING (SIZE(1)) |  | - |  |
| SD | O |  | OCTET STRING (SIZE(3)) |  | - |  |
| >Cell Reselection Network Slice Group ID | O |  | OCTET STRING (SIZE(1)) | This IE indicates the network slice group ID for cell reselection purpose of the S-NSSAI. This IE encoded in the same format as theSliceGroupID-r17 IE as defined in TS 38.331 [10]. | Yes | ignore |

Next Change

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

--skip unchanged part

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-SFN-Offset,

id-QoS-Mapping-Information,

id-AdditionLocationInformation,

id-dataForwardingInfoFromTargetE-UTRANnode,

id-NetworkSliceGroupID,

Next Change

S-NSSAI ::= SEQUENCE {

sst						OCTET STRING (SIZE(1)),

sd						OCTET STRING (SIZE(3))							OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { {S-NSSAI-ExtIEs} } OPTIONAL,

...

}

- NSSAI-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

- { ID id-NetworkSliceGroupID	CRITICALITY ignore	EXTENSION NetworkSliceGroupID 		PRESENCE optional },

...

}

NetworkSliceGroupID ::= OCTET STRING (SIZE(1))

Next Change

id-UESpecificDRX																					ProtocolIE-ID ::= 248

id-PDUSessionExpectedUEActivityBehaviour															ProtocolIE-ID ::= 249

id-QoS-Mapping-Information																			ProtocolIE-ID ::= 250

id-AdditionLocationInformation																		ProtocolIE-ID ::= 251

id-dataForwardingInfoFromTargetE-UTRANnode															ProtocolIE-ID ::= 252

id-NetworkSliceGroupID															                  ProtocolIE-ID ::= 2XX

END

-- ASN1STOP

End of Change