3GPP TSG-RAN WG3 Meeting #116-e	R3-223594

# E-meeting, 09 May – 19 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.470 | CR | 0097 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting the disaster roaming information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Orange, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | CT1 and SA2 has specifed a feature referred to as MINT (Minimization of Service Interruption). This feature allows UEs of PLMN which is experiencing so called "disaster conditions" to roam in other networks. Such type of roaming is called disaster roaming. In TS 38.331, RAN2 agrees to introduce SIB15 containing Provision of disaster roaming information, which enables network to indicate which PLMNs' UEs are allowed to do disaster roaming. This CR provides the functional support of the MINT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add description texts of SIB15 accordingly. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). The impact can be considered isolated because the change only conerns the gNB-DU System Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MINT feature not supported in CU/DU architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS38.473 CR0949 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Change Begins

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BH	Backhaul

DRB	Data Radio Bearers

eDRX	extended Discontinuous Reception

F1-U	F1 User plane interface

F1-C	F1 Control plane interface

F1AP	F1 Application Protocol

GTP-U	GPRS Tunnelling Protocol

IAB	Integrated Access and Backhaul

IP	Internet Protocol

L2	Layer-2

MBS	Multicast/Broadcast Service

NR-MIB	NR-Master Information Block

O&M	Operation and Maintenance

PA	Paging Area

PDC	Propagation Delay Compensation

PF	Paging Frame

PH	Paging Hyperframes

PO	Paging Occasion

PTP	Point to Point

PTM	Point to Multipoint

QoE	Quality of Experience

QoS	Quality of Service

RedCap	Reduced Capability

RIM	Remote Interference Management

RLC	Radio Link Control

RRC	Radio Resource Control

SCTP	Stream Control Transmission Protocol

SRB	Signalling Radio Bearers

SIB1	System Information Block 1

SIB10	System Information Block 10

SIB12	System Information Block 12

SIB13	System Information Block 13

SIB14	System Information Block 14

SIB15	System Information Block 15

SIB17	System Information Block 17

SL	Sidelink

TNL	Transport Network Layer

U2N 	UE-to-Network

V2X	Vehicle-to-Everything

## 5.2	F1-C functions

### 5.2.2	System Information management function

Scheduling of system broadcast information is carried out in the gNB-DU. The gNB-DU is responsible for transmitting the system information according to the scheduling parameters available.

The gNB-DU is responsible for the encoding of the NR-MIB message. In case broadcast of SIB1 and other SIBs is needed, the gNB-DU is responsible for the encoding of the SIB1 message, SIB10, SIB12, SIB13, SIB14, SIB15 and SIB17 and the gNB-CU is responsible for the encoding of other SIBs. The gNB-DU may re-encode SIB9. The gNB-DU is responsible for the generation of the SystemInformation message.

The gNB-CU is responsible for receiving the positioning assistance information from LMF, e.g the positioning related SIBs. The gNB-CU notifies gNB-DU about the SIBs, and the gNB-DU signals them directly.

To support Msg3 based on-demand SI as described in TS 38.331 [11], the gNB-CU can confirm the received SI request from the UE by including the UE identity, and command the gNB-DU to broadcast the requested other SIs.

Change Ends