3GPP TSG-RAN WG3 #116-e	R3-223610

# Online, 9 May – 19 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0836 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RA-SDT without anchor relocation in Xn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Core |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the RAN2 agreement, the SDT can be configured only for SRB without resumption of SDT DRB. Agreement in RAN2 #117-e: 33. SDT can be configured only for SRB (i.e. without resuming any DRB). Define the sdt-DRB-List-r17 with the list starting in 0, or as SetupRelease type. … RRC Release in TS 38.331: SDT-Config-r17 ::= SEQUENCE { sdt-DRB-List-r17 SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity OPTIONAL, -- Need M sdt-SRB2-Indication-r17 ENUMERATED {allowed} OPTIONAL, -- Need R sdt-MAC-PHY-CG-Config-r17 SetupRelease {SDT-CG-Config} OPTIONAL, -- Need M sdt-DRB-ContinueROHC-r17 ENUMERATED { cell, rna } OPTIONAL -- Need R }. However, in current XnAP specification, the SDT DRBs To Be Setup List IE is mandatory in the Partial UE Context Information for SDT IE. This means that in RA-SDT without UE context relocation, even though the anchor gNB configure the UE only with the SDT SRB in the RRC Release message, the SDT DRB related information is always provided to the new gNB. Therefore, a new indication is optionally included into the Partial UE Context Information for SDT IE in order to indicate the new gNB that the SDT is configured only for SRB without resumption of SDT DRB. Based on this new indicator, the new gNB can ignore the SDT DRBs To Be Setup List IE in the Partial UE Context Information for SDT IE. |
| Summary of change: |  | - Add the SDT SRB Only Indicator IE in the Partial UE Context Information for SDT IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Small Data Transmission cannot be configured only for SRB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.164, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------Start of the First Change-----------------------------

9.2.3.164	Partial UE Context Information for SDT

This IE contains the UE context information within the PARTIAL UE CONTEXT TRANSFER message for NR SDT.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SDT DRBs To Be Setup List |  | 1 |  |  | YES | ignore |
| >SDT DRBs to Be Setup Item |  | 1 .. <maxnoofDRBs> |  |  | – |  |
| >>DRB ID | M |  | 9.2.3.33 |  | – |  |
| >>UL TNL Information | M |  | UP Transport Parameters 9.2.3. 76 |  | – |  |
| >>DRB RLC Bearer Configuration | M |  | OCTET STRING | RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [10] | – |  |
| >>DRB QoS | M |  | QoS Flow Level QoS Parameters 9.2.3.5 |  | – |  |
| >>S-NSSAI | M |  | 9.2.3.21 |  | – |  |
| >>RLC Mode | M |  | 9.2.3.28 |  | – |  |
| >>PDCP SN Length | M |  | 9.2.3.63 |  | – |  |
| SDT SRBs to Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SDT SRBs to Be Setup Item |  | 1 .. <maxnoofSRBs> |  |  | – |  |
| >>SRB ID | M |  | 9.2.3.165 | In this version of the specification, values "0", "3", and "4" shall not be set by the sender and ignored by the receiver. | – |  |
| >>SRB RLC Bearer Configuration | M |  | OCTET STRING | RLC-BearerConfig IE defined in subclause 6.3.2 of TS 38.331 [10]. | – |  |
| SDT SRB Only Indicator | O |  | ENUMERATED (true,…) | Indicates whether SDT is configured only for SRB. If included, the SDT SRBs to Be Setup List IE shall be present and the SDT DRBs To Be Setup List IE shall be ignored. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 32. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 5. |

<Unchanged Text Omitted>

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

[Unchanged text skipped]

id-Redcap-Bcast-Information,

id-UESliceMaximumBitRateList,

id-PositioningInformation,

id-ServedCellSpecificInfoReq-NR,

id-SDTSRBOnlyIndicator,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

[Unchanged text skipped]

SDTPartialUEContextInfo ::= SEQUENCE {

dRBsToBeSetup					SDT-DRBsToBeSetupList,

sRBsToBeSetup					SDT-SRBsToBeSetupList		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { SDTPartialUEContextInfo-ExtIEs} } 	OPTIONAL,

...

}

SDTPartialUEContextInfo-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SDTSRBOnlyIndicator		CRITICALITY ignore	EXTENSION SDTSRBOnlyIndicator		PRESENCE optional},

...

}

SDTSRBOnlyIndicator ::= ENUMERATED {true, ...}

<Unchanged Text Omitted>

9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

[Unchanged text skipped]

id-PEIPSassistanceInformation										ProtocolIE-ID ::= 357

id-UESliceMaximumBitRateList										ProtocolIE-ID ::= 358

id-S-NG-RANnodeUE-Slice-MBR											ProtocolIE-ID ::= 359

id-PositioningInformation											ProtocolIE-ID ::= 360

id-UEAssistantIdentifier											ProtocolIE-ID ::= 361

id-SDTSRBOnlyIndicator												ProtocolIE-ID ::= xxx

--------------------------------End of the Changes-----------------------------