3GPP TSG- RAN WG3 Meeting #116-e	R3-223616

# Online, 9-19 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | - | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on stage 2 for IoT over NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clalify the eNB basically construct the Mapped Cell ID with best effort without location information and the Mapped Cell ID may define different mapped cell layers; To enhance the UE using CP CIoT EPS optimisation only, eNB may provide coarse-grained location. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the same note as NR NTN to cover the case of defining different mapped cell layers. - Clarifiy the case when UE not provided UE location for eNB. - Add new note to clarification the case that eNB may provide coarse-grained location for UE using CP CIoT EPS optimisation only. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NB-IoT/eMTC for Non-Terrestrial Networks is not supported |  |  |  |  |  |  |  |  |
| t |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.21.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modification

3.21.6	Signalling

The Cell Identity, as defined in TS 36.413 [25] and TS 36.423 [42], used in following cases corresponds to a Mapped Cell ID, irrespective of the orbit of the NTN payload or the types of service links supported.

-	The Cell Identity indicated by the eNB to the Core Network as part of the User Location Information, or as E-UTRAN CGI in the related S1AP messages;

-	The Cell Identity used for Paging Optimization in S1 interface;

-	The Cell Identity used for PWS.

For a BL UE or a UE in enhanced coverage, the Cell Identity included within the target identification of the handover messages allows identifying the correct target cell.

NOTE 1:	A specific geographical location may be mapped to multiple Mapped Cell ID(s), and such Mapped Cell IDs may be configured to indicate differerent geographical areas (e.g. overlapping and/or with different dimensions).

Next Modification

For a BL UE or a UE in enhanced coverage, the eNB is responsible for constructing the Mapped Cell ID based on the UE location info received from the UE, if available. The mapping may be pre-configured (e.g., up to operator’s policy) or up to implementation.

Next Modification

Note2: For UE using CP CIoT EPS optimisation only, to increasing the probability of selecting the correct MME eNB can provide coarse-grained location by implement (e.g., based on the satellite beam footprint), if available.

Next Modification

NOTE3:	As described in TS 23.401 [17], the User Location Information may enable the MME to determine whether the UE is allowed to operate at its present location. Pre-configuration of special mapped cell identifiers may be used to indicate areas outside the serving PLMN’s country.

The eNB reports the broadcasted TAC(s) of the selected PLMN to the MME. In case the eNB knows the UE’s location information, the eNB may determine the TAI the UE is currently located in and provide that TAI to the MME.

End of Modification