3GPP TSG-RAN WG3 Meeting #116-e	R3-223637

# May.09-Mar.19. 2022, E-meeting

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0227 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.401 on the introduction of RAN visible QoE metrics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN visible QoE metric was introduced, which will have impacts on F1 for disaggregated architecture, this is not reflected in this spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce new texts clarifying that the function of RAN visible QoE metrics may impact F1, for which gNB-CU forwards received RAN visible QoE report to gNB-DU for its scheduling consideration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The function of RAN visible QoE metric is not reflected in stage 2 spec |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

7	NG-RAN functions description

/////////////////////////////////////////////////////////////////////unchanged texts omitted///////////////////////////////////////////////////////////////////

7.x	Support of RAN visible QoE measurement

The RAN visible QoE measurement function is specified in 38.300 [2].

In split gNB architecture, upon the reception of the RAN visible QoE metric report from UE, the gNB-CU may forward it to the gNB-DU.

End change