3GPP TSG-RAN WG3 Meeting #116-e	R3-223644

# E-meeting, 9 May - 19 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0229 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for stage 2 on CPAC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Lenovo, China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-4-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-17, no new procedures for CPAC involving E1 and F1 were introduced in stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 8.16 is removed. Impact assessment towards the previous version of the specification (same release): This CR has no impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clause 8.16 is useless. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.16 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------Start of the change---------------------------------------------------

8.15.1.2	Multicast MBS Session Activation

Figure 8.15.1.2-1 illustrates an examplified interaction of NGAP, E1AP, F1AP and RRC protocol functions at Multicast MBS Session Activation.

Figure 8.15.1.2-1: Multicast MBS Session Context establishment

1.	A multicast session context is established by the 5GC.

2/3. If not yet existing, the gNB-CU-CP establishes the multicast bearer context at the gNB-CU-UP, in order to retrieve for unicast NG-U transport the GTP DL TEID, a shared resource address (GTP DL TEID).

4/5.	If applicable, the gNB-CU-CP establishes the Multicast Context at the DU, providing MRB configuration. It may contain MBS Area Session ID information.

6.	Dependent on e.g. joined UEs, the gNB-DU triggers the establishment of an F1-U tunnel, which is established either per DU or per cell or per MBS Area Session ID. The receiving gNB-CU-CP may need to fetch a gNB side NG-U TNL address information for the gNB-CU-UP by means of a E1AP MC Bearer Context Modification procedure.

7/8. The gNB-CU CP triggers the NGAP Distribution Setup procedure. For unicast transport, DL/UL GTP TEIDs are exchanged, for multicast transport, multicast address information is fetched from the 5GC.

9/10. The gNB-CU-UP side of the F1-U and NG-U UP entity is established by means of the E1AP MC Bearer Context Modification procedure, providing the DU side F1-U TNL address and the 5GC NG-U TNL address to the gNB-CU-UP, which provides the gNB-CU-UP side F1-U TNL address in return.

11.	The gNB-CU-UP side F1-U TNL address is provided to the gNB-DU.

12.	In case of NG-U multicast transport, the gNB-CU-UP joins the NG-U multicast group.

13.	The gNB-CU-CP RRC-configures each UE which has joined the multicast group.

14.	The gNB successfully terminates the NGAP procedure for establishing the multicast session context.

15.	The multicast MBS media stream is provided to the UEs.

On NG-U, in case of location dependent multicast MBS Sessions, multiple shared NG-U transport tunnels may need to be setup, one per Area Session ID served by the gNB.

In case of shared NG-U termination,

-	the gNB-CU-UP may provide the gNB-CU-CP at E1 setup or configuration update about established shared NG-U terminations, indicated by one or several MBS Session IDs.

-	at establishment of the MC bearer context in the gNB-CU-UP, the gNB-CU-UP may overwrite the QoS flow mapping indicated by the gNB-CU-CP, if the gNB-CU-CP has provided its consent to do so at MC bearer context setup.

Editor’s Note:	providing the consent from the gNB-CU-CP to the gNB-CU-UP needs further discussions.

8.16	Void

8.17	IAB Inter-CU Topology Management

8.17.1	IAB Inter-donor-DU Re-routing

When an IAB-donor-DU is configured with the information to support inter-donor-DU re-routing, the IAB-donor-DU may identify a re-routed UL IP packet based on the source IP address field of the UL packet, and forward the re-routed UL IP packet to the peer IAB-donor-DU via a tunnel. In the intra-donor-CU scenario for UL inter-donor-DU rerouting, the IAB-donor-DU and its peer IAB-donor-DU are controlled by the same IAB-donor-CU. In the inter-donor-CU case, the IAB-donor-DU and its peer IAB-donor-DU are controlled by different IAB-donor-CUs. The inter-donor-DU tunnel is a GTP-U tunnel. IAB-donor-CU configures the IAB-donor-DU with a list of TNL addresses and/or prefixes that are subject to inter-donor-DU tunneling and exempt from TNL address filtering.

NOTE:	Other tunnel types may be used for the inter-donor-DU tunnel, by implementation.

-----------------------------------------------------End of the change-------------------------------------------------------