3GPP TSG-RAN WG3 Meeting #116-e	R3-223651

# E-meeting, 09 May – 19 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0843 | rev | 0 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of broadcast session release required procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the reply from SA2, they share the same view with RAN3, supporting to introduce the RAN triggered broadcast session release procedure. In case NG-RAN is not able to provide the service over radio, due to lack of radio resources, it is beneficial for NG-RAN to notify CN to introduce a RAN initiated Broadcast Session Release Required procedure. Thus, the introduction of a non-UE associated Class2 NGAP procedure is essential. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a non-UE associated Class2 NGAP procedure named Broadcast Session Release Required procedure in the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG-RAN is not able to trigger the CN to initiate the broadcast session release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.17.x, 9.2.16.x, 9.4.3, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------Start of the change---------------------------------------------------

## 8.1	List of NGAP Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs (see subclause 3.1 for explanation of the different classes):

//Skip unchanged part

Table 8.1-2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Downlink RAN Configuration Transfer | DOWNLINK RAN CONFIGURATION TRANSFER |
| Downlink RAN Status Transfer | DOWNLINK RAN STATUS TRANSFER |
| Downlink NAS Transport | DOWNLINK NAS TRANSPORT |
| Error Indication | ERROR INDICATION |
| Uplink RAN Configuration Transfer | UPLINK RAN CONFIGURATION TRANSFER |
| Uplink RAN Status Transfer | UPLINK RAN STATUS TRANSFER |
| Handover Notification | HANDOVER NOTIFY |
| Initial UE Message | INITIAL UE MESSAGE |
| NAS Non Delivery Indication | NAS NON DELIVERY INDICATION |
| Paging | PAGING |
| PDU Session Resource Notify | PDU SESSION RESOURCE NOTIFY |
| Reroute NAS Request | REROUTE NAS REQUEST |
| UE Context Release Request | UE CONTEXT RELEASE REQUEST |
| Uplink NAS Transport | UPLINK NAS TRANSPORT |
| AMF Status Indication | AMF STATUS INDICATION |
| PWS Restart Indication | PWS RESTART INDICATION |
| PWS Failure Indication | PWS FAILURE INDICATION |
| Downlink UE Associated NRPPa Transport | DOWNLINK UE ASSOCIATED NRPPA TRANSPORT |
| Uplink UE Associated NRPPa Transport | UPLINK UE ASSOCIATED NRPPA TRANSPORT |
| Downlink Non UE Associated NRPPa Transport | DOWNLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Uplink Non UE Associated NRPPa Transport | UPLINK NON UE ASSOCIATED NRPPA TRANSPORT |
| Trace Start | TRACE START |
| Trace Failure Indication | TRACE FAILURE INDICATION |
| Deactivate Trace | DEACTIVATE TRACE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| Location Reporting Control | LOCATION REPORTING CONTROL |
| Location Reporting Failure Indication | LOCATION REPORTING FAILURE INDICATION |
| Location Report | LOCATION REPORT |
| UE TNLA Binding Release | UE TNLA BINDING RELEASE REQUEST |
| UE Radio Capability Info Indication | UE RADIO CAPABILITY INFO INDICATION |
| RRC Inactive Transition Report | RRC INACTIVE TRANSITION REPORT |
| Overload Start | OVERLOAD START |
| Overload Stop | OVERLOAD STOP |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Uplink RIM Information Transfer | UPLINK RIM INFORMATION TRANSFER |
| Downlink RIM Information Transfer | DOWNLINK RIM INFORMATION TRANSFER |
| Retrieve UE Information | RETRIEVE UE INFORMATION |
| UE Information Transfer | UE INFORMATION TRANSFER |
| RAN CP Relocation Indication | RAN CP RELOCATION INDICATION |
| Connection Establishment Indication | CONNECTION ESTABLISHMENT INDICATION |
| AMF CP Relocation Indication | AMF CP RELOCATION INDICATION |
| Handover Success | HANDOVER SUCCESS |
| Uplink RAN Early Status Transfer | UPLINK RAN EARLY STATUS TRANSFER |
| Downlink RAN Early Status Transfer | DOWNLINK RAN EARLY STATUS TRANSFER |
| Multicast Group Paging | MULTICAST GROUP PAGING |
| Broadcast Session Released Required | BROADCAST SESSION RELEASE REQUIRED |

------------------------------------------------------Next change-------------------------------------------------------

8.17.x		Broadcast Session Release Required

8.17.x.1	General

The purpose of the Broadcast Session Release Required procedure is to trigger the CN to release the MBS context corresponding to the previous established MBS session. The procedure uses non-UE associated signalling.

8.17.x.2	Successful Operation

Figure 8.17.X.2-1: Broadcast Session Release Required, successful operation.

The NG-RAN node initiates the procedure by sending a BROADCAST SESSION RELEASE REQUIRED message to the AMF.

Upon receiption of the BROADCAST SESSION RELEASE RLEASE REQUIRED message, the AMF shall realize there is no adequate resources for RAN to provide the broadcast service over radio and start to release the MBS context corresponding to the previous established MBS session.

8.17.x.3	Unsuccessful Operation

Void.

8.17.x.4	Abnormal Conditions

Void.

------------------------------------------------------Next change-------------------------------------------------------

9.2.16.x	BROADCAST SESSION RELEASE REQUIRED

This message is sent by the NG-RAN node to trigger the AMF to release corresponding MBS context and the MBS-service-associated logical NG connection.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

------------------------------------------------------Next change-------------------------------------------------------

9.4.3	Elementary Procedure Definitions

-- ASN1START

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

NGAP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM NGAP-CommonDataTypes

AMFConfigurationUpdate,

AMFConfigurationUpdateAcknowledge,

AMFConfigurationUpdateFailure,

AMFCPRelocationIndication,

AMFStatusIndication,

BroadcastSessionModificationFailure,

BroadcastSessionModificationRequest,

BroadcastSessionModificationResponse,

BroadcastSessionReleaseRequest,

BroadcastSessionReleaseRequired,

BroadcastSessionReleaseResponse,

BroadcastSessionSetupFailure,

BroadcastSessionSetupRequest,

BroadcastSessionSetupResponse,

CellTrafficTrace,

ConnectionEstablishmentIndication,

DeactivateTrace,

DistributionReleaseRequest,

DistributionReleaseResponse,

DistributionSetupFailure,

DistributionSetupRequest,

DistributionSetupResponse,

//Skip unchanged part

FROM NGAP-PDU-Contents

id-AMFConfigurationUpdate,

id-AMFCPRelocationIndication,

id-AMFStatusIndication,

id-BroadcastSessionModification,

id-BroadcastSessionRelease,

id-BroadcastSessionReleaseRequired,

id-BroadcastSessionSetup,

id-CellTrafficTrace,

id-ConnectionEstablishmentIndication,

id-DeactivateTrace,

id-DistributionSetup,

id-DistributionRelease,

id-DownlinkNASTransport,

id-DownlinkNonUEAssociatedNRPPaTransport,

id-DownlinkRANConfigurationTransfer,

id-DownlinkRANEarlyStatusTransfer,

id-DownlinkRANStatusTransfer,

id-DownlinkUEAssociatedNRPPaTransport,

id-ErrorIndication,

id-HandoverCancel,

id-HandoverNotification,

id-HandoverPreparation,

id-HandoverResourceAllocation,

id-HandoverSuccess,

id-InitialContextSetup,

id-InitialUEMessage,

id-LocationReport,

id-LocationReportingControl,

id-LocationReportingFailureIndication,

id-MulticastSessionActivation,

id-MulticastSessionDeactivation,

id-MulticastSessionUpdate,

id-MulticastGroupPaging,

id-NASNonDeliveryIndication,

id-NGReset,

id-NGSetup,

id-OverloadStart,

id-OverloadStop,

id-Paging,

id-PathSwitchRequest,

id-PDUSessionResourceModify,

id-PDUSessionResourceModifyIndication,

id-PDUSessionResourceNotify,

id-PDUSessionResourceRelease,

id-PDUSessionResourceSetup,

id-PrivateMessage,

id-PWSCancel,

id-PWSFailureIndication,

id-PWSRestartIndication,

id-RANConfigurationUpdate,

id-RANCPRelocationIndication,

id-RerouteNASRequest,

id-RetrieveUEInformation,

id-RRCInactiveTransitionReport,

id-SecondaryRATDataUsageReport,

id-TraceFailureIndication,

id-TraceStart,

id-UEContextModification,

id-UEContextRelease,

id-UEContextReleaseRequest,

id-UEContextResume,

id-UEContextSuspend,

id-UEInformationTransfer,

id-UERadioCapabilityCheck,

id-UERadioCapabilityIDMapping,

id-UERadioCapabilityInfoIndication,

id-UETNLABindingRelease,

id-UplinkNASTransport,

id-UplinkNonUEAssociatedNRPPaTransport,

id-UplinkRANConfigurationTransfer,

id-UplinkRANEarlyStatusTransfer,

id-UplinkRANStatusTransfer,

id-UplinkUEAssociatedNRPPaTransport,

id-WriteReplaceWarning,

id-UplinkRIMInformationTransfer,

id-DownlinkRIMInformationTransfer

FROM NGAP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode	UNIQUE,

&criticality				Criticality	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

NGAP-PDU ::= CHOICE {

initiatingMessage			InitiatingMessage,

successfulOutcome			SuccessfulOutcome,

unsuccessfulOutcome			UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	NGAP-ELEMENTARY-PROCEDURE.&procedureCode		({NGAP-ELEMENTARY-PROCEDURES}),

criticality		NGAP-ELEMENTARY-PROCEDURE.&criticality			({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value			NGAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({NGAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

NGAP-ELEMENTARY-PROCEDURES NGAP-ELEMENTARY-PROCEDURE ::= {

NGAP-ELEMENTARY-PROCEDURES-CLASS-1			|

NGAP-ELEMENTARY-PROCEDURES-CLASS-2,

...

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-1 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFConfigurationUpdate			|

broadcastSessionModification	|

broadcastSessionRelease			|

broadcastSessionSetup			|

distributionSetup				|

distributionRelease				|

handoverCancel					|

handoverPreparation				|

handoverResourceAllocation		|

initialContextSetup				|

multicastSessionActivation					|

multicastSessionDeactivation				|

multicastSessionUpdate			|

nGReset							|

nGSetup							|

pathSwitchRequest				|

pDUSessionResourceModify		|

pDUSessionResourceModifyIndication			|

pDUSessionResourceRelease		|

pDUSessionResourceSetup			|

pWSCancel						|

rANConfigurationUpdate			|

uEContextModification			|

uEContextRelease				|

uEContextResume								|

uEContextSuspend							|

uERadioCapabilityCheck			|

uERadioCapabilityIDMapping		|

writeReplaceWarning,

...

}

NGAP-ELEMENTARY-PROCEDURES-CLASS-2 NGAP-ELEMENTARY-PROCEDURE ::= {

aMFCPRelocationIndication		|

aMFStatusIndication				|

broadcastSessionRleaseRequired              |

cellTrafficTrace				|

connectionEstablishmentIndication	|

deactivateTrace					|

downlinkNASTransport			|

downlinkNonUEAssociatedNRPPaTransport	|

downlinkRANConfigurationTransfer	|

downlinkRANEarlyStatusTransfer	|

downlinkRANStatusTransfer		|

downlinkRIMInformationTransfer	|

downlinkUEAssociatedNRPPaTransport	|

errorIndication					|

handoverNotification			|

handoverSuccess								|

initialUEMessage				|

locationReport					|

locationReportingControl		|

locationReportingFailureIndication	|

multicastGroupPaging			|

nASNonDeliveryIndication		|

overloadStart					|

overloadStop					|

paging							|

pDUSessionResourceNotify		|

privateMessage					|

pWSFailureIndication						|

pWSRestartIndication			|

rANCPRelocationIndication		|

rerouteNASRequest				|

retrieveUEInformation						|

rRCInactiveTransitionReport					|

secondaryRATDataUsageReport		|

traceFailureIndication			|

traceStart						|

uEContextReleaseRequest			|

uEInformationTransfer						|

uERadioCapabilityInfoIndication	|

uETNLABindingRelease			|

uplinkNASTransport				|

uplinkNonUEAssociatedNRPPaTransport	|

uplinkRANConfigurationTransfer	|

uplinkRANEarlyStatusTransfer				|

uplinkRANStatusTransfer			|

uplinkRIMInformationTransfer				|

uplinkUEAssociatedNRPPaTransport,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

aMFConfigurationUpdate NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AMFConfigurationUpdate

SUCCESSFUL OUTCOME		AMFConfigurationUpdateAcknowledge

UNSUCCESSFUL OUTCOME	AMFConfigurationUpdateFailure

PROCEDURE CODE			id-AMFConfigurationUpdate

CRITICALITY				reject

}

aMFCPRelocationIndication NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		AMFCPRelocationIndication

PROCEDURE CODE			id-AMFCPRelocationIndication

CRITICALITY				reject

}

aMFStatusIndication NGAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		AMFStatusIndication

PROCEDURE CODE			id-AMFStatusIndication

CRITICALITY				ignore

}

broadcastSessionModification NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionModificationRequest

SUCCESSFUL OUTCOME		BroadcastSessionModificationResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionModificationFailure

PROCEDURE CODE			id-BroadcastSessionModification

CRITICALITY				reject

}

broadcastSessionRelease NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionReleaseRequest

SUCCESSFUL OUTCOME		BroadcastSessionReleaseResponse

PROCEDURE CODE			id-BroadcastSessionRelease

CRITICALITY				reject

}

broadcastSessionReleaseRequired NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionReleaseRequired

PROCEDURE CODE			id-BroadcastSessionReleaseRequired

CRITICALITY				ignore

}

broadcastSessionSetup NGAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		BroadcastSessionSetupRequest

SUCCESSFUL OUTCOME		BroadcastSessionSetupResponse

UNSUCCESSFUL OUTCOME	BroadcastSessionSetupFailure

PROCEDURE CODE			id-BroadcastSessionSetup

CRITICALITY				reject

}

cellTrafficTrace NGAP-ELEMENTARY-PROCEDURE ::={

INITIATING MESSAGE		CellTrafficTrace

PROCEDURE CODE			id-CellTrafficTrace

CRITICALITY				ignore

}

//Skip unchanged part

-- **************************************************************

--

#### -- MBS SESSION MANAGEMENT ELEMENTARY PROCEDURES

--

-- **************************************************************

-- **************************************************************

--

##### -- Broadcast Session Setup Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST SESSION SETUP REQUEST

--

-- **************************************************************

BroadcastSessionSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionSetupRequestIEs} },

...

}

BroadcastSessionSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-S-NSSAI										CRITICALITY reject	TYPE S-NSSAI											PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea								CRITICALITY reject	TYPE MBS-ServiceArea									PRESENCE mandatory	}|

{ ID id-MBSSessionInformationRequestTransfer	CRITICALITY reject	TYPE MBSSessionInformationRequestTransfer		PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST SESSION SETUP RESPONSE

--

-- **************************************************************

BroadcastSessionSetupResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionSetupResponseIEs} },

...

}

BroadcastSessionSetupResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID									CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBSSessionInformationResponseTransfer	CRITICALITY reject	TYPE MBSSessionInformationResponseTransfer	PRESENCE optional		}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- BROADCAST SESSION SETUP FAILURE

--

-- **************************************************************

BroadcastSessionSetupFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionSetupFailureIEs} },

...

}

BroadcastSessionSetupFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBSSessionInformationFailureTransfer	CRITICALITY reject	TYPE MBSSessionInformationFailureTransfer		PRESENCE optional 	}|

{ ID id-Cause											CRITICALITY ignore	TYPE Cause												PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Broadcast Session Modification Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST SESSION MODIFICATION REQUEST

--

-- **************************************************************

BroadcastSessionModificationRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionModificationRequestIEs} },

...

}

BroadcastSessionModificationRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea								CRITICALITY reject	TYPE MBS-ServiceArea									PRESENCE optional		}|

{ ID id-MBSSessionInformationRequestTransfer	CRITICALITY reject	TYPE MBSSessionInformationRequestTransfer		PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- BROADCAST SESSION MODIFICATION RESPONSE

--

-- **************************************************************

BroadcastSessionModificationResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionModificationResponseIEs} },

...

}

BroadcastSessionModificationResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID									CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBSSessionInformationResponseTransfer	CRITICALITY reject	TYPE MBSSessionInformationResponseTransfer	PRESENCE optional 	}|

{ ID id-CriticalityDiagnostics						CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- BROADCAST SESSION MODIFICATION  FAILURE

--

-- **************************************************************

BroadcastSessionModificationFailure ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionModificationFailureIEs} },

...

}

BroadcastSessionModificationFailureIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID									PRESENCE mandatory	}|

{ ID id-MBSSessionInformationFailureTransfer	CRITICALITY reject	TYPE MBSSessionInformationFailureTransfer		PRESENCE mandatory	}|

{ ID id-Cause											CRITICALITY ignore	TYPE Cause												PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional		},

...

}

-- **************************************************************

--

##### -- Broadcast Session Release Elementary Procedure

--

-- **************************************************************resource

-- **************************************************************

--

##### -- BROADCAST SESSION RELEASE REQUEST

--

-- **************************************************************

BroadcastSessionReleaseRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionReleaseRequestIEs} },

...

}

BroadcastSessionReleaseRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY reject	TYPE MBS-SessionID								PRESENCE mandatory	}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause											PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST SESSION RELEASE REQUIRED Elementary Procedure

--

-- **************************************************************

-- **************************************************************

--

##### -- BROADCAST SESSION RELEASE REQUIRED

--

-- **************************************************************

BroadcastSessionReleaseRequired ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionReleaseRequiredIEs} },

...

}

BroadcastSessionReleaseReiredIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID							CRITICALITY reject	TYPE MBS-SessionID								PRESENCE mandatory	}|

{ ID id-Cause										CRITICALITY ignore	TYPE Cause											PRESENCE mandatory	},

...

}

-- **************************************************************

--

##### -- BROADCAST SESSION RELEASE RESPONSE

--

-- **************************************************************

BroadcastSessionReleaseResponse ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {BroadcastSessionReleaseResponseIEs} },

...

}

BroadcastSessionReleaseResponseIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID								CRITICALITY reject	TYPE MBS-SessionID								PRESENCE mandatory	}|

{ ID id-CriticalityDiagnostics					CRITICALITY ignore	TYPE CriticalityDiagnostics					PRESENCE optional		},

...

}

//Skip unchanged part

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-AMFConfigurationUpdate					ProcedureCode ::= 0

id-AMFStatusIndication						ProcedureCode ::= 1

id-CellTrafficTrace							ProcedureCode ::= 2

id-DeactivateTrace							ProcedureCode ::= 3

id-DownlinkNASTransport						ProcedureCode ::= 4

id-DownlinkNonUEAssociatedNRPPaTransport	ProcedureCode ::= 5

id-DownlinkRANConfigurationTransfer			ProcedureCode ::= 6

id-DownlinkRANStatusTransfer				ProcedureCode ::= 7

id-DownlinkUEAssociatedNRPPaTransport		ProcedureCode ::= 8

id-ErrorIndication							ProcedureCode ::= 9

id-HandoverCancel							ProcedureCode ::= 10

id-HandoverNotification						ProcedureCode ::= 11

id-HandoverPreparation						ProcedureCode ::= 12

id-HandoverResourceAllocation				ProcedureCode ::= 13

id-InitialContextSetup						ProcedureCode ::= 14

id-InitialUEMessage							ProcedureCode ::= 15

id-LocationReportingControl					ProcedureCode ::= 16

id-LocationReportingFailureIndication		ProcedureCode ::= 17

id-LocationReport							ProcedureCode ::= 18

id-NASNonDeliveryIndication					ProcedureCode ::= 19

id-NGReset									ProcedureCode ::= 20

id-NGSetup									ProcedureCode ::= 21

id-OverloadStart							ProcedureCode ::= 22

id-OverloadStop								ProcedureCode ::= 23

id-Paging									ProcedureCode ::= 24

id-PathSwitchRequest						ProcedureCode ::= 25

id-PDUSessionResourceModify					ProcedureCode ::= 26

id-PDUSessionResourceModifyIndication		ProcedureCode ::= 27

id-PDUSessionResourceRelease				ProcedureCode ::= 28

id-PDUSessionResourceSetup					ProcedureCode ::= 29

id-PDUSessionResourceNotify					ProcedureCode ::= 30

id-PrivateMessage							ProcedureCode ::= 31

id-PWSCancel								ProcedureCode ::= 32

id-PWSFailureIndication						ProcedureCode ::= 33

id-PWSRestartIndication						ProcedureCode ::= 34

id-RANConfigurationUpdate					ProcedureCode ::= 35

id-RerouteNASRequest						ProcedureCode ::= 36

id-RRCInactiveTransitionReport				ProcedureCode ::= 37

id-TraceFailureIndication					ProcedureCode ::= 38

id-TraceStart								ProcedureCode ::= 39

id-UEContextModification					ProcedureCode ::= 40

id-UEContextRelease							ProcedureCode ::= 41

id-UEContextReleaseRequest					ProcedureCode ::= 42

id-UERadioCapabilityCheck					ProcedureCode ::= 43

id-UERadioCapabilityInfoIndication			ProcedureCode ::= 44

id-UETNLABindingRelease						ProcedureCode ::= 45

id-UplinkNASTransport						ProcedureCode ::= 46

id-UplinkNonUEAssociatedNRPPaTransport		ProcedureCode ::= 47

id-UplinkRANConfigurationTransfer			ProcedureCode ::= 48

id-UplinkRANStatusTransfer					ProcedureCode ::= 49

id-UplinkUEAssociatedNRPPaTransport			ProcedureCode ::= 50

id-WriteReplaceWarning						ProcedureCode ::= 51

id-SecondaryRATDataUsageReport				ProcedureCode ::= 52

id-UplinkRIMInformationTransfer				ProcedureCode ::= 53

id-DownlinkRIMInformationTransfer			ProcedureCode ::= 54

id-RetrieveUEInformation					ProcedureCode ::= 55

id-UEInformationTransfer					ProcedureCode ::= 56

id-RANCPRelocationIndication				ProcedureCode ::= 57

id-UEContextResume							ProcedureCode ::= 58

id-UEContextSuspend							ProcedureCode ::= 59

id-UERadioCapabilityIDMapping				ProcedureCode ::= 60

id-HandoverSuccess							ProcedureCode ::= 61

id-UplinkRANEarlyStatusTransfer				ProcedureCode ::= 62

id-DownlinkRANEarlyStatusTransfer			ProcedureCode ::= 63

id-AMFCPRelocationIndication				ProcedureCode ::= 64

id-ConnectionEstablishmentIndication		ProcedureCode ::= 65

id-BroadcastSessionModification				ProcedureCode ::= 66

id-BroadcastSessionRelease					ProcedureCode ::= 67

id-BroadcastSessionSetup					ProcedureCode ::= 68

id-DistributionSetup 						ProcedureCode ::= 69

id-DistributionRelease 						ProcedureCode ::= 70

id-MulticastSessionActivation 				ProcedureCode ::= 71

id-MulticastSessionDeactivation 			ProcedureCode ::= 72

id-MulticastSessionUpdate 					ProcedureCode ::= 73

id-MulticastGroupPaging						ProcedureCode ::= 74

id-BroadcastSessionRleaseRequired           ProcedureCode ::= FFS

------------------------------------------------------ End of the change -------------------------------------------------------