3GPP TSG-RAN WG3 Meeting #116-e	 R3-223766

E-Meeting, 9th – 19th May 2022

Agenda Item:	9.3.9

Source:	Samsung - Moderator

Title:	Summary of Offline Discussion – F1UDelay

Document for:	Approval

# Introduction

This is the summary document for the following come back:

CB: # 60_F1UDelay

- Try to find the solution acceptable for the group

(SS - moderator)

Summary of offline disc R3-223766

Deadline for view input: 8:00AM UTC, 18th May.

# For the Chairman’s Notes

Propose the following:

Proposal: Agree compromised solution (adding NR-U SN and feedback delay in DDDS) as the R17 solution for F1-U delay measurement and agree corresponding CR in R3-224057.

# Discussion

Based on the previous and online discussions, there are two candidate solutions to down select:

- Solution 2: Based on current polling function and DDDS reporting, add NR-U sequence number in DDDS. The F1-U delay is (T4-T1)/2, where immediate reporting is needed to guarantee the negligible inner DU feedback delay.

- Solution 3 variant: Use a dedicated polling function, and enhance DDDS reporting by adding feedback delay result. When the received dedicated polling equals to 1, DU feeds back the DDDS with feedback delay time for F1-U delay measurement. The F1-U delay is (T4-T1-feedback time)/2.

For Solution 2, there are three drawbacks based on online discussion:

- NBC: Currently, when DU receives DL User Data with pulling flag, the DU sends DDDS without NR-U. Without any new trigger, it changes the DU behavior to report the NR-U. This is a functional NBC change.

- Heavy overhead: As F1-U delay measurement is from CN requirement, the RAN does not need to measure all the time. Adding NR-U SN in every DDDS reporting is unnecessary.

- Immediate reporting: Solution 2 relies on immediate reporting for DDDS to guarantee the negligible inner DU feedback delay. But immediate reporting in 38.425 is not clearly stated. 38.425 only defines DU “shall send” the DDDS when receiving polling unless overhead is encountered (as following). Thus the immediate reporting can not be guaranteed.

TS 38.425:

The corresponding node shall send the DL DATA DELIVERY STATUS if the Report Polling Flag is set to 1 or when the NR PDCP PDU with the indicated DL report NR PDCP PDU SN has been successfully delivered, unless a situation of overload at the corresponding node is encountered.

Solution 3 variant does the high accurate and efficient measurement without immediate reporting burden as:

- High accuracy: Exact and accurate DU feedback delay is reported to CU if the received dedicated polling flag is set to 1. CU does the measurement based on the method defined in TS 28.552. This solution solves the issue that inaccurate measurement results come from the DU who can not do the immediate feedback.

- High efficiency: DU adds DU feedback delay in DDDS only when the received dedicated polling flag for F1-U delay measurement purpose equals 1. Otherwise, DU reports the current DDDS without DU feedback delay for the normal polling function.

- No reporting burden: There is no time limitation for DU reporting, so it does not lead to the burden for DU to do DDDS reporting.

- Workable at both high traffic and low traffic situation: There is no traffic status limitation for this solution. Even though DU encounters overload, the accurate feedback delay can be send to CU to support F1-U delay measurement.

Q1: Companies are invited to provide their views on above two solutions, which solution is preferred.

| Company | Preferred Solution 2 or Solution 3 variant | Comment |
| --- | --- | --- |
| Samsung | Solution 3 variant | As summarized above, solution 2 has the issue of NBC, heavy overhead and dependency of immediate-reporting. Solution 3 variant provides a way for high accurate and efficient measurement without immediate reporting burden. So we prefer solution 3 variant. |
| Huawei | Solution 3 variant is preferred. | We also accept solution 2 as a compromised solution although it is not the best one. |
| Ericsson | We do not see the need for any solution, but we can accept Solution 2 |  |
| Verizon | Prefer Solution 3 |  |
| Intel | Prefer Solution 3 variant but can accept the Solution 2 spirit (i.e. without new polling flag) | Maybe be Solution 2 + feedback delay in DDDS could resolve the concern of burdens of immediate reporting and allow (T4-T1-feedback time)/2 |
| Nokia | Neutral on solution 3 variant, but can't accept solution 2 at this meeting | Solution 2 has higher impact on legacy implementations and therefore discarded in earlier discussions. It was not part of the proposals submitted to this meeting. We therefore can't accept this proposal at the present meeting. |

Q2: If prefer Solution 3 variant in Q1, companies are invited to provide their views on whether agree the R17 CR in R3-223565.

| Company | Agree or not | Comment |
| --- | --- | --- |
| Samsung | Agree | Solution 3 variant is needed to support F1-U delay measurement. |
| Verizon | Agree |  |
| Intel |  | If Solution 3 variant is not agreeable, the Solution 2 + feedback delay could be considered which at least allows (T4-T1-feedback time)/2, without new polling flag. |
| Nokia | Neutral |  |

Q3: If solution3 variant or solution 2 cannot be agreed, the compromised solution is solution 2 + feedback delay in DDDS. This may be the best we can achieve. What's your view?

| Company | Agree or not | Comment |
| --- | --- | --- |
| Samsung | Fine for this compromise |  |
| Nokia | Don't agree | Similar to solution 2, we need to further analyze impact on legacy implementation, and can't agree this solution at the present meeting. |
| Ericsson | Agree | We can agree to this compromise |

Moderator’s summary:

Majority companies are fine with the compromised solution.

Proposal: Agree compromised solution (adding NR-U SN and feedback delay in DDDS) as the R17 solution for F1-U delay measurement and agree corresponding CR in R3-224057.

# Conclusion

The following is proposed:

Proposal: Agree compromised solution (adding NR-U SN and feedback delay in DDDS) as the R17 solution for F1-U delay measurement and agree corresponding CR in R3-224057.

# References

R3-223564 Discussion on F1-U Delay Measurement for QoS Monitoring (Samsung, Verizon Wireless, Intel Corporation, Huawei, CMCC, KDDI)

R3-223565 Correction of F1-U Delay Measurement for QoS Monitoring (Samsung, Verizon Wireless, Intel Corporation, Huawei, CMCC, KDDI)