3GPP TSG-RAN WG3 Meeting #116-e	R3-223779

# E-meeting, 9 May - 19 May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0228 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for stage 2 on SCG (de)activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Huawei, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, Ericsson, Lenovo, CATT, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-4-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Two corrections are needed for SCG (de)activation feature. - In the RAN #115 e-meeting, it was agreed that MN-CU-CP shall notify the SCG status to MN-CU-UP for MN terminated bearer. Changes are introduced to reflect this agreement. - The overall description in section 8.4.4 is wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce text and figures changes for SCG (de)activation feature. Impact assessment towards the previous version of the specification (same release): This CR has no impact towards the previous version of the specification (same release). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Stage 2 related to SCG status notification to MN-CU-UP is missing. - The overall description in section 8.4.4 is wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.4, 8.4.4.1, 8.4.4.2, 8.4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is the revision of R3-223642. |  |  |  |  |  |  |  |  |

--------------------------------------------------Start of the change---------------------------------------------------

### 8.4.4	SCG Deactivation and Activation

This clause gives the NR SCG deactivation and activation procedures in MR-DC given that the SN consists of a gNB-CU and gNB-DU(s).

8.4.4.1	SN Addition with SCG Activation or Deactivation

Figure 8.4.4.1-1: SCG Activation or Deactivation in SN Addition procedure

1. The MN sends the SN Addition Request message to the SN, indicates the request of SCG activation or deactivation.

2. The SN-CU-CP sends the BEARER CONTEXT SETUP REQUEST message to the SN-CU-UP to setup bearer context and notify the activation or deactivation of the SCG.

3. The SN-CU-UP sends the BEARER CONTEXT SETUP RESPONSE message to the SN-CU-CP.

NOTE 1: Step 2 and 3 are performed in case of SN terminated SCG bearer or split bearer.

4. The SN-CU sends the UE CONTEXT SETUP REQUEST message to the SN-DU to setup UE context and indicate the request of SCG activation or deactivation.

5. The SN-DU sends the UE CONTEXT SETUP RESPONSE message to the SN-CU, indicates the SCG status. In case SCG activation is requested, the SN-DU shall indicate SCG activated in the message.

6. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP as described in Section 8.9.2 and may notify the SCG status if needed.

7. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

NOTE 2: Step 6 and 7 are performed in case of SN terminated SCG bearer or split bearer.

8. The SN sends the SN Addition Request Acknowledge message to the MN, indicates the SCG status. In case SCG activation is requested, the SN shall indicate SCG activated in the message.

9. The MN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the MN-CU-UP to notify the SCG status.

10. The MN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the MN-CU-CP.

NOTE 3: Step 9 and 10 are performed in case of MN terminated SCG bearer or split bearer.

8.4.4.2	MN initiated SN Modification with SCG Activation or Deactivation

Figure 8.4.4.2-1: SCG Activation or Deactivation in MN initiated SN Modification procedure

1. SCG is deactivated or activated.

2. The MN sends the SN Modification Request message to the SN, indicates the request of SCG activation or deactivation.

3. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the SCG activation or deactivation.

4. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

NOTE 1: Step 3 and 4 are performed in case of SN terminated SCG bearer or split bearer.

5. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation or deactivation.

6. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates the SCG status.

7. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the SCG status.

8. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

NOTE 2: Step 7 and 8 may be skipped in case the SN-DU accepted the SCG activation or deactivation request or in case of MN terminated SCG bearer or split bearer.

9. The SN sends the SN Modification Request Acknowledge message towards the MN, indicates the SCG status.

10. The MN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the MN-CU-UP to notify the SCG status.

11. The MN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the MN-CU-CP.

NOTE 3: Step 10 and 11 are performed in case of MN terminated SCG bearer or split bearer.

#### 8.4.4.3	SN initiated SN Modification with SCG Activation or Deactivation

Figure 8.4.4.3-1: SCG Activation or Deactivation in SN initiated SN Modification procedure

1. SCG is deactivated or activated.

2. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the SCG activation or deactivation.

3. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

NOTE 1: Step 2 and 3 are performed in case of SN terminated SCG bearer or split bearer.

4. The SN-CU sends the UE CONTEXT MODIFICATION REQUEST message to the SN-DU to indicate the request of SCG activation or deactivation.

5. The SN-DU sends the UE CONTEXT MODIFICATION RESPONSE message to the SN-CU, indicates the SCG status.

6. The SN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the SN-CU-UP to notify the SCG status.

7. The SN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the SN-CU-CP.

NOTE 2: Step 6 and 7 may be skipped in case the SN-DU accepted the SCG activation or deactivation request or in case of MN terminated SCG bearer or split bearer.

8. The SN sends the SN Modification Required message to the MN, indicates the request of SCG activation or deactivation.

9. The MN-CU-CP sends the BEARER CONTEXT MODIFICATION REQUEST message to the MN-CU-UP to notify the SCG status.

10. The MN-CU-UP sends the BEARER CONTEXT MODIFICATION RESPONSE message to the MN-CU-CP.

NOTE 3:	Step 9 and 10 are performed in case of MN terminated SCG bearer or split bearer. Based on implementation, step 9 and 10 can be performed after step 11.

11. The MN sends the SN Modification Confirm message to the SN.

NOTE 4: Step 8-11 are not performed in case the SN-DU rejected the SCG activation or deactivation request.

-----------------------------------------------------End of the change-------------------------------------------------------