3GPP TSG-RAN WG3 #116-e						    						R3-223801

9th – 19th May 2022

Online

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0821 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.413 for Corrections on NR QoE Capability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  |  | Date: |  | 2022-05-19 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - RAN2 specified that the UE QMC capability is included in the UE Radio Capability in 38.331. QoE-Parameters-r17 ::= SEQUENCE { qoe-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL qoe-MTSI-MeasReport-r17 ENUMERATED {supported} OPTIONAL, qoe-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-Streaming-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ran-VisibleQoE-VR-MeasReport-r17 ENUMERATED {supported} OPTIONAL, ul-MeasurementReportAppLayer-Seg-r17 ENUMERATED {supported} OPTIONAL, ... } UE-NR-Capability-v1700 ::= SEQUENCE { inactiveStatePO-Determination-r17 ENUMERATED {supported} OPTIONAL, highSpeedParameters-v1700 HighSpeedParameters-v1700 OPTIONAL, powSav-Parameters-v1700 PowSav-Parameters-v1700 OPTIONAL, mac-Parameters-v1700 MAC-Parameters-v1700 OPTIONAL, ims-Parameters-v1700 IMS-Parameters-v1700 OPTIONAL, measAndMobParameters-v1700 MeasAndMobParameters-v1700, qoe-Parameters-r17 QoE-Parameters-r17 OPTIONAL, redCapParameters-r17 RedCapParameters-r17 OPTIONAL, - RAN3 include the UE Radio Capability and UE QMC Capability in the UE RADIO CAPABILITY INFO INDICATION. So the UE QMC Capability is duplicated in the message to AMF. We may have more thinking on the usage of UE QMC Capability in AMF. The AMF does not decode the UE Radio Capability and use the explicitly UE QMC Capability to check the qualification for the UE when s-based QMC is initiated. But in the SA5 QMC specification 28.405 v17.1.0 section 4.4.2, they state the RAN is responsible for the capability match.” 4)The eNB checks if the UE capability that matches the criteria for serviceType in the ActivateAreaQMCJob.” - So based on the above discussion, the UE QMC Capability does not need to be explicitly included in the message UE RADIO CAPABILITY INFO INDICATION to the AMF. Solution: in case RAN is responsible for capability match for S-based QMC, remove the UE QMC Capability from UE RADIO CAPABILITY INFO INDICATION and remove the contents of 9.3.1.226 UE QMC Capability |  |  |  |  |  |  |  |  |
| Summary of change: |  | RAN is responsible for capability match for S-based QMC, the change is as below: - remove the UE QMC Capability from UE RADIO CAPABILITY INFO INDICATION - remove the contents of 9.3.1.226 UE QMC Capability and change it to void - remove the corresponding ASN.1 code of above changes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification text is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.13.1, 9.3.1.226, 9.4.4, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev1: revised based on the outcome of the online discussion. |
| --- | --- |

Start changes

#### 9.2.13.1	UE RADIO CAPABILITY INFO INDICATION

This message is sent by the NG-RAN node to provide UE radio capability related information to the AMF.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| UE Radio Capability | M |  | 9.3.1.74 |  | YES | ignore |
| UE Radio Capability for Paging | O |  | 9.3.1.68 |  | YES | ignore |
| UE Radio Capability – E-UTRA Format | O |  | 9.3.1.74a |  | YES | ignore |

------------------------Omitted text not changed-----------------------------------------------

#### 9.3.1.226	Void

------------------------Omitted text not changed-----------------------------------------------

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

------------------------Omitted text not changed-----------------------------------------------

id-UEPresenceInAreaOfInterestList,

id-UERadioCapability,

id-UERadioCapabilityForPaging,

id-UERadioCapabilityID,

id-UERadioCapability-EUTRA-Format,

id-UERetentionInformation,

id-UESecurityCapabilities,

id-UESliceMaximumBitRateList,

id-UE-UP-CIoT-Support,

id-UL-CP-SecurityInformation,

id-UnavailableGUAMIList,

id-UserLocationInformation,

id-W-AGFIdentityInformation,

id-WarningAreaCoordinates,

id-WarningAreaList,

id-WarningMessageContents,

------------------------Omitted text not changed-----------------------------------------------

-- **************************************************************

--

##### -- UE RADIO CAPABILITY INFO INDICATION

--

-- **************************************************************

UERadioCapabilityInfoIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container		{ {UERadioCapabilityInfoIndicationIEs} },

...

}

UERadioCapabilityInfoIndicationIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AMF-UE-NGAP-ID					CRITICALITY reject	TYPE AMF-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-RAN-UE-NGAP-ID					CRITICALITY reject	TYPE RAN-UE-NGAP-ID					PRESENCE mandatory	}|

{ ID id-UERadioCapability				CRITICALITY ignore	TYPE UERadioCapability				PRESENCE mandatory	}|

{ ID id-UERadioCapabilityForPaging		CRITICALITY ignore	TYPE UERadioCapabilityForPaging		PRESENCE optional		}|

{ ID id-UERadioCapability-EUTRA-Format	CRITICALITY ignore	TYPE UERadioCapability				PRESENCE optional		},

...

}

------------------------Omitted text not changed-----------------------------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

------------------------Omitted text not changed-----------------------------------------------

------------------------Omitted text not changed-----------------------------------------------

RejectedNSSAIinTA ::= OCTET STRING (SIZE(32))

SST ::= OCTET STRING (SIZE(1))

SupportedTAList ::= SEQUENCE (SIZE(1..maxnoofTACs)) OF SupportedTAItem

------------------------Omitted text not changed-----------------------------------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

------------------------Omitted text not changed-----------------------------------------------

id-SurvivalTime											ProtocolIE-ID ::= 327

id-QMCConfigInfo											ProtocolIE-ID ::= 328

id-QMCDeactivation										ProtocolIE-ID ::= 329

id-PDUSessionPairID										ProtocolIE-ID ::= 331

id-NR-PagingeDRXInformation								ProtocolIE-ID ::= 332

id-RedCapIndication										ProtocolIE-ID ::= 333

id-TargetNSSAIInformation								ProtocolIE-ID ::= 334

id-UESliceMaximumBitRateList							ProtocolIE-ID ::= 335

------------------------Omitted text not changed-----------------------------------------------

End of changes