3GPP TSG-RAN WG3 Meeting #116-e	R3-223804

E-meeting, 09 May – 19 May 2022

Title:	LS on Flexible Global RAN Node ID

Release:	Rel-17

Work Item:	TEI17

Source:	RAN3

To:	RAN2

Cc:	-

Contact person:	Angelo Centonza

Angelo.Centonza@Ericsson.com

Attachments:	XXXXX

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Overall description

Following discussions at RAN-94e, RAN3 has further investigated the problem of Global gNB ID disambiguation from CGIs reported by the UE.

RAN3 has analysed alternative network-based solutions and it has identified that such solutions are viable for a number of use cases. However, RAN3 has now identified that broadcast based solutions may have some advantages in certain scenarios e.g. if a very high level of gNB-ID allocation flexibility is required by the operator, or when deriving the gNB ID during CN-based HO preparation towards a newly detected cell.

On the basis of such conclusions, and in light of the confirmed feasibility stated in R2-2102449, RAN3 thinks that it would be useful to also include support for broadcast and reporting of gNB-ID length assistance to ensure addressing of all identified use cases.

RAN3 has endorsed the stage 2 CR attached as a baseline (subject to further feedback and progress in RAN2/RAN3), which explains how the elements of network- and broadcast-based solutions may be supported in a unified framework. In this framework, the UE reports an index (broadcast in SIB1) as part of ANR measurements, where the index is mapped to the gNB-ID length (or length set) by operator configuration.

# 2	Actions

To RAN2:

ACTION: 	RAN3 respectfully asks RAN2 to support in their specifications functionality for broadcasting and reporting of gNB-ID length assistance, as described in the attached CR.

# 3	Dates of next RAN3 meetings

The upcoming RAN3 meetings can be found in the RAN3 Meetings calendar