3GPP TSG-RAN WG3 Meeting #116-e	R3-223805

E-meeting, 09 May - 19 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0927 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting network slice AS group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, LG Electronics, CATT, CMCC, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_slice-Core |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SA2 has agreed set of CRs to support Network Slice AS Groups (NSAGs) for slice aware cell reselection/RACH in S2-2203618 and S2-2203619. This CR contains the related protocol changes of the NSAGs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the TAI NSAG Support List IE in F1 setup request and gNB-DU configuration update messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Network Slice AS Group is not supported by RAN. Not aligned with SA2 specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.2, 8.2.4.2, 9.3.1.10, 9.3.1.aaa, 9.4.5, 9.4.7. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.413 CR0785 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-223467 Rev1: R3-223805 Update based on online comments. |  |  |  |  |  |  |  |  |

Change Begins

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GC	5G Core Network

5QI	5G QoS Identifier

AMF	Access and Mobility Management Function

ARP	Antenna Reference Point

ARPI	Additional RRM Policy Index

BH	Backhaul

CAG	Closed Access Group

CN	Core Network

CG	Cell Group

CG-SDT	Configured Grant-Small Data Transmission

CGI	Cell Global Identifier

CHO	Conditional Handover

CP	Control Plane

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAPS	Dual Active Protocol Stack

DL	Downlink

DL-PRS	Downlink Positioning Reference Signal

EN-DC	E-UTRA-NR Dual Connectivity

EPC	Evolved Packet Core

FSA ID	MBS Frequency Selection Area (FSA) ID

IAB	Integrated Access and Backhaul

IMEISV	International Mobile station Equipment Identity and Software Version number

LMF	Location Management Function

MBS	Multicast/Broadcast Service

NID	Network Identifier

NPN	Non-Public Network

NSAG	Network Slice AS Group

NSSAI	Network Slice Selection Assistance Information

PDC	Propagation Delay Compensation

PEIPS	Paging Early Indication with Paging Subgrouping

posSIB	Positioning SIB

PNI-NPN	Public Network Integrated NPN

PTP	Point to Point

PTM	Point to Multipoint

QoE	Quality of Experience

RANAC	RAN Area Code

RedCap	Reduced Capability

RIM	Remote Interference Management

RIM-RS	RIM Reference Signal

RRC	Radio Resource Control

RSRP	Reference Signal Received Power

SDT	Small Data Transmission

SNPN	Stand-alone Non-Public Network

S-NSSAI	Single Network Slice Selection Assistance Information

SUL	Supplementary Uplink

TAC	Tracking Area Code

TAI	Tracking Area Identity

TEG	Timing Error Group

TRP	Transmission-Reception Point

UL-AoA	Uplink Angle of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SRS	Uplink Sounding Reference Signal

Z-AoA	Zenith Angles of Arrival

<Unchanged Text Omitted>

### 8.2.3	F1 Setup

#### 8.2.3.1	General

The purpose of the F1 Setup procedure is to exchange application level data needed for the gNB-DU and the gNB-CU to correctly interoperate on the F1 interface. This procedure shall be the first F1AP procedure triggered for the F1-C interface instance after a TNL association has become operational.

NOTE:	If F1-C signalling transport is shared among multiple F1-C interface instances, one F1 Setup procedure is issued per F1-C interface instance to be setup, i.e. several F1 Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE:	Exchange of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received. This procedure also re-initialises the F1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do.

#### 8.2.3.2	Successful Operation

Figure 8.2.3.2-1: F1 Setup procedure: Successful Operation

<Unchanged Text Omitted>

If the NR Cell PRACH Configuration IE is included in the Served Cell Information IE contained in the F1 SETUP REQUEST message, the gNB-CU may store the information, and forward it to other RAN nodes for RACH optimisation.

If the RedCap Broadcast Information IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the F1 SETUP REQUEST message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

<Unchanged Text Omitted>

### 8.2.4	gNB-DU Configuration Update

#### 8.2.4.1	General

The purpose of the gNB-DU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and the gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

NOTE:	Update of application level configuration data also applies between the gNB-DU and the gNB-CU in case the DU does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: gNB-DU Configuration Update procedure: Successful Operation

<Unchanged Text Omitted>

If the Cells for SON IE is present in the GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-DU may store or update this information and behaves as follows:

-	For each served cell indicated by the NR CGI IE included within the Cells for SON Item IE, the gNB-DU may adjust the PRACH configuration of this served cell.

-	If the Neighbour NR Cells for SON List IE is present in the Cells for SON Item IE, the gNB-DU may take the PRACH configuration of neighbour cells included in the Neighbour NR Cells for SON List IE into consideration when adjusting the PRACH configuration of the served cell.

If the RedCap Broadcast Information IE is contained in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU may use this information to determine a suitable target in case of subsequent outgoing mobility involving RedCap UEs.

If the TAI NSAG Support List IE is included in the Served Cell Information IE in the GNB-DU CONFIGURATION UPDATE message, the gNB-CU shall, if supported, use this information as specified in TS 23.501 [21].

<Unchanged Text Omitted>

#### 9.3.1.10	Served Cell Information

This IE contains cell configuration information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR CGI | M |  | 9.3.1.12 |  | - |  |
| NR PCI | M |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| 5GS TAC | O |  | 9.3.1.29 | 5GS Tracking Area Code | - |  |
| Configured EPS TAC | O |  | 9.3.1.29a |  | - |  |
| Served PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB 1 associated to the NR Cell Identity in the NR CGI IE | - |  |
| >PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | YES | ignore |
| >NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.aaa | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| CHOICE NR-Mode-Info | M |  |  |  | - |  |
| >FDD |  |  |  |  | - |  |
| >>FDD Info |  | 1 |  |  | - |  |
| >>>UL FreqInfo | M |  | NR Frequency Info 9.3.1.17 | This IE is ignored if the Cell Direction IE is included and set to “dl-only”. | - |  |
| >>>DL FreqInfo | M |  | NR Frequency Info 9.3.1.17 | This IE is ignored if the Cell Direction IE is included and set to “ul-only”. | - |  |
| >>>UL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | This IE is ignored if the Cell Direction IE is included and set to “dl-only”. | - |  |
| >>>DL Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 | This IE is ignored if the Cell Direction IE is included and set to “ul-only”. | - |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the UL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>DL Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the DL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >TDD |  |  |  |  | - |  |
| >>TDD Info |  | 1 |  |  | - |  |
| >>>NR FreqInfo | M |  | NR Frequency Info 9.3.1.17 |  | - |  |
| >>>Transmission Bandwidth | M |  | Transmission Bandwidth 9.3.1.15 |  | - |  |
| >>>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | YES | ignore |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | The tdd-UL-DL-ConfigurationCommon as defined in TS 38.331 [8] | YES | ignore |
| >>>Carrier List | O |  | NR Carrier List 9.3.1.137 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >NR-U |  |  |  |  | YES | ignore |
| >>NR-U Channel Info List |  | 1..< maxnoofNR-UChannelIDs> |  |  | - |  |
| >>>NR-U Channel Info Item |  |  |  |  | - |  |
| >>>>NR-U Channel ID | M |  | INTEGER (1.. maxnoofNR-UChannelIDs, …) | Index to uniquely identify the part of the NR-U Channel Bandwidth consisting of a contiguous set of resource blocks (RBs) on which a channel access procedure is performed in shared spectrum. Value 1 represents the first part of the NR-U Channel Bandwidth on which a channel access procedure is performed. Value 2 represents the second part of the NR-U Channel Bandwidth on which a channel access procedure is performed, and so on. | - |  |
| >>>>NR-U ARFCN | M |  | INTEGER (0.. maxNRARFCN) | It represents the centre frequency of the NR-U Channel Bandwidth. Only values specified in TS 38.101-1 [26] for NR shared spectrum are valid. | - |  |
| >>>>NR-U Channel Bandwidth | M |  | ENUMERATED (10MHz, 20MHz, 40MHz, 60 MHz, 80 MHz,. …) |  | - |  |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message defined in TS 38.331 [8]. | - |  |
| RANAC | O |  | RAN Area Code 9.3.1.57 |  | YES | ignore |
| Extended Served PLMNs List |  | 0..1 |  | This is included if more than 6 Served PLMNs is to be signalled. | YES | ignore |
| >Extended Served PLMNs Item |  | 1 ..<maxnoofExtendedBPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.1.14 |  | - |  |
| >>TAI Slice Support List | O |  | Slice Support List 9.3.1.37 | Supported S-NSSAIs per PLMN or per SNPN. | - |  |
| >>NPN Support Information | O |  | 9.3.1.156 | Supported NPNs per PLMN. | YES | reject |
| >>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.165 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >TAI NSAG Support List | O |  | 9.3.1.aaa | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| Cell Direction | O |  | 9.3.1.78 |  | YES | ignore |
| Broadcast PLMN Identity Info List |  | 0..<maxnoofBPLMNsNR> |  | This IE corresponds to the PLMN-IdentityInfoList IE and the NPN-IdentityInfoList IE (if available) in SIB1 as specified in TS 38.331 [8]. All PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE and NPN identities and associated information contained in the NPN-IdentityInfoList IE (if available) are included and provided in the same order as broadcast in SIB1. NOTE: In case of NPN-only cell, the PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are not included. | YES | ignore |
| Cell Type | O |  | 9.3.1.87 |  | YES | ignore |
| >PLMN Identity List | M |  | Available PLMN List 9.3.1.65 | Broadcast PLMN IDs in SIB1 associated to the NR Cell Identity IE | - |  |
| >Extended PLMN Identity List | O |  | Extended Available PLMN List 9.3.1.76 |  | - |  |
| >5GS-TAC | O |  | OCTET STRING (3) |  | - |  |
| >NR Cell Identity | M |  | BIT STRING (36) |  | - |  |
| >RANAC | O |  | RAN Area Code 9.3.1.57 |  | - |  |
| >Configured TAC Indication | O |  | 9.3.1.87a | NOTE: This IE is associated with the 5GS TAC in the Broadcast PLMN Identity Info List IE | YES | ignore |
| >NPN Broadcast Information | O |  | 9.3.1.157 | If this IE is included the content of the PLMN Identity List IE and Extended PLMN Identity List IE if present in the Broadcast PLMN Identity Info List IE is ignored. | YES | reject |
| Configured TAC Indication | O |  | 9.3.1.87a | NOTE: This IE is associated with the 5GS TAC on top-level of the Served Cell Information IE | YES | ignore |
| Aggressor gNB Set ID | O |  | 9.3.1.93 | This IE indicates the associated aggressor gNB Set ID of the cell | YES | ignore |
| Victim gNB Set ID | O |  | 9.3.1.93 | This IE indicates the associated Victim gNB Set ID of the cell | YES | ignore |
| IAB Info IAB-DU | O |  | 9.3.1.106 |  | YES | ignore |
| SSB Positions In Burst | O |  | 9.3.1.138 |  | YES | ignore |
| NR PRACH Configuration | O |  | 9.3.1.139 |  | YES | ignore |
| SFN Offset | O |  | 9.3.1.208 |  | YES | ignore |
| NPN Broadcast Information | O |  | 9.3.1.157 |  | YES | reject |
| Supported MBS FSA ID List |  | 0..<maxnoofMBSFSAs> |  | Shall contain all MBS Frequency Selection Area Identities associated with the NR CGI. | YES | ignore |
| >MBS Frequency Selection Area Identity | M |  | OCTET STRING(3) |  | – |  |
| RedCap Broadcast Information | O |  | BIT STRING (SIZE(8)) | The presence of this IE indicates that the intraFreqReselectionRedCap IE is broadcast in SIB1 of the corresponding cell, see TS 38.331 [8]. Each position in the bitmap indicates which RedCap UEs are allowed access, according to the setting of RedCap barring indicators in SIB1, see TS 38.331 [8]. First bit = 1Rx, second bit = 2Rx, other bits reserved for future use. Value '1' indicates 'access allowed'. Value '0' indicates 'access not allowed”. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMN Ids. Value is 6. |
| maxnoofExtendedBPLMNs | Maximum no. of Extended Broadcast PLMN Ids. Value is 6. |
| maxnoofBPLMNsNR | Maximum no. of PLMN Ids.broadcast in an NR cell. Value is 12. |
| maxnoofNR-UChannelIDs | Maximum no. NR-U Channel IDs in a cell. Value is 4. |
| maxnoofMBSFSAs | Maximum no. of MBS FSAs by a cell. Value is 256. |

<Unchanged Text Omitted>

#### 9.3.1.aaa	TAI NSAG Support List

This IE indicates the list of NSAGs configured at the gNB-DU and their associated S-NSSAIs as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NSAG Support Item |  | 1..<maxnoofNSAGs> |  |  |
| >NSAG ID | M |  | INTEGER (0.. 255, …) |  |
| >NSAG Slice Support List | M |  | Extended Slice Support List 9.3.1.165 | Indicates the list of slices which belong to the NSAG. |

| Range bound | Explanation |
| --- | --- |
| maxnoofNSAGs | Maximum no. of signalled NSAGs. Value is 256. |

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Unchanged Text Omitted>

id-DRBMappingInfo,

id-LastUsedCellIndication,

id-SIB17-message,

id-MUSIM-GapConfig,

id-TAINSAGSupportList,

maxNRARFCN,

maxnoofErrors,

<Unchanged Text Omitted>

axnoofQoEInformation,

maxnoofUuRLCChannels,

maxnoofPC5RLCChannels,

maxnoofSMBRValues,

maxnoofNSAGs

<Unchanged Text Omitted>

BPLMN-ID-Info-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNsNR)) OF BPLMN-ID-Info-Item

BPLMN-ID-Info-Item ::= SEQUENCE {

pLMN-Identity-List			AvailablePLMNList,

extended-PLMN-Identity-List	ExtendedAvailablePLMN-List	OPTIONAL,

fiveGS-TAC					FiveGS-TAC					OPTIONAL,

nr-cell-ID					NRCellIdentity,

ranac						RANAC						OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { BPLMN-ID-Info-ItemExtIEs} } OPTIONAL,

...

}

BPLMN-ID-Info-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-ConfiguredTACIndication		CRITICALITY ignore	EXTENSION ConfiguredTACIndication		PRESENCE optional }|

{	ID id-NPNBroadcastInformation		CRITICALITY reject EXTENSION NPNBroadcastInformation		PRESENCE optional},

...

}

ServedPLMNs-List ::= SEQUENCE (SIZE(1..maxnoofBPLMNs)) OF ServedPLMNs-Item

ServedPLMNs-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

iE-Extensions				ProtocolExtensionContainer { { ServedPLMNs-ItemExtIEs} } OPTIONAL,

...

}

ServedPLMNs-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-TAISliceSupportList	CRITICALITY ignore	EXTENSION SliceSupportList		PRESENCE optional	}|

{ ID id-NPNSupportInfo	CRITICALITY reject	EXTENSION NPNSupportInfo		PRESENCE optional	}|

{ ID id-ExtendedTAISliceSupportList	CRITICALITY reject	EXTENSION ExtendedSliceSupportList		PRESENCE optional	}|

{ ID id-TAINSAGSupportList			CRITICALITY ignore	EXTENSION NSAGSupportList		PRESENCE optional},

...

}

<Unchanged Text Omitted>

ExtendedAdditionalPathListRequest ::= ENUMERATED {true, ...}

ExtendedAvailablePLMN-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

ExtendedServedPLMNs-List ::= SEQUENCE (SIZE(1.. maxnoofExtendedBPLMNs)) OF ExtendedServedPLMNs-Item

ExtendedServedPLMNs-Item ::= SEQUENCE {

pLMN-Identity				PLMN-Identity,

tAISliceSupportList 		SliceSupportList	OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ExtendedServedPLMNs-ItemExtIEs} } OPTIONAL,

...

}

ExtendedServedPLMNs-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-NPNSupportInfo			CRITICALITY reject	EXTENSION NPNSupportInfo				PRESENCE optional	}|

{ ID id-ExtendedTAISliceSupportList	CRITICALITY reject	EXTENSION ExtendedSliceSupportList		PRESENCE optional	}|

{ ID id-TAINSAGSupportList			CRITICALITY ignore	EXTENSION NSAGSupportList		PRESENCE optional},

...

}

ExtendedSliceSupportList ::= SEQUENCE (SIZE(1.. maxnoofExtSliceItems)) OF SliceSupportItem

<Unchanged Text Omitted>

FiveGS-TAC ::= OCTET STRING (SIZE(3))

Configured-EPS-TAC ::= OCTET STRING (SIZE(2))

TargetCellList ::= SEQUENCE (SIZE(1..maxnoofCHOcells)) OF TargetCellList-Item

TargetCellList-Item ::= SEQUENCE {

target-cell								NRCGI,

iE-Extensions							ProtocolExtensionContainer { { TargetCellList-Item-ExtIEs} } OPTIONAL

}

TargetCellList-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NSAGSupportList ::= SEQUENCE (SIZE(1.. maxnoofNSAGs)) OF NSAGSupportItem

NSAGSupportItem ::= SEQUENCE {

nSAG-ID					NSAG-ID,

nSAGSliceSupport	ExtendedSliceSupportList,

iE-Extensions		ProtocolExtensionContainer { {NSAGSupportItem-ExtIEs} }	OPTIONAL,

...

}

NSAGSupportItem-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

NSAG-ID ::= INTEGER (0..255, ...)

TDD-Info ::= SEQUENCE {

nRFreqInfo							NRFreqInfo,

transmission-Bandwidth			Transmission-Bandwidth,

iE-Extensions				ProtocolExtensionContainer { {TDD-Info-ExtIEs} } OPTIONAL,

...

}

TDD-Info-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ID	id-IntendedTDD-DL-ULConfig	CRITICALITY ignore	EXTENSION	IntendedTDD-DL-ULConfig	PRESENCE optional}|

{ID id-TDD-UL-DLConfigCommonNR	CRITICALITY ignore	EXTENSION TDD-UL-DLConfigCommonNR	PRESENCE optional }|

{ID id-CarrierList				CRITICALITY ignore	EXTENSION NRCarrierList				PRESENCE optional },

...

}

TDD-InfoRel16 ::= SEQUENCE {

tDD-FreqInfo									FreqInfoRel16													OPTIONAL,

sUL-FreqInfo									FreqInfoRel16													OPTIONAL,

tDD-UL-DLConfigCommonNR							TDD-UL-DLConfigCommonNR											OPTIONAL,

iE-Extensions									ProtocolExtensionContainer { {TDD-InfoRel16-ExtIEs} }		OPTIONAL,

...

}

TDD-InfoRel16-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

TDD-UL-DLConfigCommonNR ::= OCTET STRING

<Unchanged Text Omitted>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<Unchanged Text Omitted>

maxnoofQoEInformation					INTEGER ::= 16

maxnoofUuRLCChannels					INTEGER ::= 32

maxnoofPC5RLCChannels					INTEGER ::= 64

maxnoofSMBRValues						INTEGER ::= 8

maxnoofNSAGs                        INTEGER ::= 256

<Unchanged Text Omitted>

id-PEIPSAssistanceInfo								ProtocolIE-ID ::= 622

id-UEPagingCapability								ProtocolIE-ID ::= 623

id-LastUsedCellIndication							ProtocolIE-ID ::= 624

id-SIB17-message										ProtocolIE-ID ::= 625

id-GNBDUUESliceMaximumBitRateList					ProtocolIE-ID ::= 626

id-TAINSAGSupportList								ProtocolIE-ID ::= aaa

<Unchanged Text Omitted>

Change Ends