3GPP TSG-RAN WG3 Meeting #116-e	R3-223827

# E-meeting, 09 May – 19 May 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.410 | CR | 0040 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NR MBS for 38410 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, China Unicom, Nokia, Nokia Shanghai Bell, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR is to remove the Editor’s Notes in TS 38.413, and include the new procedure for NG-RAN triggered release procedure for BC. And add the missed Multicast Session Update procedure in the list of NR MBS Session Management Procedures. About the Editor’s Notes: Editor’s note: The definition of MBS session can be refined. it is not related to any issue, should be simply removed. Editor’s Note 4: FFS on NG-RAN triggered release procedures for both, BC and MC session resources.  it is needed to add NG-RAN triggered release procedure for BC. Editor’s note: Changes are expected for MBS session.  the sentence above this EN has already been updated by adding MBS Session in the sentence, therefore this one should be simply removed as well < TBD > Figure 7.2-2: NG-U protocol structure for MBS Session Editor’s note: If NG-U protocol structure is the same for both PDU Session and MBS Session, the Figure 7.2-2 shall be removed.  the user plane structure is the same for PDU session and MBS session, but the existing Figure only mentioned PDU Session, it is better to update the existing Figure to be applicable for both PDU Session and MBS Session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - And add the missed Multicast Session Update procedure in the list of NR MBS Session Management Procedures - Add NG-RAN triggered release procedure for BC. - Remove all the Editor’s notes. - Update Figure 7.2-1 to be applicable for both PDU Session and MBS Session. - Remove Figure 7.2-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editor’s Notes remaining in specification, NG-RAN triggerred release procedure for BC and multicast session update procedure are missing.. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4, 6.23, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev-1: add co-source companies. |  |  |  |  |  |  |  |  |

-----------Start of the First Change -----------

## 4.4	NG interface capabilities

The NG interface supports:

-	procedures to establish, maintain and release NG-RAN part of PDU sessions;

-	procedures to perform intra-RAT handover and inter-RAT handover;

-	the separation of each UE on the protocol level for user specific signalling management;

-	the transfer of NAS signalling messages between UE and AMF;

-	mechanisms for resource reservation for packet data streams;

-	procedures to establish, maintain and release NG-RAN part of MBS sessions.

-----------Start of the Next Change -----------

## 6.23	NR MBS Session Management Procedures

The following list of MBS Session management procedures are used to establish, release, or modify NG-RAN resources for a NR MBS session:

-	Broadcast Session Resource Setup;

-	Broadcast Session Modification;

-	Broadcast Session Resource Release;

-	Broadcast Session Resource Release Required;

-	Multicast Session Activation;

-	Multicast Session Deactivation;

-	Multicast Session Update;

-	Distribution Setup;

-	Distribution Release;

-----------Start of the Next Change -----------

## 7.2	NG User Plane

The NG user plane (NG-U) interface is defined between a NG-RAN node and a UPF. The NG-U interface provides non guaranteed delivery of PDU Session/MBS session user plane PDUs between the NG-RAN node and the UPF.

The protocol stack for NG-U is shown in Figure 7.2-1.

Figure 7.2-1: NG-U protocol structure for PDU/MBS Session

-----------End of the Changes -----------