3GPP TSG-RAN WG3 #116-e																		  R3-223894

9th – 19th May 2022

Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0776 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on CPAC to 38.423 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Lenovo, CATT, Ericsson, Google |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the SoD R3-223789, RAN3 has the following agreement in the Chair’s note. - For SN initiated Inter-SN CPC, RRC Container for CG-Config message can be sent in per target SN manner for better association between the suggested PSCells in CG-Config and target SN. - In case of SN initiated inter SN CPC, if the step 2 is skipped, i.e., MN does not inform S-SN about the list of PSCells prepared by T-SN using SN MODIFICAITION REQUEST message, MN will include the list of prepared PSCells to S-SN in the SN CHANGE CONFIRM message - The “Maximum Number of PSCells To Prepare” IE in the SN CHANGE REQUIRED message should be present as Mandatory. - Add Target SN-ID of accepted PSCells in SN MODIFICATION REQUEST message for SN initiated CPAC. Editorial for better consistency with rest of the spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture the above agreement and editorial correction |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some errors are existing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.3, 8.3.5, 8.3.19, 9.1.2.5, 9.1.2.11, 9.1.2.12, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0: R3-223113 |  |  |  |  |  |  |  |  |

8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

<Skip unchanged part>

If the Conditional PSCell Addition Information Modification Request IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider that the request concerns an update of the previous CPAC preparation, as described in TS 37.340 [8]. Accordingly, the S-NG-RAN shall, if supported, include the Conditional PSCell Addition Information Modification Acknowledge IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

If the CG-CandidateList is included in the S-NG-RAN node to M-NG-RAN node Container IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message, the M-NG-RAN node shall, if supported, use it for the purpose of CPAC.

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Addition Information Modification Request IE included in the S-NODE MODIFICATION REQUEST message, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC procedure.

<Skip unchanged part>

8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

8.3.5.1	General

This procedure is used by the S-NG-RAN node to trigger the change of the S-NG-RAN node.

The procedure uses UE-associated signalling.

<Skip unchanged part>

The M-NG-RAN node may also provide configuration information in the M-NG-RAN node to S-NG-RAN node Container IE.

If the Conditional PSCell Change Information Required IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, consider that the requirement concerns CPAC, as described in TS 37.340 [8]. The S-NG-RAN node to M-NG-RAN node Container IE within the Conditional PSCell Change Information Required IE contains at least the suggested PSCell list for each candidate target S-NG-RAN node. Accordingly, the M-NG-RAN node may include the Conditional PSCell Change Information Confirm IE in the S-NODE CHANGE CONFIRM message.

If the Estimated Arrival Probability IE is contained in the Conditional PSCell Change Information Required IE included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, forward this information to the candidate target S-NG-RAN node, then the candidate target S-NG-RAN node may use the information to allocate necessary resources for the incoming CPAC procedure.

Interaction with M-NG-RAN node initiated S-NG-RAN node Release:

If the M-NG-RAN node receives the S-NODE CHANGE REQUIRED message indicating releasing target S-NG-RAN node(s) and cancelling all prepared PSCells in the target S-NG-RAN node(s), the M-NG-RAN shall, if supported, trigger the M-NG-RAN node initiated S-NG-RAN node release procedure to the target S-NG-RAN node(s) and cancel all the prepared PSCells at the target S-NG-RAN node(s).

<Skip unchanged part>

8.3.19	Conditional PSCell Change Cancel

8.3.19.1	General

This procedure is used by the M-NG-RAN node to inform the source S-NG-RAN node that all the prepared PSCells are cancelled in the target S-NG-RAN node during a Conditional PSCell Change.

The procedure uses UE-associated signalling.

8.3.19.2	Successful Operation

Figure 8.3.19.2-1: Conditional PSCell Change Cancel, successful operation

The M-NG-RAN node initiates the procedure by sending the CONDITIONAL PSCELL CHANGE CANCEL message to the S-NG-RAN node including the Target S-NG-RAN node ID IE.

<Skip unrelated part>

9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |  | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node |  | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  |  | YES | ignore |
| <Skip unrelated part> |  |  |  |  |  |  |  |
| CHO Information SN Modification | O |  |  |  |  | YES | reject |
| >Conditional Reconfiguration | M |  | ENUMERATED (intra-MN-CHO, ...) |  |  | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  |  | – |  |
| SCG Activation Request | O |  | 9.2.3.154 |  |  | YES | ignore |
| Conditional PSCell Addition Information Modification Request | O |  |  |  |  | YES | ignore |
| >Maximum Number of PSCells To Prepare | O |  | INTEGER (1..8, ...) |  | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >Estimated Arrival Probability | O |  | INTEGER (1..100) |  | Indicates the arrival probability for the UE towards the candidate target SN. |  |  |
| Conditional PSCell Change Information Update | O |  |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  |  | – |  |
| >>> Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  |  | – |  |
| >>>Candidate PSCell ID List |  | 1 |  |  |  | – |  |
| >>>>Candidate PSCell ID Item |  | 1 .. <maxnoofPSCellCandidate> |  |  |  | – |  |
| >>>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  |  | – |  |
| S-NG-RAN node UE Slice Maximum Bit Rate | O |  | UE Slice Maximum Bit Rate List 9.2.3.167 |  | This IE indicates the S-NG-RAN node portion of the UE Slice Aggregate Maximum Bit Rate as specified in TS 23.501 [7] | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

<Skip unrelated part>

9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 | This IE shall be ignored if the Conditional PSCell Change Information Required IE is present. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| PDU Session SN Change Required List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Required Item |  | 1 .. <maxnoofPDUsessions> |  | NOTE: If the PDU Session Resource Change Required Info – SN terminated IE is not present in a PDU Session SN Change Required Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Required Info – SN terminated | O |  | 9.2.1.18 |  | – |  |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. This IE shall be ignored if the Conditional PSCell Change Information Required IE is present. | YES | reject |
| SCG UE History Information | O |  | 9.2.3.151 |  | YES | ignore |
| SN Mobility Information | O |  | BIT STRING (SIZE (32)) | Information related to PSCell change; S-NG-RAN node provides it in order to enable later analysis of the conditions that led to wrong PSCell change. | YES | ignore |
| Source PSCell ID | O |  | NG-RAN Cell Identity 9.2.2.9 |  | YES | ignore |
| Conditional PSCell Change Information Required | O |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>CPAC Indicator | M |  | ENUMERATED (CPAC-initiation, CPC-modification, CPC-cancellation, ...) |  | – |  |
| >>>Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >>>Estimated Arrival Probability | O |  | INTEGER (1..100) | Indicates the arrival probability for the UE towards the candidate target SN. | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

9.1.2.12	S-NODE CHANGE CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node that the preparation of the S-NG-RAN node initiated S-NG-RAN node change was successful.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| PDU Session SN Change Confirm List |  | 0..1 |  |  | YES | ignore |
| >PDU Session SN Change Confirm Item |  | 1 .. <maxnoof PDUsessions> |  | NOTE: If the PDU Session Resource Change Confirm Info – SN terminated IE is not present in a PDU Session SN Change Confirm Item IE, abnormal conditions as specified in clause 8.3.5.4 apply. | – |  |
| >>PDU Session ID | M |  | 9.2.3.18 |  | – |  |
| >>PDU Session Resource Change Confirm Info – SN terminated | O |  | 9.2.1.19 |  | – |  |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Conditional PSCell Change Information Confirm | O |  |  |  | YES | reject |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>Candidate PSCell ID List |  | 1 |  |  | – |  |
| >>>>Candidate PSCell ID Item |  | 1 .. <maxnoofPSCellCandidate> |  |  | – |  |
| >>>>>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10]. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

<Skip unrelated part>

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Skip unrelated part>

CPACcandidatePSCells-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPACcandidatePSCells-item

CPACcandidatePSCells-item ::= SEQUENCE {

pscell-id						NR-CGI,

iE-Extensions					ProtocolExtensionContainer { {CPACcandidatePSCells-item-ExtIEs}}	OPTIONAL,

...

}

CPACcandidatePSCells-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPACindicator ::= ENUMERATED {cpac-initiation, cpc-modification, cpc-cancellation, ...}

CPAInformationRequest ::= SEQUENCE {

max-no-of-pscells					INTEGER (1..maxnoofPSCellCandidates, ...),

cpac-EstimatedArrivalProbability	CHO-Probability 	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CPAInformationRequest-ExtIEs} } OPTIONAL,

...

}

CPAInformationRequest-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPAInformationAck ::= SEQUENCE {

candidate-pscells					CPACcandidatePSCells-list,

iE-Extensions		ProtocolExtensionContainer { { CPAInformationAck-ExtIEs} } OPTIONAL,

...

}

CPAInformationAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPCInformationRequired::= SEQUENCE {

cpc-target-sn-required-list		CPC-target-SN-required-list,

iE-Extensions					ProtocolExtensionContainer { {CPCInformationRequired-ExtIEs} } OPTIONAL,

...

}

CPCInformationRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPC-target-SN-required-list ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF CPC-target-SN-required-list-Item

CPC-target-SN-required-list-Item	::= SEQUENCE {

target-S-NG-RANnodeID			GlobalNG-RANNode-ID,

cpac-indicator					CPACindicator,

max-no-of-pscells				INTEGER (1..maxnoofPSCellCandidates, ...)	,

cpac-EstimatedArrivalProbability	CHO-Probability 	OPTIONAL,

SN-to-MN-Container				OCTET STRING,

iE-Extensions					ProtocolExtensionContainer { { CPC-target-SN-required-list-Item-ExtIEs} } OPTIONAL,

...

}

CPC-target-SN-required-list-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPCInformationConfirm ::= SEQUENCE {

cpc-target-sn-confirm-list CPC-target-SN-confirm-list,

iE-Extensions		ProtocolExtensionContainer { { CPCInformationConfirm-ExtIEs} } OPTIONAL,

...

}

CPCInformationConfirm-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPC-target-SN-confirm-list ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF CPC-target-SN-confirm-list-Item

CPC-target-SN-confirm-list-Item ::= SEQUENCE {

target-S-NG-RANnodeID			GlobalNG-RANNode-ID,

candidate-pscells				CPACcandidatePSCells-list,

iE-Extensions					ProtocolExtensionContainer { { CPC-target-SN-confirm-list-Item-ExtIEs} } OPTIONAL,

...

}

CPC-target-SN-confirm-list-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPAInformationModReq ::=  SEQUENCE {

max-no-of-pscells					INTEGER (1..8, ...)	OPTIONAL,

cpac-EstimatedArrivalProbability	CHO-Probability 	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { CPAInformationModReq-ExtIEs} } OPTIONAL,

...

}

CPAInformationModReq-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPAInformationModReqAck ::=  SEQUENCE {

candidate-pscells					CPACcandidatePSCells-list,

iE-Extensions						ProtocolExtensionContainer { { CPAInformationModReqAck-ExtIEs} } OPTIONAL,

...

}

CPAInformationModReqAck-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPC-DataForwarding-Indicator ::= ENUMERATED {triggered, early-data-transmission-stop, ...}

CPAInformationModRequired ::= SEQUENCE {

candidate-pscells	CPACcandidatePSCells-list,

iE-Extensions		ProtocolExtensionContainer { { CPAInformationModRequired-ExtIEs} } OPTIONAL,

...

}

CPAInformationModRequired-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPCInformationUpdate::= SEQUENCE {

cpc-target-sn-list					CPC-target-SN-mod-list,

iE-Extensions		ProtocolExtensionContainer { { CPCInformationUpdate-ExtIEs} } OPTIONAL,

...

}

CPCInformationUpdate-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPC-target-SN-mod-list ::= SEQUENCE (SIZE(1..maxnoofTargetSNs)) OF CPC-target-SN-mod-item

CPC-target-SN-mod-item ::= SEQUENCE {

target-S-NG-RANnodeID				GlobalNG-RANNode-ID,

candidate-pscells					CPCInformationUpdatePSCells-list,

iE-Extensions		ProtocolExtensionContainer { {CPC-target-SN-mod-item-ExtIEs} } OPTIONAL,

...

}

CPC-target-SN-mod-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

CPCInformationUpdatePSCells-list ::= SEQUENCE (SIZE(1..maxnoofPSCellCandidates)) OF CPCInformationUpdatePSCells-item

CPCInformationUpdatePSCells-item ::= SEQUENCE {

pscell-id						NR-CGI,

iE-Extensions					ProtocolExtensionContainer { {CPCInformationUpdatePSCells-item-ExtIEs}}	OPTIONAL,

...

}

CPCInformationUpdatePSCells-item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

----------End of the Change--------------