3GPP TSG-RAN WG3 #116-e	R3-224029

Online, 9th – 19th May 2022

Agenda Item:	9.1.2.1

Source:	Lenovo (moderator)

Title:	Summary for CB # IAB_02_CR 38.401

Document for:	Approval

Introduction

This paper provides the summary for following offline discussion: And in this paper, only some essential issues which may need to be discussed are listed, and the remaining stage-2 wording and editorial corrections are summarized in another parallel draft TP, and the output of the draft TP will be merged into the final single CR.

CB: # IAB_02_CR38.401 - Agree on needed corrections - Converge on Single CR (Lenovo - moderator) Summary of offline disc R3-223706 rev in R3-223813 rev in R3-224029

Phase I：Please give your feedback before Wednesday, 11th May 2022, 23:59 UTC. This allows us to give some inputs for Thursday’s online session (12th May 2022).

Phase II：Please give your feedback before Monday, 16th May 2022, 23:59 UTC.

For the Chairman’s Notes

Summary of Phase III

Based on the comments from Phase-III discussion, following potential proposals are achieved.

Proposal 1: Agree the LS in R3-224061 for concurrent TNL migration.

Summary of Phase II

Based on the comments from Phase-II discussion, following potential proposals are achieved.

Proposal 1-1: Add the following texts in Step 12 of intra-CU topology adaptation (section 8.2.3.1)

- If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the migrating IAB-node and the IAB-donor-CU may be established using the new TNL address information of the migrating IAB-node. The migrating IAB-node sends an F1AP gNB-DU CONFIGURATION UPDATE message to the IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for the F1-U traffic to be switched to the target path.

Proposal 1-2: Add the following texts in Step 11 of intra-CU topology redundancy (section 8.2.4)

- If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the dual-connecting IAB-node and the IAB-donor-CU may be established using the new TNL address information of the dual-connecting IAB-node. The dual-connecting IAB-node sends an F1AP gNB-DU CONFIGURATION UPDATE message to the IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for the F1-U traffic to be switched to the target path.

Proposal 1-3: Add the following texts in Step 15 of inter-CU topology adaptation (section 8.17.3.1)

- If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the migrating IAB-node and the F1-terminating IAB-donor-CU may be established using the new TNL address information of the migrating IAB-node. The migrating IAB-node sends an F1AP gNB-DU CONFIGURATION UPDATE message to the F1-terminating IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for the F1-U traffic to be switched to the target path.

Proposal 1-4: Adjust the sequence between gNB-DU CONFIGURATION UPDATE and SCTP association in Step 9 of section 8.17.2.1.

Proposal 2: Make following corrections for revoking in Section 8.17.3.1.

- Replace “In this case” with “The non-F1-terminating IAB-donor-CU can initiate the full revoking of traffic offload by executing the XnAP Handover Preparation procedure for the migrating IAB-MT. After” and start with a new paragraph

- Delete “and” in “and the traffic of the migrating IAB-DU…”

- Remove the following texts “The non-F1-terminating IAB-donor-CU can trigger the full traffic revoking by executing the XnAP Handover Preparation procedure for the migrating IAB-MT towards the F1-terminating IAB-donor-CU.”

Proposal 3: Make following corrections for inter-CU topology adaptation procedure in section 8.17.3.1

- Replace “each F1-U tunnel and non-UP traffic type” with “F1-U traffic” in Step 15

- Update Step 20 as “Repetition of steps 16 to 19, as needed, where the source IAB-donor-CU can request the offload of further traffic, or the modification or release of offloaded traffic. The target IAB-donor-CU can fully or partially reject addition or modification requests by the source IAB-donor-CU.”

Proposal 4: Send an LS to RAN2 informing the following alternatives to avoid the packet discarding due to no matched routing entries during parallel TNL migration of the descendant nodes in IAB intra-CU topology adaptation.

- Alt. 1: the IAB-donor-CU pre-configures the BH routing configurations before sending the RRCReconfiguration message to parent node of each descendant node

- Alt. 2: In case the BH Routing Configuration has not been (re)configured by F1AP, the descendant IAB-node of the migrating IAB node use the default configuration, e.g., default BH RLC CH to forward the TNL migration related UL packets received from the child node, to the parent node corresponding to the default configuration.

Proposal 5: Support Option 1 (via RRC signaling) for IP address request of the boundary IAB-node in IAB inter-CU topology redundancy procedure, and make following corrections in section 8.17.2.1.

- Remove “This message may include a request for new TNL address(es).”in Step 2.

Proposal 6:

In section 8.17.1, replace “In the intra-donor-CU scenario for UL inter-donor-DU rerouting … controlled by different IAB-donor-CUs” with “The IAB-donor-DU and the peer IAB-donor-DU may be controlled by the same IAB-donor-CU or by two different IAB-donor-CUs”

Proposal 7: Agree the CR (R3-223939) for IAB stage-2 corrections in TS 38.401.

Summary of Phase I

Based on the comments below, following potential proposals are achieved.

Inter-CU Topology Redundancy (section 8.17.2.1):

Proposal 1-2: Remove the following NOTE, and remove the similar NOTE in section 8.17.3.1.

- NOTE: The non-F1-terminating IAB-donor-CU should select the same IAB-donor-DU in its topology for all to-be-offloaded traffic, whose UL BH mappings received from the F1-terminating IAB-donor-CU in step 2 share the same BAP address.

Proposal 1-3: Add the following NOTE in Step 9.

- NOTE: The IP address selected by the boundary node for a traffic type needs to be anchored at the IAB-donor-DU whose BAP address is contained in the BAP routing ID of the UL mapping for this traffic type, and configured by donor (MN/SN) of the egress topology.

Proposal 1-4: Add the following texts in Step 9.

- The dual-connecting IAB-node sends F1AP gNB-DU CONFIG UPDATE to the F1-terminating IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for offloaded F1-U traffic. If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the dual-connecting IAB-node and the F1-terminating IAB-donor-CU may be established using the new TNL address information of the dual-connecting IAB-node.

Inter-CU Topology Adaptation (section 8.17.3.1):

Proposal 2-1: Add the description for MOBIKE in Step 15.

Proposal 2-2: Step 1 for descendant IAB-node may be performed in parallel with the Step 0 after the source IAB-donor-CU receives HANDOVER REQUEST ACKNOWLEDGE message.

Proposal 2-3: Remove the following texts.

- The non-F1-terminating IAB-donor-CU can trigger the full traffic revoking by executing the XnAP Handover Preparation procedure for the migrating IAB-MT towards the F1-terminating IAB-donor-CU.

Proposal 2-4: Correct the description in step 15 and step 20 in IAB inter-CU topology adaptation procedure in section 8.17.3.1 to clarify how could source CU obtain the new IP addresses and transfer the new IP addresses to the target CU.

Intra-CU Topology Adaptation (section 8.2.3.1):

Proposal 3: In case no routing entry exists for the received packets, use the default BH RLC CH to forward the TNL migration related UL packets received from the child node.

Inter-CU BH RLF Recovery (section 8.17.4):

Proposal 4-1: Further clarification in Step 14 that the RRC configuration includes a BAP address for the recovery IAB-node in the new IAB-donor-CU’s IAB topology, default BH RLC channel and a default BAP routing ID configuration for UL F1-C/non-F1 traffic mapping on the recovery path.

Proposal 4-2: Use recovering IAB-node for the IAB-node which performs inter-CU BH RLF recovery.

IP Address Allocation (section 8.9.13):

Proposal 5: Add the following clarification of IP address allocation for inter-CU cases.

- In case of IAB inter-CU topology management, the F1-terminating IAB-donor-CU may obtain the IP addresses for each usage in non-F1-terminating IAB-donor-CU’s topology from the non-F1-terminating IAB-donor-CU for the boundary IAB-node and descendant IAB-nodes of the boundary IAB-node.

IAB-node Integration (section 8.12.1):

Proposal 6: Add the following NOTE for IAB node integration.

- NOTE: For OAM-based donor selection, if the IAB-node establishes NR-DC before the establishment of F1-C connection, the IAB-node indicates the F1-terminating donor node by signaling its IP address(es) to this donor node by using the RRC signaling.

IAB protocol stack (section 6.1.4):

Proposal 7: Add the protocol stacks for CP-UP separation scenarios 1 and 2 in section 6.1.4.

Discussion – 1st Round

Issues for IAB Inter-CU Topology Redundancy

In the last RAN3 meeting, following agreements were achieved for IAB Inter-CU Topology Redundancy procedure of the boundary node.

2a: The baseline procedure for inter-donor redundancy of the boundary node includes at least the following steps: 1). DC setup (IP address request or configuration need not be included) 2). XnAP Transport Migration Management Request: Includes QoS info per traffic without IP address. 3). RRC Reconfiguration by CU2: Includes configuration of additional IP addresses. 4). XnAP Transport Migration Management Response: Includes L2 info per traffic. 5). F1AP IAB UP CONFIGURATION UPDATE REQUEST: Includes new UL mappings for each migrated traffic 6). F1AP IAB UP CONFIGURATION UPDATE RESPONSE: Includes IP address selected for each migrated traffic 7). XnAP Transport Migration Management Request: Includes modification of each traffic with new IP address. 8). XnAP Transport Migration Management Response: Acknowledgement of modification. IP addresses are requested to CU2. Options for the request are either Step 1 or Step 2 above or via RRC. Details on how the are requested are FFS.

In contribution [2], IP addresses for the boundary node based on either Step1 or Step 2 are both discussed. And Step 1 is preferred in contribution [2] where RRC container with the boundary node’s IP address information is included in XnAP messages for dual connectivity setup or modification. And the corresponding CR is provided in contribution [3].

While in the current TS 38.401, IP address request for the boundary node is included in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message in Step 2, and the non-F1-terminating IAB-donor-CU sends the RRC configuration which include the new TNL address(es) anchored at the second-path IAB-donor-DU to the boundary node.

Q1-1: Please share your views on the corrections for IP address request for boundary node in IAB Inter-CU Topology Redundancy in contribution [3]. And please provide the corresponding text proposal based on the [3] or based on the latest TS 38.401, if needed.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Qualcomm |  | The proposals by [2] are incorrect. CU1 requests IP addresses from CU2 only for descendant nodes. For the boundary node in topology 2, the IP addresses are allocated by CU2 via RRC as captured in 38.401 section 8.17.2.1 IAB Inter-CU topological redundancy procedure in steps 3 and 4. This is based on agreement: For IP address addition, non-F1-terminating CU to configure IP addresses on the boundary node via Rel-16 RRC signalling, and boundary node reports the F1-U IP addresses it wants to use via Rel-16 F1AP signalling to the F1-terminating CU. |
| Ericsson |  | Same view as QC. |
| Huawei |  | We think [2] and [3] raised a valid issue. Currently, the step 2 in 8.17.2.1 stated that the IP address request may be included in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message, and the non-F1 terminating CU (CU2) will send DL RRC message includes the new TNL address(es) to the boundary node in step 3. For descendant nodes, CU1 includes the IP address request in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message, and CU2 will respond the IP address in the XnAP message send to CU1, and then CU1 configure the new IP address to descendant nodes via RRCReconfiguraiton. However, if the IP address request is included in the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message, CU2 is not able to know whether the IP address request in the XnAP message is for the boundary node or descendant nodes, then how can it know whether to include the new TNL address(es) in a RRCReconfiguraiton message and send to the boundary node directly, or include the TNL address in the XnAP respond message to CU1? Based on the above understanding, the intention of the change proposed by [3] make sense to us, while some details improvement are still expected. Suggest the following proposal: Proposal: The new TNL address request and allocation for the boundary IAB-node in the inter-donor redundancy scenario are performed during the DC setup procedure. |
| ZTE |  | We prefer to request for IP address for the boundary node via step 1, i.e. via RRC container in NR-DC establishment procedure. As analyzed in [2], if the IP address for boundary node is requested via step 2, non-F1-terminating donor would send the allocated IP addresses via the response message to the F1-terminating donor since it cannot differentiate whether the IP address request is for boundary node or for descendant node. And the non-F1-terminating donor doesn’t know how many IP addresses the boundary requires. And then F1-terminating donor can send the IP addresses to the boundary node via RRC, which is not aligned with the agreements. Regarding QC’s comment, we don’t think CU1 requests IP addresses from CU2 only for descendant nodes. We think CU1 needs to request IP addresses from CU2 for the boundary node as well. Otherwise, CU2 doesn’t know exactly how many IP addresses the boundary node requires. |
| Fujitsu |  | We think the corrections in [3] is necessary. The present spec text for IP addresses request of the boundary node is based on Step 2. If the IP addresses for boundary node are requested in step 2, i.e., using XnAP transport migration management procedure, non-F1-terminating CU will not know the IP addresses request is for boundary node or descendant nodes. Non-F1-terminating CU should always provide the allocated IP addresses in the response message upon receiving IP address request in the transport migration request message. For scenario that SN is F1-terminating donor, if the allocated IP addresses are provided to F1-terminating donor (SN) from non-F1-terminating CU (MN), the F1-terminating CU (SN) cannot configure the allocated IP address via RRC if SRB3 is not set up. To answer QC: It is true we have such an agreement. If the IP address request is from the boundary node, it’s not clear how the boundary node knows the number of IP addresses needed for the migrated traffic, because it should be the F1-terminating CU rather the boundary node to decide the traffic to be migrated. We think the most reasonable solution is step 1. |
| Nokia |  | Agree with QC. We are not sure the proposal can help. Without the QoS info, CU2 can only “blindly” assign the TNL address. It is better the TNL address is assigned with the QoS. |
| Lenovo |  | Agree with QC that the IP address addition for the boundary IAB-node is based on the agreement above. |
| Samsung |  | Agree with QC |

Summary:

Based on the comments above, (3/8) companies support IP address request for the boundary IAB-node during inter-CU topology redundancy via DC setup procedure in Step 1. And (5/8) companies support IP address request via the RRC signalling.

Since the correction is not agreed by majority, no conclusion will be made for this issue.

In contribution [3], it’s proposed to add the following texts for non-F1-terminating CU initiated traffic release procedure for inter-CU topology redundancy.

The traffic offload for descendant nodes follows the same procedure as defined for the partial migration in clause 8.17.3.2. The F1-terminating IAB-donor-CU may request full or partial release of the offloaded traffic from the non-F1-terminating IAB-donor-CU by initiating the IAB Transport Migration Management procedure towards the non-F1-terminating IAB-donor-CU (e.g., for the purpose of revoking or in case UE bearers are released). The non-F1-terminating IAB-donor-CU may request full or partial release of the offloaded traffic by initiating the IAB Transport Migration Modification procedure towards the F1-terminating IAB-donor-CU (e.g., for the purpose of revoking). The traffic offloaded through the inter-CU topological redundancy procedure described in steps 1 to 12 (including the offload of traffic pertaining to the dual-connecting boundary IAB-node and its descendant IAB-nodes and their served UEs) can be partially or fully revoked, resulting in the return of the offloaded traffic back to the F1-terminating IAB-donor-CU’s topology. Full or partial traffic revoking can be initiated by the F1-terminating IAB-donor-CU by initiating the IAB Transport Migration Management procedure towards the non-F1-terminating IAB-donor-CU. The non-F1-terminating IAB-donor-CU can request partial or full traffic revoking from the F1-terminating IAB-donor-CU by initiating the IAB Transport Migration Modification procedure towards the F1-terminating IAB-donor-CU.

However, in contribution [9], they think that the description of the second paragraph has been covered in the third paragraph and the whole second paragraph looks redundant.

Q1-2: Do you agree to add the texts for non-F1-terminating CU initiated traffic release procedure for inter-CU topology redundancy? Or to delete the second paragraph?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | No | This is already captured in the third paragraph |
| Ericsson | No | The 3rd paragraph above captures it already. |
| Huawei | No | Agree with QC and Ericsson that the content has already been captured. |
| ZTE | No | Same view as above. |
| Fujitsu |  | OK, we can compromise. The 3rd paragraph also captures the F1-terminating CU initiated traffic revocation, we understand the second paragraph should be deleted. |
| Nokia | No | This seems already covered by the next paragraph, e.g. last sentence. |
| Lenovo |  | The second paragraph has been covered by the next parachurch. Suggest removing the whole second paragraph. |
| Samsung | No | The added part is redundant. |

Summary:

All companies think the texts proposed by [3] are redundant and most of companies think the original texts in second paragraph have been already covered by the next paragraph. So, we can have the following proposal:

Proposal 1-1: Delete the following texts in section 8.17.2.1.

- The F1-terminating IAB-donor-CU may request full or partial release of the offloaded traffic from the non-F1-terminating IAB-donor-CU by initiating the IAB Transport Migration Management procedure towards the non-F1-terminating IAB-donor-CU (e.g., for the purpose of revoking or in case UE bearers are released).

In contributions [4], [5] and [11], it’s proposed to remove the following NOTE in the IAB Inter-CU topological redundancy procedure and IAB inter-CU topology adaptation procedure since there is no BAP header re-writing for the boundary IAB’s F1-C/U.

NOTE:	The non-F1-terminating IAB-donor-CU should select the same IAB-donor-DU in its topology for all to-be-offloaded traffic, whose UL BH mappings received from the F1-terminating IAB-donor-CU in step 2 share the same BAP address.

While in contributions [8] and [9], they think the intention of the above NOTE is for the source IP address selection for the descendant nodes of the boundary node and the content is not appropriate for the boundary node. And they propose another NOTE as follow.

NOTE:	The boundary node selects the TNL address for UP/non-UP traffic to be forwarded towards the (Non)-F1-terminating topology corresponds to an anchored donor DU’s BAP address, among the TNL address(es) configured by donor (MN/SN) of the egress topology.

Q1-3: Do you agree to remove the first NOTE above in the IAB Inter-CU topology redundancy procedure and IAB inter-CU topology adaptation procedure? And do you agree to add the second NOTE?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM |  | On the removal of the original NOTE: Indeed, this NOTE should be removed in this sections on boundary nodes, but it should be included in section 8.17.3.2, Migration of the DESCENDENT node. On the new NOTE: It makes sense to include such a Note but it is needs to be made more comprehensible. We propose: NOTE: The IP address selected by the boundary node for a traffic type needs to be anchored at the BAP address contained in the BAP routing ID of the UL mapping for this traffic type. |
| Ericsson | Yes and yes | Same view as QC, with the following modification of the QC version of the new NOTE: NOTE: The IP address selected by the boundary node for a traffic type needs to be anchored at the IAB-donor-DU whose BAP address is contained in the BAP routing ID of the UL mapping for this traffic type. |
| Huawei | Yes for both | Suggest some rewording on the new NOTE based on the Ericsson and QC’s version, in case the two donor DU in two topology has same BAP address: NOTE: The IP address selected by the boundary node for a traffic type needs to be anchored at the IAB-donor-DU whose BAP address is contained in the BAP routing ID of the UL mapping for this traffic type, and configured by donor (MN/SN) of the egress topology. |
| ZTE | Yes | - We agree to remove the first NOTE above. - We are ok to add another NOTE with some rewording: NOTE: The IP address selected by the boundary node for a traffic type needs to be associated with the donor DU BAP address which is contained in the BAP routing ID of the UL mapping for this traffic type, and configured by donor (MN/SN) of the egress topology. |
| Fujitsu | See comment | - Agree to remove the first NOTE. - Understand and agree to the intention that the boundary node should select the TNL address for the traffic according to the relative topology. In fact, the boundary node should firstly differentiate the topology of TNL address configuration based on which CU configures the TNL address and then select the correct TNL address. Since the performance is complicated, the text from HW is too vague to instruct the performance. The text from QC is not complete. We suggest revising the second NOTE as following: NOTE: The TNL address selected by the boundary node for a traffic type needs to be anchored at the BAP address belonging to the topology contained in the BAP routing ID of the UL mapping for this traffic type. The boundary node can derive which topology the TNL address of the BAP address belongs to basing on which donor-CU (F1-terminating CU/non-F1-terminating CU) provides the TNL address configuration. |
| Nokia | Yes | The first NOTE is incorrect and should be removed. For the 2nd NOTE, prefer ZTE text, and change “donor DU” to “IAB-donor-DU” |
| Lenovo | Yes for Both | The first NOTE needs to be removed for the procedures of boundary IAB-node. Fine with the wordings above for the second NOTE and prefer Nokia’s version. |
| Samsung | Yes for both |  |

Summary:

For the first NOTE, all companies agree to remove it.

And for the second NOTE, all companies think that it’s necessary and provide some further refinements for the wording.

Proposal 1-2: Remove the following NOTE in section 8.17.2.1, and remove the similar NOTE in section 8.17.3.1.

- NOTE: The non-F1-terminating IAB-donor-CU should select the same IAB-donor-DU in its topology for all to-be-offloaded traffic, whose UL BH mappings received from the F1-terminating IAB-donor-CU in step 2 share the same BAP address.

Proposal 1-3: Add the following NOTE in Step 9 of section 8.17.2.1.

- NOTE: The IP address selected by the boundary IAB-node for a traffic type needs to be associated with the IAB-donor-DU BAP address which is contained in the BAP routing ID of the UL mapping for this traffic type, and configured by donor (MN/SN) of the egress topology.

In contribution [6], F1AP gNB-DU CONFIG UPDATE and SCTP association establishment are proposed to be added in Step 9 of IAB inter-CU topology redundancy with following correction.

9. The F1-terminating IAB-donor-CU updates the boundary node with the UL BH information received form the non-F1-terminating IAB-donor-CU in Step 8. This step may also update UL FTEID and DL FTEID associated with individual GTP-tunnel(s). The affected GTP tunnel(s) will be switched to use the dual-connecting IAB-node’s new TNL address(es). This step may use non-UE associated signaling in E1 and/or F1 interface to provide updated UP configuration for F1-U tunnels of multiple connected UEs or child IAB-MTs. Implementation must ensure the avoidance of potential race conditions, i.e., no conflicting configurations are concurrently performed using UE-associated and non-UE-associated procedures. The F1-terminating IAB-donor-CU may also provide UL BH information associated with non-UP traffic. New TNL addresses for F1-C traffic configured in step 3, if any, can be added to the dual-connecting IAB-DU’s F1-C association(s) with the F1-terminating IAB-donor-CU. The dual-connecting IAB-node sends F1AP gNB-DU CONFIG UPDATE to the F1-terminating IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for offloaded F1-U traffic. If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the dual-connecting IAB-node and the F1-terminating IAB-donor-CU may be established using the new TNL address information of the dual-connecting IAB-node.

In contribution [11], it’s also proposed to clarify that the boundary node updates the IAB-donor-CU with DL IP addresses it decided to use for the offloaded traffic.

Q1-4: Please share your view to add the F1AP gNB-DU CONFIG UPDATE and SCTP association establishment in Step 9?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | Yes | This step is essential. The boundary DC IAB-node needs to tell CU1 which IP address is uses for each offloaded traffic. The inner IP address for offloaded F1-U does not need to change. |
| Ericsson | Yes |  |
| Huawei | Yes |  |
| ZTE | Yes |  |
| Fujitsu | See comments | Agree to add the F1AP gNB-DU CONFIG UPDATE in step 9, but SCTP association establishment is not necessary. |
| Nokia | See comments | This is obvious, and technically correct. Whenever the IAB receives new TNL address, it needs to establish new SCTP, and send the F1AP DU CONFIGURATION UPDATE. This applies to all intra/inter-CU migration/redundancy. So is it really needed to only capture it for inter-CU? |
| Lenovo | Yes | And change “gNB-DU CONFIG UPDATE” to “GNB-DU CONFIGURATION UPDATE”. |
| Samsung | Yes |  |

Summary:

All companies think F1AP GNB-DU CONFIGURATION UPDATE is valid, and majority companies except one think the SCTP establishment is also necessary.

Then moderator think we can have the quite proposal below for this issue, and we can further refine the wording later.

Proposal 1-4: Add the following texts in Step 9 of section 8.17.2.1.

- The dual-connecting IAB-node sends F1AP gNB-DU CONFIG UPDATE to the F1-terminating IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for offloaded F1-U traffic. If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the dual-connecting IAB-node and the F1-terminating IAB-donor-CU may be established using the new TNL address information of the dual-connecting IAB-node.

In contribution [6], it’s proposed that the non-F1-terminating donor cannot obtain boundary node’s new IP address for each GTP-U tunnel until step 10. So the DL mapping configured by non-F1-terminating IAB-donor in step 7 is only for CP traffic while the DL mapping configured in step 10 is for UP and non-UP traffic, which is similar as in inter-donor migration procedure.

Q1-5: Do you agree that DL mapping configured by non-F1-terminating IAB-donor in step 7 is only for CP traffic and the DL mapping configured in step 10 is for UP and non-UP traffic?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | No | This DL mapping in step 7 is necessary for F1-C but it can also be used for F1-U. There is no reason to restrict it to F1-C only. |
| Ericsson | No | Same view as QC |
| Huawei | No | We think the DL mapping configuration in step 7 is not possible for either CP or UP traffic. Due to that the CU2 is lack of the new TNL address information. Although CU2 provides the new TNL address for the boundary node at earlier step, e.g., via RRCReconfiguraion in step 3, but the new TNL address allocation only indicates the boundary node which IP address(es) is(are) used for F1-C/F1-U/non-F1 traffic, CU2 has no idea about which new IP address will be selected by the boundary node for each traffic. For example, if CU2 allocates two new IP address (IP1, and IP2) for CP traffic of boundary IAB-node, and the boundary node will use the IP1 for UE associated F1AP, and use IP2 for the Non-UE associated F1AP, then how can the CU2 perform DL mapping for boundary node’s CP traffic to the IAB-donor-DU before it knows which IP address is selected for UA/NUA F1AP message? Similar issue also applies to the UP traffic. That’s why we observe that the DL mapping configuration in the step 7 is not correct and should be removed. |
| ZTE | Yes | In step 7, the non-F1-terminating node is not able to configure DL mapping for UP traffic since it can’t obtain boundary node’s new IP address for each GTP-U tunnel. So the DL mapping configured by non-F1-terminating IAB-donor in step 7 is only for non-UP traffic. |
| Fujitsu | Yes |  |
| Nokia | No | Existing text is valid, e.g. only one TNL address is assigned for F1-C/U. |
| Lenovo | No | Same view as QC. |
| Samsung | No | Agree with QC |

Summary:

(6/8) companies don’t support the correction introduced by [6], and 5 of them think that DL mapping configured by non-F1-terminating IAB-donor in step 7 is for CP or UP traffic, and 1 of them thinks that DL mapping configured by non-F1-terminating IAB-donor in step 7 is not possible for either CP or UP traffic.

(2/8) companies support the corrections.

Since the corrections don’t receive support from majority, no conclusion will be made for this issue.

In contribution [9], it’s proposed that the non-F1 terminating donor is not aware of the DL TNL address for a given offloaded F1-U/C traffic before Step 7, thus the DL mapping configuration cannot be performed in this step.

7. The non-F1-terminating IAB-donor-CU may configure or modify BH RLC channels and BAP-sublayer routing entries on the second path between the dual-connecting IAB-node and the second-path IAB-donor-DU as well as DL mappings on the second-path IAB-donor-DU for the dual-connecting IAB-node’s second path. These configurations may support the transport of UP and non-UP traffic on the second path.

Similar problems observed in step 14 in Section 8.17.3.1, and step 2 in 8.17.3.2.

Q1-6: Do you agree that the non-F1 terminating donor is not aware of the DL TNL address for a given offloaded F1-U/C traffic before Step 7? If yes, do you agree to delete the corresponding sentence in Step 7 of Section 8.17.2.1, Step 14 of Section 8.17.3.1, and Step 2 of Section 8.17.3.2?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | No | CU2 configures the new TNL addresses used in topology 2 via RRC directly on the boundary node in step 3. It can use the knowledge of these addresses to configure the DL mapping in Step 7. |
| Ericsson | No | Same view as QC |
| Huawei | Yes for both | We think the DL mapping configuration in step 7 is not possible for either CP or UP traffic. Due to that the CU2 is lack of the new TNL address information. Although CU2 provides the new TNL address for the boundary node at earlier step, e.g., via RRCReconfiguraion in step 3, but the new TNL address allocation only indicates the boundary node which IP address(es) is(are) used for F1-C/F1-U/non-F1 traffic, CU2 has no idea about which new IP address will be selected by the boundary node for each traffic. For example, if CU2 allocates two new IP address (IP1, and IP2) for CP traffic of boundary IAB-node, and the boundary node will use the IP1 for UE associated F1AP, and use IP2 for the Non-UE associated F1AP, then how can the CU2 perform DL mapping for boundary node’s CP traffic to the IAB-donor-DU before it knows which IP address is selected for UA/NUA F1AP message? Similar issue also applies to the UP traffic. That’s why we observe that the DL mapping configuration in the step 7 is not correct and should be removed. |
| ZTE | Yes, and No | We agree that the non-F1 terminating donor is not aware of the DL TNL address for a given offloaded F1-U/C traffic before Step 7. But we think the above sentence should not be removed since the non-F1 terminating donor could DL mapping for non-UP traffic for the boundary node based on the allocated TNL address(es). |
| Fujitsu | No | Step 7 is for non-UP traffic configuration, suggest revising step 7 as following: 7. The non-F1-terminating IAB-donor-CU may configure or modify BH RLC channels and BAP-sublayer routing entries on the second path between the dual-connecting IAB-node and the second-path IAB-donor-DU as well as DL mappings on the second-path IAB-donor-DU for the dual-connecting IAB-node’s second path. The DL mappings may be based on the TNL address(es) allocated to the dual-connecting IAB-node for non-UP traffic in step 3. These configurations may support the transport of non-UP traffic on the second path. Similar change is needed for Step 14 of Section 8.17.3.1, and Step 2 of Section 8.17.3.2 |
| Nokia | No |  |
| Lenovo | No | Same view as QC |
| Samsung | No | Agree with QC |

Summary:

(7/8) companies don’t support the correction introduced by [9], 4 of them further clarify that CU2 can use the new TNL addresses on the boundary node to configure the DL mapping in Step 7. And 2 of them think the DL mappings may be based on the TNL address(es) allocated to the dual-connecting IAB-node in step 3 only for non-UP traffic.

(1/8) company support the corrections.

Since the corrections don’t receive support from majority, no conclusion will be made for this issue.

In contribution [10], they want to clarify the header rewriting, routing update, and BAP mapping configuration update for inter-donor topology redundancy as below.

9. The F1-terminating IAB-donor-CU updates the boundary node with the UL BH information received form the non-F1-terminating IAB-donor-CU in Step 8. This step may also update UL FTEID and DL FTEID associated with individual GTP-tunnel(s). The affected GTP tunnel(s) will be switched to use the dual-connecting IAB-node’s new TNL address(es). This step may use non-UE associated signaling in E1 and/or F1 interface to provide updated UP configuration for F1-U tunnels of multiple connected UEs or child IAB-MTs. Implementation must ensure the avoidance of potential race conditions, i.e., no conflicting configurations are concurrently performed using UE-associated and non-UE-associated procedures. The F1-terminating IAB-donor-CU may also provide UL BH information associated with non-UP traffic. New TNL addresses for F1-C traffic configured in step 3, if any, can be added to the dual-connecting IAB-DU’s F1-C association(s) with the F1-terminating IAB-donor-CU. The F1-terminating IAB-donor-CU also configures the dual-connecting IAB-node with the BAP-sublayer routing, header-rewriting and BH RLC CH mapping entries.

In addition, same clarification is added for inter-donor topology adaptation procedure.

Q1-7: Do you agree to clarify the header rewriting, routing update, and BAP mapping configuration update for inter-donor topology redundancy and inter-donor topology adaptation procedure?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | No | This section is on the boundary node. Header rewriting and BH RLC CH mapping only applies to descendent-node traffic. It is properly captured in 8.17.3.2. |
| Ericsson | No | Wrong section |
| Huawei | Yes | Agree with QC that this should be captured in 8.17.3.2 |
| ZTE | No | Agree with QC. |
| Fujitsu | No | No need to add the description, since the BAP-sublayer routing, header-rewriting and BH RLC mapping are configured for the traffic of descendant nodes. For the traffic of the boundary node, the UL BH information is used. |
| Nokia | No | Header rewriting is not needed for boundary node’s traffic. Ok to add it in 8.17.3.2 |
| Lenovo | No | And these configurations are already captured in Step 5 of section 8.17.3.2. |
| Samsung |  | QC’s claim on “This section is on the boundary node” may not be precise since in this section, we also have the following sentence, i.e., The traffic offload for descendant nodes follows the same procedure as defined for the partial migration in clause 8.17.3.2. However, with this sentence, the added sentence may not be needed since Step 5 in Section 8.17.3.2 address this. So, we are fine to not have such modification. |

Summary:

All companies agree that the clarification is for descendant node rather than for boundary node.

In addition, these configurations are already captured in Step 5 of section 8.17.3.2. So, existing text is fine, and no further correction is needed for this issue.

Issues for IAB Inter-CU Topology Adaptation

In both contributions [5] and [6], same text of MOBIKE is proposed to be added in the inter-CU topology adaptation procedure.

Q2-1: Please share your views on adding the texts for MOBIKE in the inter-CU topology adaptation procedure? And if you agree with it, please share your preference on Opt.1 (after Step 15[5]) or Opt.2 (after Step 19[6])?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | Yes | Option 1 is correct. MOBIKE can already be applied to the migration of F1-C, which is migrated in Step 15. Option 2 introduces this too late, i.e., it appears as if it only applied to F1-U. |
| Ericsson | Option 1 |  |
| Huawei | Option 1 |  |
| ZTE | Yes | Ok to add the text for MOBIKE after step 15, i.e. option 1. |
| Fujitsu | Option 1 |  |
| Nokia | Yes | Option 1 (after Step 15), since it is part of the F1-C migration procedure, i.e. reuse the inner IP address, SCTP, etc. |
| Lenovo | Yes | Option 1. |
| Samsung | Option 1 | After adding in Step 5 in Section 8.17.3.1, the option 2 is not needed, since Step 10 in Section 8.17.3.2 is given as “10. The F1-C connections may be switched to use the migrating IAB-node’s new TNL address(es), if any, as described in Step 15 of the inter-CU topology adaptation procedures in section 8.17.3.1.” |

Summary:

All companies think the texts for MOBIKE in the inter-CU topology adaptation procedure is necessary, and option 1 is supported by all of them.

Proposal 2-1: Add the description for MOBIKE in Step 15 in section 8.17.3.1.

In contribution [6], it’s proposed that the source IAB-donor-CU couldn’t derive the new IP addresses for each F1-U tunnel until F1-U redirection (i.e. step 19). So, the migrating IAB-node cannot report IP address for each F1-U tunnel and non-UP traffic type via the gNB-DU configuration update message in Step 15, and two the IAB Transport Migration Management procedure needs to be performed twice. For the first one, it doesn’t include the new DL TNL address information necessary for the target IAB-donor-CU to configure or modify DL mappings on the target IAB-donor-DU and it can be performed earlier, e.g. after receiving HO request ACK in step 4. And the second one is introduced after Step 19 which include the new DL TNL address information necessary for the target IAB-donor-CU to configure or modify DL mappings on the target IAB-donor-DU.

Q2-2: Please share your views on the changes above for IAB inter-CU topology adaptation?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM |  | For F1-U, everything is fine. CU1 sends the original IP addresses used by the IAB-node for F1-U in the RRC container to CU2, and CU2 simply replaces them. The gNB-DU then reports the F1-U IP addresses it uses via the gNB-DU Config Update. Therefore, CU1 knows the new IP addresses used by the IAB_node for F1-U. All of this works fine. The IP addresses for non-F1 are not reported by the IAB-node in the gNB-DU Config Update, and CU1 therefore doesn’t know them. However, it is sufficient that CU2 knows these IP addresses so that it can configure the DL mapping. In summary: the change above is not correct. |
| Ericsson | Disagree | CU1 need not know the IP addresses for non-F1 traffic, since this traffic does not pass through CU1. For F1-U, there is no problem. |
| Huawei |  | Even for F1-U traffic, there may still have some problem. CU2 can know which new IP is used to replace which old IP address in advance, but CU2 cannot know which new IP address will be used for which traffic (or for which F1-U tunnel), because CU2 has no idea about which F1-U tunnel use which old IP address. Then the DL mapping configuration for the IAB-donor-DU in CU2’s topology will not be performed correctly, before the CU2 get the new IP address information via the IAB Transport migration management procedure. The gNB-DU Config Update only provides CU1 the information of inner IP address(es) and the new outer IP address(es) for F1-U traffic, if the inner IP address is same as before (e.g. using MOBIKE), then CU1 can derive the new outer IP address to be used for each F1-U tunnel implicitly. However, if the inner IP address also changes, CU1 will not know the new outer IP address for each F1-U tunnel. In such sense, the issue raised by [6] seems valid. |
| ZTE | Yes | For F1-U traffic, there is a problem when MOBIKE is not applied: The F1AP gNB-DU CONFIG UPDATE message sent to the source CU in step 15 includes only new (outer) IP addresses and corresponding new (inner) IP address for F1-U traffic. If MOBIKE is used, source CU would know that the same inner IP address is used after migration upon receiving F1AP gNB-DU CONFIG UPDATE message. However, source CU can’t obtain the new inner IP address until F1-U redirection in step 17 if MOBIKE is not applied. So it’s incorrect to say that “The migrating IAB-node may report the new TNL address information it wants to use for each F1-U tunnel and non-UP traffic type to the source IAB-donor-CU via the gNB-DU CONFIGURATION UPDATE message.” in step 15. |
| Fujitsu | Not strong view |  |
| Nokia |  | In Step 15, the existing text “for each F1-U tunnel” causes the confusion. The DU Configuration Update message does not contain the information for each F1-U tunnel. So the “each” should be deleted. For “it can be performed earlier, e.g. after receiving HO request ACK in step 4.” This is not ok, since source CU does not know if the HO succeeded before the step 12. No need to add the new steps, since current text states “Repetition of steps 16 to 19, as needed, where the source IAB-donor-CU can request addition, modification or release of QoS information for non-UP and UP traffic.” |
| Lenovo |  | Agree with Huawei and ZTE that migrating IAB-node cannot report IP address for each F1-U tunnel in Step 15 if the MOBIKE is not used. We and delete the “each” in the “for each F1-U tunnel” and add the corresponding description after Step 19. As for the second IAB Transport Migration Management procedure, it’s not needed and can be realized by the repetition steps. |
| Samsung | Disagree | We share the same understanding as QC |

Summary:

Companies have much divergence for these corrections.

(4/8) companies agree that the source IAB-donor-CU couldn’t derive the new IP addresses for each F1-U tunnel in Step 15 until F1-U redirection (i.e. step 19). While (3/8) companies think everything is fine for F1-U in the current procedure. And 1 company has no strong view on it.

As for the necessary of two IAB Transport Migration Management procedure, we cannot find the support based on the comments above.

Since the corrections don’t receive support from majority, no conclusion will be made for this issue.

In contribution [6], it’s proposed that the Step 1 for descendant IAB-node can be performed in parallel with the Step 0 after the source IAB-donor-CU receives HANDOVER REQUEST ACKNOWLEDGE message. For example, the context of the traffic to be offloaded for the migrating/descendant nodes and IP address request information could be contained in the same IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message.

Q2-3: Do you agree that Step 1 for descendant IAB-node can be performed in parallel with the Step 0?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | Yes |  |
| Ericsson | OK |  |
| Huawei | Fine |  |
| ZTE | Yes |  |
| Fujitsu | No | Before step 19 of inter-CU topology adaption for the boundary node, the routing configuration for non-F1-terminating topology is not ready in the boundary node. According to 38.340, when a BAP Data PDU that contains a BAP address which is not included in the configured BH Routing Configuration the BAP entity shall discard the BAP PDU. If the procedure for descendant IAB-node is performed in parallel with that of boundary node, the UL data from descendant node may be discared by the boundary node. |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| Samsung | Yes |  |

Summary:

(7/8) companies are fine with the corrections.

(1/8) company doesn’t support it because of the potential UL data discard for descendant node.

However, Step 1 for descendant IAB-node only includes the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message, there is no risk for UL data discard for descendant node.

Proposal 2-2: Step 1 for descendant IAB-node can be performed in parallel with the Step 0 after the source IAB-donor-CU receives HANDOVER REQUEST ACKNOWLEDGE message.

In contribution [9], the second paragraph of traffic revocation seems not necessary in Section 8.17.3.1, since the previous paragraph has stated that the IAB-MT is handed over in reverse direction.

The traffic offloaded through the inter-CU topology adaptation described in steps 1 to 20 (including the offloaded traffic pertaining to the migrating IAB-node and its descendant IAB-nodes and their served UEs) can be fully revoked. In this case, the migrating IAB-MT is handed over in reverse direction, i.e., from the non-F1-terminating IAB-donor-CU to the F1-terminating IAB-donor-CU, and the traffic of the migrating IAB-DU and the descendant IAB-DUs is routed again along the former source path.

Q2-4: Do you agree to delete the second paragraph above for traffic revoke?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | Yes |  |
| Ericsson | Yes, but | The deleted part is a part of the next paragraph, not the entire nextparagraph! We would then prefer to reword the paragraph above to the following: The traffic offloaded through the inter-CU topology adaptation described in steps 1 to 20 (including the offloaded traffic pertaining to the migrating IAB-node and its descendant IAB-nodes and their served UEs) can be fully revoked. If the full revoking is initiated by the non-F1-terminating IAB-donor-CU, the migrating IAB-MT is handed over in reverse direction, i.e., from the non-F1-terminating IAB-donor-CU to the F1-terminating IAB-donor-CU, and the traffic of the migrating IAB-DU and the descendant IAB-DUs is routed again along the former source path. |
| Huawei | Yes |  |
| ZTE | Yes | The revision suggested by Ericsson above is not necessary since the migrating IAB-MT is handed over in reverse direction when the full revoking is initiated by the F1-terminating IAB-donor-CU as well. |
| Fujistu | No | The second paragraph for traffic revoking should not be deleted. The first paragraph clarifies the scenario of full revocation, i.e, the migrating IAB-MT is handed over back to F1-terminating CU when all traffic is revoked, while the second paragraph is saying the non-F1-terminating CU can trigger the full revocation via XnAP Handover Preparation procedure. |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| Samsung | Yes |  |

Summary:

(7/8) companies are fine to delete the text above proposed in [9]. And 1 company want to further clarify that the full revoking is initiated by the non-F1-terminating IAB-donor-CU, while it’s disagreed by another company.

(1/8) company doesn’t support it because of XnAP Handover Preparation procedure is not mentioned in the first paragraph.

For the XnAP Handover Preparation procedure to realize the revoke, it’s covered by the whole inter-CU migration procedure. We may no need to emphasize it.

Proposal 2-3: Remove the following texts in section 8.17.3.1.

- The non-F1-terminating IAB-donor-CU can trigger the full traffic revoking by executing the XnAP Handover Preparation procedure for the migrating IAB-MT towards the F1-terminating IAB-donor-CU.

Issues for IAB Intra-CU Topology Adaptation

To enable the parallel TNL migration of the descendant nodes, it’s proposed to clarify the routing entry in the migrating IAB and descendant IAB should be configured before the migration [4]. Otherwise, the migrating IAB cannot forward the further RRCReconfigurationComplete message received from child IAB node, and the parallel TNL migration cannot be performed. In addition, the corresponding stage-2 update in [5] is pasted below.

Based on implementation, these steps can be performed after or in parallel with the handover of the migrating IAB-node. To enable performing these steps in parallel, the IAB-donor-CU sends the RRCReconfiguration message with the new TNL address(es) and the new default UL mapping to the descendent node while the migrating IAB-MT is still connected with source parent node, for example, before Step 5. In this case, the UE CONTEXT MODIFICATION REQUEST message carrying this RRCReconfiguration message includes a conditional delivery indication for the descendent node’s parent IAB-DU to withhold the delivery of the RRCReconfiguration message, as specified in TS 38.473 [4]. The IAB-donor-CU also configure the migrating IAB-node and descendant IAB node(s) with routing entry, at least the routing entry for the default UL BAP Routing ID, while the migrating IAB-MT is still connected with source parent node.

Same issue is discussed in contribution [7], and 2 alternatives are given as follows to solve it,

- Alt #1: The trigger condition for the descendent IAB node to transfer the buffered RRC message to its child IAB node is that the descendent IAB-node has one or more routing entries for the target path.

- Alt #2: In case no routing entry exists for the received packets, use the default configuration of the descendant nodes to forward the TNL migration related UL packets received from the child node.

Q3: Please share your view on the trigger condition of descendant node? And do you agree to the stage-2 update in [5]?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM |  | We do not need to add new trigger conditions. We should solely add on St2 that the descendent nodes need to be configured with routing entries, and that this should be done before the migration. No need to confine this to the default mapping only: The IAB-donor-CU also configures the migrating IAB-node and descendant IAB node(s) with routing entryies, at least the routing entry for the default UL BAP Routing ID, while the migrating IAB-MT is still connected with source parent node. |
| Ericsson | See comment | We should not add any triggering conditions to stage2 – putting the agreed conditions in stage3 was the outcome of RAN3 discussion. If we are going to add any text, we prefer to refer to the default UL BRID only - we do not recall any agreement saying that multiple entries can be configured while the node is still connected to the source parent. |
| Huawei | See comment | As stated in our paper [8], the current trigger condition for descendant node will cause some problem. So we need to discuss how to solve it. About the two alternatives, we are open for both, but slightly prefer Alt 2, considering that alt 1 will result in more latency and diminish the benefit for the concurrent TNL migration. We disagree with the stage-2 update in [5], there is no such agreement to require the CU pre-configure some routing entries to the descendant IAB-nodes before the migrating IAB node’s migrating. And how to perform such pre-configuration is unclear, e.g., it may require some prediction of IAB-node’s migration but how to perform such prediction is not clear. |
| ZTE | See comment | We prefer Alt #2 as proposed in [7]. We think configuration of routing entry before the migration of the migrating MT would lead to additional service interruption during the migration procedure. And in some scenarios, the donor CU cannot be able to configure the migrating IAB-node and descendant IAB node(s) with routing entry before the migration of the migrating MT, e.g. when the radio condition deteriorates rapidly. |
| Fujitsu | See comment | Regarding to the issue that there is no routing entry available during the parallel migration, we prefer Alt.2, i.e., using the default configuration to forward the UL packets. According to present 38.340, the default configuration can be used to deliver the BAP data from upper layer when the routing configuration is not reconfigured. It’s not necessary to preclude using the default configuration to deliver the data received from child nodes. Based on Alt.2, we don’t have to restrict the IAB-donor-CU to configure the migrating node and descendant nodes with multiple routing entries before the parallel migration. The description added according to [5] is not needed if Alt.2 is applied, because the RRCReconfiguration message for descendant node includes the default UL mapping. |
| Nokia | Yes | Alt #1. In case CU want to support the parallel TNL migration, the CU should configure the routing entry for the target path in advance. |
| Lenovo | See comments | Prefer Alt #2. Alt #1 may introduce additional service interruption which is misaligned with the motivation to enhance for intra-CU migration. |
| Samsung |  | We didn’t make any agreement on configuring new routing entries toward target path before migration. In some cases, the migration happens due to signaling quality degradation. There is no time to do pre-configuration, and we cannot relies on this. In this sense, a IAB node may receive UL BAP PDU from descendant node where the routing entries may not exist, e.g., - UL BAP PDU with new default BAP header before new routing entries are received: efore donor CU configures new routing entries for the target path, the UL packets contain the new default UL BAP header, which belongs to the non-F1-U traffic of descendant node, such new header is for target path - UL BAP PDU with old BAP header after new routing entries are received: After donor CU configures new routing entries for the target path, the UL packets may contain the old BAP header (belonging to source path), which is received before migration, and buffered at the descendant node To avoid the discard of the above packets, the IAB node can use the default configuration (default BH RLC CH) to forward those packets. Please note that, currently, we only have default BH RLC CH for F1-C. In summary, we prefer to use default BH configuration (e.g., default BH RLC CH) to forward the packets without corresponding routing entries. Whether the default configuration for F1-C can be used or not needs further discussion. |

Summary:

(6/8) companies prefer alt. 2 since alt. 1 will introduce additional service interruption, and no need to add any stage-2 on trigger condition.

While (2/8) companies prefer alt.1.

Maybe we can make the following potential proposal based on the majority’s view.

Proposal 3: In case no routing entry exists for the received packets, use the default configuration of the descendant nodes to forward the TNL migration related UL packets received from the child node.

Issues for IAB Inter-CU Backhaul RLF recovery

In contributions [6] and [11], the following texts are added in the Step 14.

14. The new IAB-donor-CU sends a DL RRC MESSAGE TRANSFER message to the new parent IAB-DU, which includes an RRCReconfiguration message for the IAB-MT undergoing recovery. The RRC configuration may include new TNL addresses anchored at the new IAB-donor-DU. The RRC configuration includes a BAP address for the recovery IAB-node in the new IAB-donor-CU’s IAB topology, default BH RLC channel and a default BAP routing ID configuration for UL F1-C/non-F1 traffic mapping on the recovery path.

Q4-1: Do you agree to clarify that the RRC configuration further includes the BAP address and the default configuration?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | Yes |  |
| Ericsson | OK |  |
| Huawei | Ok |  |
| ZTE | Yes |  |
| Fujitsu | Yes |  |
| Nokia | See comments | Technically correct, but this is Stage-3 detail, and the content of the RRC message is in RAN2 scope. Is it really needed to add RAN2 stage-3 detail in RAN3 spec? |
| Lenovo | Yes |  |
| Samsung | Yes |  |

Summary:

All companies are fine with the clarification. And one of them wonders that whether to add the RAN3 stage-3 detail in RAN3 specs.

Based on the majority/s view. Following proposal is made.

Proposal 4-1: Further clarification in Step 14 that the RRC configuration includes a BAP address for the recovery IAB-node in the new IAB-donor-CU’s IAB topology, default BH RLC channel and a default BAP routing ID configuration for UL F1-C/non-F1 traffic mapping on the recovery path.

For the definition of the IAB-node which performs BH RLF recovery to another parent node underneath a different IAB-donor-CU. Following 3 options are suggested in different contributions.

- Option 1: Boundary IAB-node [5]

- Option 2: Recovery IAB-node [6]

- Option 3: Recovering IAB-node [1] [11]

Q4-2: Please share your preference for the definition of the IAB-node which perform inter-CU BH RLF recovery?

| Company | Option 1/2/3 | Comment |
| --- | --- | --- |
| QCOM | Opt 3, potentially Opt 1 | Option 2 is not good. There is not a “recovery node”, e.g. , as there is a “boundary node”. |
| Ericsson | Opt3 | Alternatively, “the IAB-node undergoing RLF recovery” (or “attempting”). |
| Huawei | Option 1 or 3 |  |
| ZTE | Option 2 | The term “recovery IAB-node” is used in the Figure 8.17.4-1. And this term is also used in intra-donor RLF recovery procedure in section 8.2.5. If the term “recovery IAB-node” should not be used, do we need to correct it in R16? |
| Fujitsu | Option 3 |  |
| Nokia | Option 1 | Please refer to the definition: Boundary IAB-node: an IAB-node with one RRC interface terminating at a different IAB-donor-CU than the F1 interface. This definition applies to partial migration, inter-donor redundancy and inter-donor RLF recovery. |
| Lenovo | Option 3 |  |
| Samsung | Opt 1 | Same view as Nokia |

Summary:

Option 3 is supported by (5/8) companies. Option 1 is supported by (4/8) companies. And Option 2 is supported by (1/7) companies.

Refer to the migration procedure, migrating IAB-node is also widely used. Similarity, we can also use the recovering IAB-node for BH RLF recovery procedure.

Proposal 4-2: Use recovering IAB-node for the IAB-node which performs inter-CU BH RLF recovery.

Issues for IP address allocation for IAB-nodes

In contribution [7], it’s proposed to add the stage-2 description of IP address allocation for IAB inter-CU topology management in clause 8.9.13 as follow.

In case of IAB-donor-based IP address allocation, the IP address(es) is(are) allocated by the IAB-donor-CU or IAB-donor-DU. In both cases, the IAB-node requests the IP address(es) via RRC from the IAB-donor-CU. It includes a separate IP address request for each usage, where the usages defined are all traffic, F1-U, F1-C and non-F1. The IAB-donor-CU may initiate the IAB TNL Address Allocation procedure to obtain IP addresses from the IAB-donor-DU. The IAB-donor-CU sends the IP addresses allocated for each usage to the IAB-node via RRC. In case of IAB inter-CU topology management, the F1-terminating IAB-donor-CU may obtain the IP addresses for each usage from the non-F1-terminating IAB-donor-CU for the boundary IAB-node and descendant IAB-nodes of the boundary IAB-node.

Q5: Do you agree to clarify IP address allocation for inter-CU cases in clause 8.9.13?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | Yes |  |
| Ericsson | OK |  |
| Huawei | YES |  |
| ZTE | OK |  |
| Fujitsu | Yes | Suggest a little change: In case of IAB inter-CU topology management, the F1-terminating IAB-donor-CU may obtain the IP addresses for each usage in non-F1-terminating IAB-donor-CU’s topology from the non-F1-terminating IAB-donor-CU for the boundary IAB-node and descendant IAB-nodes of the boundary IAB-node. |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| Samsung | Yes |  |

Summary:

All companies are fine with the clarification for IP address allocation for inter-CU cases in clause 8.9.13. And one company suggest small update on the wording.

Proposal 5: Add the following clarification of IP address allocation for inter-CU cases in clause 8.9.13.

- In case of IAB inter-CU topology management, the F1-terminating IAB-donor-CU may obtain the IP addresses for each usage in non-F1-terminating IAB-donor-CU’s topology from the non-F1-terminating IAB-donor-CU for the boundary IAB-node and descendant IAB-nodes of the boundary IAB-node.

Issues for IAB node Integration

In contribution [9], it’s proposed to add a NOTE to capture the following agreement for the OAM based F1-terminating donor selection in Section 8.12.1.

For OAM-based donor selection, the IAB-node indicates the F1-terminating donor node by signaling its IP address(es) to this donor node using the Rel-16 RRC-based signaling mechanism.

NOTE: If the IAB-node establishes NR-DC before the establishment of F1-C connection, and the IAB-donor is selected by OAM, the MN or SN of the IAB-node will perform as the F1-terminating donor if receiving IP address(es) of the IAB node through RRC signaling.

Q6-1: Do you agree to add the NOTE above for IAB node Integration?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | Yes |  |
| Ericsson | OK, but... | We prefer a note that resembles more the agreement: NOTE: For OAM-based donor selection, if the IAB-node establishes NR-DC before the establishment of F1-C connection, the IAB-node indicates the F1-terminating donor node by signaling its IP address(es) to this donor node by using the Rel-16 RRC signaling. |
| Huawei | Yes | Also ok with Ericsson’s version of the wording. |
| ZTE | Ok |  |
| Fujitsu | Yes |  |
| Nokia | Yes | Ok with Ericsson text, but no need to say “Rel-16” |
| Lenovo | Yes |  |
| Samsung | Yes |  |

Summary:

All companies are fine to add the NOTE to capture the agreement. And some companies suggest small update on the wording.

Proposal 6: Add the following NOTE for IAB node integration.

- NOTE: For OAM-based donor selection, if the IAB-node establishes NR-DC before the establishment of F1-C connection, the IAB-node indicates the F1-terminating donor node by signaling its IP address(es) to this donor node by using the RRC signaling.

In contribution [11], it’s proposed to capture the following RAN3 agreement for IAB-donor determination.

For donor-based IP-address allocation, the MN determines the F1-terminating node.

NOTE: If the IAB-node establishes NR-DC before the establishment of F1-C connection, the MN decides which of MN or SN becomes the F1-terminating IAB-donor. In case it decides that the SN become the F1-terminating IAB-donor, it signals via Xn to the SN to establish the backhaul connectivity (phases 2.1 and 2.2). The IAB-node can implicitly derive whether the MN or the SN is the F1-terminating donor, e.g., based on the entity which provides the default BAP configuration.

Q6-2: Do you agree to modify the NOTE above to capture the agreement for IAB node Integration?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | Yes | Also needs to be added to 38.423 |
| Ericsson | OK, but with modifications | NOTE: If the IAB-node establishes NR-DC before the establishment of F1-C connection, the MN decides whether the MN or the SN becomes the F1-terminating IAB-donor. In case it decides that the SN becomes the F1-terminating IAB-donor, it notifies the SN via Xn (Phases 2.1 and 2.2). The IAB-node can implicitly derive whether the MN or the SN is the F1-terminating donor, e.g., based on the entity which provides the default BAP configuration. |
| Huawei | ok | Prefer Ericsson’s version. |
| ZTE | No | We don't think an explicit indicator is needed to be sent to indicate SN to be the F1-terminating donor. If MN decides that SN become the F1-terminating donor, after receiving IP address request from IAB node via RRC, MN would send IAB transport migration management request message to SN, which includes IP address request info but doesn't include traffic to be offloaded information. After receiving this IAB transport migration management request message without traffic to be offloaded information, SN could derive implicitly that it should take the role of F1-terminating donor and provide default BAP configuration and allocated IP addresses to the IAB node. |
| Fujitsu | Yes |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| Samsung | Yes |  |

Summary:

(7/8) companies are fine to modify the NOTE to capture the agreement. And one company suggest small update on the wording which receives support from another 2 companies.

While (1/8) company think an explicit indicator is needed to be sent to indicate SN to be the F1-terminating donor. And this can be left to implementation via the IAB transport migration management procedure.

Since this issue has been parallelly handled by CB # IAB_03_CR38.423, we can add the modification above to the NOTE if it’s agreed in another CB.

Issues for IAB protocol stack

Q7: Do you agree to add the protocol stacks for CP-UP separation scenarios 1 and 2 [11]?

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | Yes | We have the stacks for F1-C routing via MeNB for ENDC. It would look a little awkward if we didn’t include those for CP-UP separation. |
| Ericsson | OK, but… | The first added figure is wrong, since it says ‘SgNB’ in the figure – it should say ‘MgNB’. |
| Huawei | ok | Agree with Ericsson, the typo of the first added figure should be corrected. |
| ZTE | Yes | Agree with Ericsson. |
| Fujitsu | Yes |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| Samsung | Yes |  |

Summary:

All companies are fine to add the protocol stacks for CP-UP separation scenarios 1 and 2.

Proposal 7: Add the protocol stacks for CP-UP separation scenarios 1 and 2 in section 6.1.4.

Others

Q8: Please provide view if any issue is missing in above discussion (except for the rewording and editorial correction in another draft CR).

| Company | Comment |
| --- | --- |
| Fujitsu | The IP address configuration is a little different to descendant node compared to the boundary node. The IP address configured for descendant nodes is anchored to IAB-donor-DUs in non-F1-terminating topology but it is configured by the BAP address of a IAB-donor-DU belonging to F1-terminating topology. Propose adding following text in step 6 of section 8.17.3.2: The RRCReconfiguration may include the new TNL address(es) anchored at the target IAB-donor-DU which are configured using a BAP address of source IAB-donor-CU topology. |
| Nokia | Do not understand the issue. What is “configured by the BAP address of a IAB-donor-DU belonging to F1-terminating topology.”? |
| Moderator | Also having same confusion on the “configured by the BAP address of a IAB-donor-DU belonging to F1-terminating topology.” |

Discussion – 2nd Round

The second round is to address the remaining open issues as discussed on the online session. Since in this round, we need to achieve some proposals which have the detail handling for the current specs. Therefore, companies are encouraged to provided comments with detail specs impacts rather than just say no.

Issue 1: GNB-DU CONFIGURATION UPDATE and SCTP Association

During the online session, we have agreed to add the following texts in the step 9 of the IAB inter-CU topology redundancy (section 8.17.2.1).

- The dual-connecting IAB-node sends F1AP gNB-DU CONFIGURATION UPDATE to the F1-terminating IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for offloaded F1-U traffic. If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the dual-connecting IAB-node and the F1-terminating IAB-donor-CU may be established using the new TNL address information of the dual-connecting IAB-node.

And it was remained to discuss that how can the actions above be applied to intra scenarios. After check with the procedures for all the intra or inter topology management scenarios. Moderator thinks that we may add the texts above in the following procedures with some wording update. And other procedures, e.g., BH RLF recovery procedure, can reuse the steps from topology adaptation procedure.

Scenario 1: Step 13 of intra-CU topology adaptation (section 8.2.3.1)

- If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the migrating IAB-node and the IAB-donor-CU may be established using the new TNL address information of the migrating IAB-node. The migrating IAB-node sends a F1AP gNB-DU CONFIGURATION UPDATE message to the IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for the F1-U traffic to be switched to the target path.

Scenario 2: Step 11 of intra-CU topology redundancy (section 8.2.4)

- If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the dual-connecting IAB-node and the IAB-donor-CU may be established using the new TNL address information of the dual-connecting IAB-node. The dual-connecting IAB-node sends a F1AP gNB-DU CONFIGURATION UPDATE message to the IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for the F1-U traffic to be switched to the target path.

Scenario 3: Step 15 of inter-CU topology adaptation (section 8.17.3.1)

- If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the migrating IAB-node and the F1-terminating IAB-donor-CU may be established using the new TNL address information of the migrating IAB-node. The migrating IAB-node sends a F1AP gNB-DU CONFIGURATION UPDATE message to the F1-terminating IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for the F1-U traffic to be switched to the target path.

Q1: Do you agree to add the texts above into the following scenarios for IAB intra or inter topology management procedure? If no, please feedback with your detail correction on the procedure.

1-1: Scenario 1

1-2: Scenario 2

1-3: Scenario 3

1-4: Other scenario(s)

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | 1-1: Yes 1-2: Yes 1-3: Yes |  |
| Fujitsu | 1-1: Yes 1-2: Yes 1-3: Yes |  |
| Samsung | Yes to 1-1~1-3 |  |
| Huawei | 1-1: Yes 1-2: Yes 1-3: Yes |  |
| ZTE | 1-1: Yes 1-2: Yes 1-3: Yes |  |
| Lenovo | 1-1: Yes 1-2: Yes 1-3: Yes |  |
| Ericsson | Yes, to all |  |
| Nokia | Yes with comments | For Scenario 1, the new text should be added in Step 12, rather step 13. Also, some rewording. The new SCTP should be established before sending the DU CONFIGURATION UPDATE message. |

Summary:

All companies are agreed to add the texts for gNB-DU CONFIGURATION UPDATE and SCTP association in 3 scenarios above. So, we can have the following proposals.

Proposal 1-1: Add the following texts in Step 12 of intra-CU topology adaptation (section 8.2.3.1)

- If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the migrating IAB-node and the IAB-donor-CU may be established using the new TNL address information of the migrating IAB-node. The migrating IAB-node sends a F1AP gNB-DU CONFIGURATION UPDATE message to the IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for the F1-U traffic to be switched to the target path.

Proposal 1-2: Add the following texts in Step 11 of intra-CU topology redundancy (section 8.2.4)

- If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the dual-connecting IAB-node and the IAB-donor-CU may be established using the new TNL address information of the dual-connecting IAB-node. The dual-connecting IAB-node sends a F1AP gNB-DU CONFIGURATION UPDATE message to the IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for the F1-U traffic to be switched to the target path.

Proposal 1-3: Add the following texts in Step 15 of inter-CU topology adaptation (section 8.17.3.1)

- If new TNL addresses for F1-C traffic are configured, new SCTP association(s) between the migrating IAB-node and the F1-terminating IAB-donor-CU may be established using the new TNL address information of the migrating IAB-node. The migrating IAB-node sends a F1AP gNB-DU CONFIGURATION UPDATE message to the F1-terminating IAB-donor-CU, which may include new (outer) IP addresses and corresponding new (inner) IP address for the F1-U traffic to be switched to the target path.

Proposal 1-4: Adjust the sequence between gNB-DU CONFIGURATION UPDATE and SCTP association in Step 9 of section 8.17.2.1.

Issue 2: Traffic Revoking for Inter-CU Topology Adaptation

Following proposal was achieved in the Phase 1 discussion.

Proposal 2-3: Remove the following texts.

- The non-F1-terminating IAB-donor-CU can trigger the full traffic revoking by executing the XnAP Handover Preparation procedure for the migrating IAB-MT towards the F1-terminating IAB-donor-CU.

Based on the comments from Ericsson, we may need to have further update as below.

The traffic offloaded through the inter-CU topology adaptation described in steps 1 to 20 (including the offloaded traffic pertaining to the migrating IAB-node and its descendant IAB-nodes and their served UEs) can be fully revoked. If the full revoking is initiated by the non-F1-terminating IAB-donor-CU, the migrating IAB-MT is handed over in reverse direction, i.e., from the non-F1-terminating IAB-donor-CU to the F1-terminating IAB-donor-CU, and the traffic of the migrating IAB-DU and the descendant IAB-DUs is routed again along the former source path. The non-F1-terminating IAB-donor-CU can initiate the full revoking of traffic offload by executing the XnAP Handover Preparation procedure for the migrating IAB-MT towards the F1-terminating IAB-donor-CU. The F1-terminating IAB-donor-CU can initiate the full revoking of traffic offload by requesting the release of all offloaded traffic from the non-F1-terminating IAB-donor-CU through the IAB TRANSPORT MIGRATION MANAGEMENT REQUEST from the non-F1-terminating IAB-donor-CU. This messages may trigger the XnAP Handover Preparation procedure for the migrating IAB-MT towards the F1-terminating IAB-donor CU.

Q2: Do you agree to make following corrections for traffic revoking of Inter-CU Topology Adaptation procedure in section 8.17.3.1? If no, please feedback with your detail correction on the procedure.

2-1: Remove the following texts.

- The non-F1-terminating IAB-donor-CU can trigger the full traffic revoking by executing the XnAP Handover Preparation procedure for the migrating IAB-MT towards the F1-terminating IAB-donor-CU.

2-2: Replace “In this case” with “If the full revoking is initiated by the non-F1-terminating IAB-donor-CU”

2-3: Combine the two paragraphs.

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | 2-1 Yes 2-2 Yes | The phrase of 2-2 replaces the phrase of 2-1. We obviously do not need both phrases since they carry the same information. This covers the revocation initiated by CU2. The revocation initiated by CU1 follows after that. All of this make sense. |
| Fujitsu | 2-3 | 2-1 is not feasible, deleting the first sentence of the second paragraph causes loss of meaning. The first sentence of the second paragraph means the full traffic revoking initiated by the non-F1-terminating CU relies on the handover from non-F1-terminating CU to F1-terminating CU, i.e., no other Xn message is needed for revoking. But the first paragraph only says the IAB-MT is handed over from non-F1-terminating CU to F1-terminating CU when full revoking happens. It’s not clear whether the full traffic revoking relies on handover message or not. 2-2 is similar with 2-1. We suggest combining the first and second paragraph. 2-3: The traffic offloaded through the inter-CU topology adaptation described in steps 1 to 20 (including the offloaded traffic pertaining to the migrating IAB-node and its descendant IAB-nodes and their served UEs) can be fully revoked. The non-F1-terminating IAB-donor-CU can initiate the full traffic revoking by executing the XnAP Handover Preparation procedure for the migrating IAB-MT. After the migrating IAB-MT is handed over in reverse direction, i.e., from the non-F1-terminating IAB-donor-CU to the F1-terminating IAB-donor-CU, the traffic of the migrating IAB-DU and the descendant IAB-DUs is routed again along the former source path. |
| Samsung | Yes |  |
| Huawei | YES |  |
| ZTE | No | 2-1: this sentence describes how the non-F1-terminating IAB-donor-CU triggers the full traffic revoking. 2-2: the migrating IAB-MT is handed over in reverse direction when the full revoking is initiated by the F1-terminating IAB-donor-CU as well. |
| Lenovo | 2-1 Yes 2-2 Yes |  |
| Ericsson | 2-3 with a modification | Fujitsu rewording says that the traffic is revoked, but, in fact, it is the traffic offloading that is revoked. So, the term ‘full revoking of traffic offload’ should be used. |
| Nokia |  | Frankly, we do not see much difference to the existing text. |

Summary:

Based on the comments above, (4/8) companies are fine with the corrections 2-1 and 2-2. And (2/8) companies stand for 2-3. Moderator thinks 2-3 can cover 2-1&2-2 and it may solve the concern from ZTE for 2-1. And the concern from ZTE for 2-2, a new paragraph is proposed by moderator by the case of revoking initiated by non-F1-terminating IAB-donor-CU.

Proposal 2: Make following corrections for revoking in Section 8.17.3.1.

- Replace “In this case” with “The non-F1-terminating IAB-donor-CU can initiate the full revoking of traffic offload by executing the XnAP Handover Preparation procedure for the migrating IAB-MT. After” and start with a new paragraph

- Delete “and” in “and the traffic of the migrating IAB-DU…”

- Remove the following texts “The non-F1-terminating IAB-donor-CU can trigger the full traffic revoking by executing the XnAP Handover Preparation procedure for the migrating IAB-MT towards the F1-terminating IAB-donor-CU.”

Issue 3: Stage-2 handling of Proposal 2-4

Following proposal has been agreed in the Phase 1 discussion for Inter-CU topology adaptation.

2-4: Correct the description in step 15 and step 20 in IAB inter-CU topology adaptation procedure in section 8.17.3.1 to clarify how could source CU obtain the new IP addresses and transfer the new IP addresses to the target CU.

And it was remained to discuss that how exactly to correct the description based on the agreement. Based on the comments received from the ZTE and Nokia, we may need to have further update as below.

15. The F1-C connection between the migrating IAB-node and the source IAB-donor-CU may be switched to the target path using the new TNL address information of the migrating IAB-node. The migrating IAB-node may report the new TNL address information it wants to use for eachF1-U traffic tunnel traffic and non-UP traffic type to the source IAB-donor-CU via the gNB-DU CONFIGURATION UPDATE message. 20. Repetition of steps 16 to 19, as needed, where the source IAB-donor-CU can request addition, modification or release of the context of the traffic to be offloaded, e.g. QoS information and/or DL TNL address for non-UP and UP traffic. The target IAB-donor-CU can fully or partially reject addition or modification requests by the source IAB-donor-CU.

Q3: Do you agree to make following corrections for Inter-CU Topology Adaptation procedure? If no, please feedback with your detail correction on the procedure.

3-1: Replace “each F1-U tunnel and non-UP traffic type” with “F1-U traffic” in Step 15

3-2: Update Step 20 as follow

- 20. Repetition of steps 16 to 19, as needed, where the source IAB-donor-CU can request addition, modification or release of the context of the traffic to be offloaded, e.g. QoS information and/or DL TNL address for non-UP and UP traffic. The target IAB-donor-CU can fully or partially reject addition or modification requests by the source IAB-donor-CU.

| Company | Yes/No | Comment |
| --- | --- | --- |
| QCOM | 3-1: Yes 3-2: No | 3-2: While step 20 refers to a repetition of prior steps, it should not repeat the prior steps. We therefore should simplify: 20. Repetition of steps 16 to 19, as needed, where the source IAB-donor-CU can request the offload of further traffic, or the modification or release of offloaded traffic. The target IAB-donor-CU can fully or partially reject addition or modification requests by the source IAB-donor-CU. |
| Fujitsu | 3-1: Yes 3-2: No | For 3-2, agree with QC’s modification. |
| Samsung | 3-1: Yes 3-2: prefer to QC’s version |  |
| Huawei | 3-1:Yes 3-2: QC’s revision is ok |  |
| ZTE | Yes | QC’s revision looks good. |
| Lenovo | 3-1: Yes 3-2: agree with QC’s version |  |
| Ericsson | 3-1: Yes 3-2: Yes, to QC rewording |  |
| Nokia | Yes | QC text is better. |

Summary:

For 3-1, all companies are fine.

For 3-2, all companies are fine with the version from QC.

Proposal 3: Make following corrections for inter-CU topology adaptation procedure in section 8.17.3.1

- Replace “each F1-U tunnel and non-UP traffic type” with “F1-U traffic” in Step 15

- Update Step 20 as “Repetition of steps 16 to 19, as needed, where the source IAB-donor-CU can request the offload of further traffic, or the modification or release of offloaded traffic. The target IAB-donor-CU can fully or partially reject addition or modification requests by the source IAB-donor-CU.”

Issue 4: Parallel TNL Migration of the Descendant Nodes

For this issue, it has been well discussed in Phase I and in the online session, there are two camps that support two different alternatives.

- Alt #1: The trigger condition for the descendent IAB node to transfer the buffered RRC message to its child IAB node is that the descendent IAB-node has one or more routing entries pre-configured from IAB-donor-CU for the target path.

  - For Alt #1, Following texts needs to be added in TS 38.401

    - The IAB-donor-CU also configures the migrating IAB-node and descendant IAB node(s) with routing entries, while the migrating IAB-MT is still connected with source parent node.

  - And following trigger condition needs to be updated in section 8.3.4.2 of TS 38.473

    - The descendent IAB-node has one or more routing entries pre-configured from IAB-donor-CU for the target path.

- Alt #2 (Proposal 3): In case no routing entry exists for the received packets, use the default BH RLC CH to forward the TNL migration related UL packets received from the child node.

  - For Alt #2, there is no stage-2 impact for RAN3

  - And we may have some BAP impacts and send an LS to RAN2 with the Proposal 3

Q4: Please share your preference on Alt #1 or Alt #2. And provide your comments on the detail correction/action on Alt #1/2.

| Company | Alt #1/#2 | Comment |
| --- | --- | --- |
| QCOM | See comment | Alt#2: define a new BAP behavior where a default BAP routing ID is used as a routing entry. Such a change would be in RAN2 scope. Alt#1: Text addition: The above text is NOT sufficient, and it does NOT integrate with the existing text. We propose the following change to the current spec (addition in bold): If needed, the IAB-donor-CU configures BH RLC channels, BAP-sublayer routing entries on the target path for the descendant nodes and the BH RLC channel mappings on the descendant nodes in the same manner as described for the migrating IAB-node in step 11. … Based on implementation, these steps can be performed after or in parallel with the handover of the migrating IAB-node. To enable performing these steps in parallel, the IAB-donor-CU sends … as specified in TS 38.473 [4]. Also, the configuration of BH RLC channels and BAP sublayer routing on the target path and the descendent nodes needs to be configured while the migrating IAB-MT is still connected with the source parent node.. Alt#1: New condition: - The descendent IAB-node has one or more routing entries pre-configured from IAB-donor-CU for the target path. This does not work for the following reasons: 1) The descendent node does NOT know if a BAP routing entry is for the source path or the target path (only the migrating node could tell). 2) If source and target path have access to the SAME IAB-donor-DU, the descendent node does NOT need a new routing entry. In this case, it would NOT release the buffered RRC message. The CU may still send the RRC Reconfiguration to the child, e.g., if the target path connects to two IAB-donor-DUs, the old one and a new one, and the CU decided to offload SOME of the F1-U tunnels to the new IAB-donor-DU. Summary: With the added text above, the CU will configure the target path before IAB-node migration, and NO NEW condition is needed. |
| Fujitsu | Alt#2 | Alt.2 has the advantage that the IAB-donor-CU doesn’t have to configure the migrating node and descendant nodes with multiple routing entries before the parallel migration which will cause much delay for the migration. When there is no routing entry available during the parallel migration, the default configuration can be used to forward the on-the-fly UL traffic. The transmission is temporary since multiple routing entries can be configured right after migration. According to present 38.340, the default configuration can be used to deliver the BAP data from upper layer when the routing configuration is not reconfigured. It’s not necessary to preclude using the default configuration to deliver the data received from child nodes. The spec correction for BAP corresponding to Alt#2 is expected in Routing section. We should send LS to RAN2 for Alt#2. |
| Samsung |  | We disagree to add “Also, the configuration of BH RLC channels and BAP sublayer routing on the target path and the descendent nodes needs to be configured while the migrating IAB-MT is still connected with the source parent node.” This may be possible if the migration is intended for load balance. However, for migration due to sudden signal degradation, the way proposed by QC may not be possible. Thus, new Alt#1 does not completely resolve issue in all scenarios. We prefer to Alt#2, and fine to send LS to RAN2. |
| Huawei | See comments | We slightly prefer Alt 2. For Alt 1, it relies on the pre-configuration from IAB-donor-CU, then it seems that more time is needed for the handover preparation of the migrating IAB-MT. So we have sympathy with Samsung that such pre-configuration may be possible if the migrating is for traffic offloading, but may not always possible if the migrating is due to the radio link quality deterioration. Suggest the following modification for Alt #2: Alt #2 (Proposal 3): In case no routing entry exists for the received packets, use the default BH RLC CH and the corresponding parent node to forward the TNL migration related UL packets received from the child node. Alt 2 will introduce impact on RAN2 BAP specification, and should be notify RAN2 if RAN3 select Alt 2. |
| ZTE | See comments | Alt 2 is not clear. The issue is about how to route the TNL migration related UL packets, not about BH RLC CH mapping. So it should be “use the default BAP routing ID” instead of “use the default BH RLC CH”. And we prefer Alt #2, i.e. use the default BAP routing ID since we think configuration of routing entry before the migration of the migrating MT would lead to additional service interruption during the migration procedure. And in some scenarios, the donor CU cannot be able to configure the migrating IAB-node and descendant IAB node(s) with routing entry before the migration of the migrating MT, e.g. when the radio condition deteriorates rapidly. |
| Lenovo | Alt#2 | Slightly prefer Alt 2 and send the LS to RAN2 with the P3. |
| Ericsson | #2 | This is exactly the point with introducing the default BRID. |
| Nokia | #1 | Agree with QC. If no time to do the configuration via source path, then CU may also have no time to send the RRC via source path. Alt #2 will be a fundamental change to RAN2, since the BH RLC CH is used for traffic mapping, rather routing. Even with default BRID, the BAP packet can still be dropped. |

Summary:

Most of the companies (6 of 8) support Alt.2. While for the P3, different companies have some divergences on the default configuration. But moderator thinks the detail BAP behavior can be discussed by RAN2, and we only need to inform RAN2 that Alt.2 is selected from RAN3 point of view.

Proposal 4: Send an LS to RAN2 informing that Alt.2 is selected by RAN3 for parallel TNL migration of the descendant nodes in IAB intra-CU topology adaptation.

- Alt. 2: In case no routing entry exists for the received packets, use the default configuration, e.g., default BH RLC CH to forward the TNL migration related UL packets received from the child node.

Issue 5: IP Address Request for IAB Inter-CU Topology Redundancy

As discussed in Phase I, companies had some divergence on the IP address request for IAB inter-CU topology redundancy procedure. Based on the comments from all companies. Moderator thinks we may have the following 3 options to realize the IP address request for the boundary IAB-node.

Option 1: Via legacy RRC signaling

- For this option, the sentence “This message may include a request for new TNL address(es).” needs to be removed from Step 2.

- Add the clarification in Step 3 with “The non-F1-terminating IAB-donor-CU can proactively obtain the new TNL address based on the IP address request from dual-connecting IAB-node via RRC”

Option 2: Perform in Step 1 as proposed in [3] via NR-DC establishment procedure or NR-DC modification procedure with following corrections

1. The NR-DC establishment procedure is performed for the IAB-MT of the dual-connecting IAB-node as described in TS 37.340 [12], clause 10.2. Or NR-DC modification procedure is performed for the IAB-MT of the dual-connecting IAB-node as described in TS 37.340 [12], clause 10.3. The F1-terminating IAB-donor-CU may include the dual-connecting IAB-node’s TNL address information in the RRC container of the XnAP message for NR-DC to the non-F1-terminating IAB-donor-CU. The non-F1-terminating IAB-donor-CU provides an RRCReconfiguration message which is included as part of the XnAP response message to the F1-terminating node, or the non-F1-terminating IAB-donor-CU deliveres the RRCReconfiguration message to the dual-connecting IAB-node. The RRC configuration includes a BAP address for the dual-connecting IAB-node in the non-F1-terminating IAB-donor-CU’s topology. The RRC configuration may include new TNL address(es) anchored at the second-path IAB-donor-DU for the dual-connecting IAB-node. In case IPsec tunnel mode is used to protect the F1 and non-F1 traffic, the new TNL address is outer IP address. 2. The F1-terminating IAB-donor-CU sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the non-F1-terminating IAB-donor-CU to provide the context of the traffic to be offloaded.

Option 3: Perform in Step 2 via IAB TRANSPORT MIGRATION MANAGEMENT procedure

1. The NR-DC establishment procedure is performed for the IAB-MT of the dual-connecting IAB-node as described in TS 37.340 [12], clause 10.2. 2. The F1-terminating IAB-donor-CU sends an IAB TRANSPORT MIGRATION MANAGEMENT REQUEST message to the non-F1-terminating IAB-donor-CU to provide the context of the traffic to be offloaded. This message may include a request for new TNL address(es). 3. The non-F1-terminating IAB-donor-CU sends a DL RRC MESSAGE TRANSFER message to the second parent IAB-node’s IAB-DU, which includes an RRCReconfiguration message for the dual-connecting IAB-node’s IAB-MT. The RRC configuration includes a BAP address for the boundary node in the non-F1-terminating IAB-donor-CU’s topology. The RRC configuration may include new TNL address(es) anchored at the second-path IAB-donor-DU for the dual-connecting IAB-node. In case IPsec tunnel mode is used to protect the F1 and non-F1 traffic, the new TNL address is outer IP address. 4. The second parent IAB-DU forwards the received RRCReconfiguration message to the dual-connecting IAB-MT. 5. The dual-connecting IAB-MT responds to the second parent IAB-node’s IAB-DU with an RRCReconfigurationComplete message. 6. The second parent IAB-node’s IAB-DU sends an UL RRC MESSAGE TRANSFER message to the non-F1-terminating IAB-donor-CU to convey the received RRCReconfigurationComplete message. 7. The non-F1-terminating IAB-donor-CU may configure or modify BH RLC channels and BAP-sublayer routing entries on the second path between the dual-connecting IAB-node and the second-path IAB-donor-DU as well as DL mappings on the second-path IAB-donor-DU for the dual-connecting IAB-node’s second path. The DL mappings may be based on the TNL address(es) allocated to the dual-connecting IAB-node in step 3. These configurations may support the transport of UP and non-UP traffic on the second path. 8. The non-F1-terminating IAB-donor-CU responds with an IAB TRANSPORT MIGRATION MANAGEMENT RESPONSE message to the F1-terminating IAB-donor-CU to provide the mapping information for the traffic to be offloaded as indicated in step 2. The message includes the L2 info necessary to configure the dual-connecting IAB-node with the UL mappings for this traffic. The message includes the DSCP/IPv6 Flow Label values to be used for the DL traffic to be offloaded. The message includes new TNL address(es) anchored at the second-path IAB-donor-DU for the dual-connecting IAB-node. In case IPsec tunnel mode is used to protect the F1 and non-F1 traffic, the new TNL address is outer IP address.

Q5: Please share your preference on the 3 options for IP address request for the boundary IAB-node during IAB inter-CU topology redundancy procedure. And provide your comments on the detail correction/action on the selected Option.

| Company | Option 1/2/3 | Comment |
| --- | --- | --- |
| QCOM | Option 1 ONLY | RAN3 agreed: For IP address addition, non-F1-terminating CU to configure IP addresses on the boundary node via Rel-16 RRC signalling, and boundary node reports the F1-U IP addresses it wants to use via Rel-16 F1AP signalling to the F1-terminating CU. This is option 1. Based on this agreement, Step 2 needs to be corrected. Option 2 and 3 do not comply with this RAN3 agreement. We are past functional freeze. There is no discussion on functional changes unless they are necessary corrections, which does not apply to options 2 and 3. |
| Fujitsu | Option 2 | RAN3 has not addressed the issue. The agreement for option 1 was made in former meeting. In the last meeting, RAN3 has agreed to further study the 3 options: Options for the request are either Step 1 or Step 2 above or via RRC. Details on how they are requested are FFS. The current spec for IP addresses request of the boundary node is based on Option 3. Problem in Option 3: Given XnAP transport migration management procedure is used for IP addresses request of the boundary node, non-F1-terminating CU will not know whether the IP addresses request is for boundary node or descendant nodes. Step 3 to Step 6 in current text are special for the boundary node. However, if non-F1-terminating CU cannot derive the request message is for boundary node during Step 2, it cannot initiate to perform Step 3, i.e., delivering the DL RRCReconfiguration with IP address allocation and BAP address to boundary node. Problem in Option 1: The boundary node doesn’t know whether or which traffic is to be migrated towards the non-F1-terminting topology. Thus, it doesn’t know whether to request IP address or how many IP addresses should be requested in the non-F1-terminating topology. Option 2: Option 2 can overcome the drawbacks of option 1 and option 3. The CU of F1-terminating donor (MN/SN) knows which traffic to be migrated and how many IP addresses are needed for the migrated traffic. It’s feasible for F1-terminating CU to initiates the IP address request for the boundary node. Based on Option2, the IP address request/allocation is via RRC container during NR-DC establishment procedure or NR-DC modification procedure, the non-F1-terminating CU will know the request is for the boundary node. It can determine how to configure the boundary node with IP address allocation and BAP address properly depending on the exact DC scenario. That is, it can deliver the configuration via RRC message on SRB3 or on SRB1, or it can deliver the configuration via RRC container in XnAP message to the F1-terminating CU, e.g., via RRC container in S-Node Addition Response message when SN is the non-F1-terminating donor and SRB3 is not set up. With Option 2, Step 3-Step 6 of current spec are not needed, as they are combined with the NR-DC procedure. With the reasons above, Option 2 is the preferred solution. General correction based on Option 2 is provided in [3], some improvement may be necessary as needed. |
| Samsung | See comments | The current IP address request procedure does not differentiate boundary node and descendant node. Thus, the non-F1 donor cannot determine the trigger of step3~6 based on IP address request. To resolve this issue, a simple solution is to add indication in IAB TNL Address Request IE to indicate whether the requested IP address contain the one for boundary node or not. |
| Huawei | Option 1 | Option 1 has minimum specification impact to solve the issue. |
| ZTE | Option 2 | The problem in option1: the IAB node doesn’t know how many IP addresses are needed for the redundancy path since the traffic offload is determined by the F1-terminating donor. The problem in option 3: in this option, the new IP addresses needs to be sent to the IAB node from the F1-terminating donor via RRC. In this case, the IAB node cannot differentiate which the IP addresses are for F1-terminating donor topology and which are for non F1-terminating donor topology if the two donor DUs have the same BAP address. |
| Lenovo | Option 1 | I think all the options may work for IP address of boundary IAB-node. However, Option 1 is preferred with minimum impacts. |
| Ericsson | Option 1 |  |
| Nokia | Option 1 |  |

Summary:

(5/8) companies share their support on Option 1.

(2/8) companies support Option2. For option 1, they think the boundary IAB-node doesn’t know how many IP addresses are needed for the redundancy path since the traffic offload is determined by the F1-terminating donor. From moderator’s view, maybe Option 2 is a preferred solution than Option 1 when considering the number of the requested IP address.

(1/8) company support Option 3 and one indication is proposed to be added in IAB TNL Address Request IE to indicate whether the requested IP address contain the one for boundary node or not. However, this can be indicated implicitly by the DL F1 Term BH Info and UL F1 Term BH Info in the F1-terminating BH information list included in the IAB TM Management Request. Anyway, IP address request for the boundary IAB-node via the IAB transport migration management procedure is also workable.

Based on the clarification above, since all 3 options are feasible, then we may propose:

Proposal 5-1: Support Option 1 (via RRC signaling) for IP address request of the boundary IAB-node in IAB inter-CU topology redundancy procedure, and make following corrections in section 8.17.2.1.

- Remove “This message may include a request for new TNL address(es).”in Step 2.

- Add the clarification in Step 3 with “The non-F1-terminating IAB-donor-CU can proactively obtain the new TNL address based on the IP address request from dual-connecting IAB-node via RRC”

Proposal 5-2: Add the following NOTE in step 3 in section 8.17.2.1 to clarify that IP address can be requested via Option 2 or Option 3.

- NOTE: The new TNL address of the dual-connecting IAB-node may be also requested via the NR-DC establishment procedure in Step 1 or via the IAB TRANSPORT MIGRATION MANAGEMENT procedure in Step 2.

- Issue 6: Others

| Company |  | Comment |
| --- | --- | --- |
|  |  | We propose to remove the following two sentences: ### 8.17.1 IAB Inter-donor-DU Re-routing … In the intra-donor-CU scenario for UL inter-donor-DU rerouting, the IAB-donor-DU and its peer IAB-donor-DU are controlled by the same IAB-donor-CU. In the inter-donor-CU case, the IAB-donor-DU and its peer IAB-donor-DU are controlled by different IAB-donor-CUs. …. We never defined „intra-donor-CU scenario” and “inter-donor-CU case” in the context inter-donor-DU rerouting. If you want to add anything like this, PLEASE define the terminology! |
| Huawei | About the above issue regarding to the inter-donor-DU re-routing, suggest to replace the sentence by “the IAB-donor-DU and the peer IAB-donor-DU may controlled by same IAB-donor CU or two different IAB-donor-CUs”, then we do not need to introduce the intra/inter-donor-CU scenario. |
| ZTE | We are fine with Huawei’s revision with the following change: the IAB-donor-DU and the peer IAB-donor-DU may be controlled by the same IAB-donor CU or two different IAB-donor-CUs |
| Lenovo | Agree with texts by Huawei and ZTE. |
| Ericsson | ZTE revision of Huawei text is ok + ‘by’ shall be added after the ‘or’. |
| Nokia | Ok for ZTE text. |

Summary:

For the issue raised by QC in the IAB inter-DU re-routing. The wording improvement by Huawei and ZTE and Ericsson are supported by all other companies.

Proposal 6:

In section 8.17.1, replace “In the intra-donor-CU scenario for UL inter-donor-DU rerouting … controlled by different IAB-donor-CUs” with “The IAB-donor-DU and the peer IAB-donor-DU may be controlled by the same IAB-donor-CU or by two different IAB-donor-CUs”

Discussion – 3rd Round

Based on the online session discussion, we still have the following issue to be discussed in phase-III.

Try to converge on a final LS - The LS should ask RAN2 if Alt2 can be supported and whether it requires enhancements to current specifications - Converge on whether Alt1 can already be supported or if it should be included in the LS to RAN2 - If the LS is not agreeable, the discussion is closed To be continued...

Since the technical discussion has been already in-depth discussed during phase-I and phase-II. In this round, we aim to converge on a final LS.

Q1-1: Do you agree that Alt1 can already be supported?

Q1-2: If not for Q1-1, do you agree that Alt1 should be included in the LS to RAN2?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Qualcomm | Q1-1: YES Q1-2: YES | Q1-1: This should not be question. This is Rel-16 IAB. The CU can configure the routing entry to the target donor-DU via F1AP before the migration. The routing entry can co-exist with the routing entry to the to the source donor-DU. This should not even be discussed. Q1-2: The LS should describe the difference between ALT1 and ALT2 and state that ALT1 is already supported. |
| Nokia | Yes for both | Agree with QC. The only argument against Alt 1 is there may be no time for CU to perform the routing configuration via source path before migration. Actually this configuration can be performed well before the actual migration. So this is not an issue. If an implementation do choose to only perform the configuration just before the migration, then it is an implementation issue, and have to accept the fact that the parallel TNL migration may not work. |
| ZTE | Q1-1: NO Q1-2: YES | We don’t think alt 1 can already be supported. In alt 1, routing entries for target path are configured before receiving default BAP configurations for target path. The problem is: routing entries for target path cannot be used anymore and should be released after receiving default BAP configuration. The reasons are listed as below: - It has been discussed whether to keep source path at migrated IAB node and its descendant node(s) to avoid unnecessary data transmission after migration during RAN3#113e and RAN3#113e meeting, but it was not agreed. Instead, we have the agreements: In Rel-17, the following aspects aiming at avoiding unnecessary DL transmissions will not be specified: - the avoidance of unnecessary DL data transmission over the source path between IAB donor CU and migrating IAB node - the transmission of in-flight DL packets buffered at migrating IAB node and its descendant node(s), after the migration No further enhancements for the avoidance of unnecessary UL transmissions, other than local UL rerouting, are specified in Rel-17. - In the intra-CU topology adaptation procedure in 8.2.3.1 in TS 38.401, there is no step for release of routing entries for source path at migrating/descendant nodes, it implies that the routing entries for source path at migrating/descendant nodes are released after receiving RRCreconfiguration message with default BAP configurations. - It’s true that in-flight packets could be re-routed via target path, but in our understanding, these in-flight packets could be routed via descendant nodes only before receiving RRCreconfiguration message with default BAP configurations. |
| Huawei | Q1-1: NO Q1-2: YES | We are not convinced that the Alt 1 has been supported since Rel-16 IAB, there is no such agreement to indicate that the IAB-donor-CU will provide routing entries related to target path before the topology update to the IAB-node. However, we admit that the donor CU may do such pre-configuration by implementation if possible. So, in our view, Alt 1 can be supported by CU implementation. And we should include the alt 1 in the LS to RAN2. |
| Ericsson | Q1-1: Yes, by implementation Q1-2: Yes, for RAN2 to get the full picture |  |

Summary:

Since companies still share different views on alt1 and alt2 based on the comments received yet. Moderator thinks we may need to converge on the LS in R3-224061 drafted based on the comments from all the companies, and we can discuss on the session to decide whether to send the LS to RAN2.

We need to motivate to RAN2 why RAN3 is considering Alt2 given that Alt1 is already available.

Q2: Why does RAN3 consider Alt2 over Alt1? What would be the potential benefit of Alt2 if any?

| Company | Comment |
| --- | --- |
| Qualcomm | Alt 1 requires: - RRC configuration of IP addresses and default BAP routing ID on descendent nodes before migration. - F1AP configuration of a routing entry on descendent nodes with child-node(s) before migration. Alt 2 requires: - RRC configuration of IP addresses and default BAP routing ID on descendent nodes before migration. - F1AP configuration of a routing entry on descendent nodes with child-node(s) after migration. Some companies believe that intra-donor migration via Alt2 can be improved, e.g., in case of imminent BH RLF on the source path, since the F1AP configuration of the descendent nodes with child node(s) can be conducted after rather than before migration Other companies believe that there is no benefit since the CU can preconfigure the routing entries a long time before the source path shows any signs of weakness. Also, ALT2 does not help to improve on the target path route configuration, which also needs to be performed before migration. |
| Nokia | We do not think the need to send a LS to RAN2. We also do not see the benefit for Alt 2 vs. Alt 1. If a LS is to be sent, then it should capture Alt 1, and describe why Alt 2 is needed (since Alt 1 can be supported without RAN2 changes, and Alt 2 requires RAN2 changes and not align with RAN2 principle on routing) |
| ZTE | As commented in Q1, we think alt 1 is not available currently. This is the main reason why we consider alt 2. Even if alt 1 could be supported, we think alt 2 could be beneficial to reduce service interruption during migration. |
| Huawei | As we commented in previous Q1-1, the Alt 1 can be achieved by CU implementation, but it may not always possible, e.g. if the topology update is due to the link deterioration in a short period. The concurrent TNL migration has been specified in Rel-17, we are not sure it is a good way to rely on an implementation-based solution (Alt 1) to support the concurrent TNL migration for the descendant nodes. |

Summary:

Since companies still share different views on alt1 and alt2 based on the comments received yet. Moderator thinks we may need to converge on the LS in R3-224061 drafted based on the comments from all the companies, and we can discuss on the session to decide whether to send the LS to RAN2.

Draft overall description is provided below for the potential LS (if needed) for both Alt1 (if needed) and Alt2.

RAN3 has agreed with the following agreements to achieve parallel TNL migration of the descendant IAB-nodes in IAB intra-CU topology adaptation. The condition for the descendant node to send the buffered RRC message to its child node is: Upon a descendant IAB-MT receiving the RRC reconfiguration for its own intra-donor migration (e.g., including the new IP address(es) without PCI change).. To avoid the parent node (e.g. the child IAB node of the migrating node) to discard the UL BAP PDU using the default UL Routing ID received from the descendant IAB node, the routing configuration for the default UL Routing ID should be available. . RAN3 discussed the following two alternatives: - Alt. 1: the IAB-donor-CU pre-configures the BH Routing Configurations before sending the RRCReconfiguration message to each descendant IAB-node - Alt. 2: In case the BH Routing Configuration has not been (re)configured by F1AP, the descendant IAB-node of the migrating IAB node use the default configuration, e.g., default BH RLC CH, to forward the TNL migration related UL packets received from the child node, to the parent node corresponding to the default configuration. RAN3 would like to ask RAN2 to provide feedback, e.g, whether Alt 2 can be supported based on current specification, and if not, whether it can be supported in Rel-17?

Q2: Comments are welcomed for the wording of the draft LS, if needed?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Qualcomm |  | The wording above is unacceptable. We propose: - The LS should not capture the RAN3 agreement since it is completely incomprehensible if taken out of context. - Instead, the LS should capture that Rel-17 supports an enhancement to intra-donor migration, where the descendent nodes are configured via the source path so that the traffic migration of migrating- and descendent IAB-nodes can occur in parallel. This is already in the 17.0.0 spec, so we can readily refer to it. - The two alternatives should be described as follows: RAN3 has considered two alternatives for the configuration of descendent nodes with child node(s): Alt 1: - The CU configures IP addresses and default BAP routing ID on each descendent node via RRC before migration. - The CU configures a routing entry on each descendent node with child-node(s) via F1AP before migration. - The descendent node performs routing based on the routing entries configured. Alt 2: - The CU configures IP addresses and default BAP routing ID on each descendent node via RRC before migration. - The CU configures a routing entry on each descendent node with child-node(s) via F1AP after migration. - The descendent node uses the default BAP routing ID for the routing of UL traffic until it receives the routing configuration via F1AP. - The LS should capture that ALT1 is readily available. - The LS should capture the reason why ALT2 is considered given that ALT1 is readily available (see QCOM reply to Q2). - The LS should capture the following question: RAN3 would like to know if for the support of Alt2, the default BAP routing ID can be used by the IAB-node for the routing of UL traffic from a child node, and if this routing behavior would require specification changes. |
| Nokia |  | Please refer to our modifications above. We agree with QC, and some modifications to the Alt ½ description from QC: Alt 1: - The CU configures IP addresses and default BAP routing ID on each descendent node via RRC before migration. - The CU configures a routing entry on each descendent node with child-node(s) via F1AP before migration. - After migration, the parent node of the descendent node performs routing based on the routing entries configured. Alt 2: - The CU configures IP addresses and default BAP routing ID on each descendent node via RRC before migration. - After migration, the parent node of the descendent node performs the routing without a match of the routing ID or BAP address in the routing configuration, i.e. using the default BAP routing ID, for the routing of UL traffic until it receives the routing configuration via F1AP. - The CU configures a routing entry on each descendent node with child-node(s) via F1AP after migration. |
| ZTE |  | - On whether alt 1 is available currently, please refer to our comment to Q1. - We suggest the following revision: Alt 1: - The CU configures IP addresses and default BAP routing ID on each descendent node via RRC before migration. - The CU configures a routing entry on each descendent node with child-node(s) via F1AP before migration. - After migration, the parent node of the descendent node performs routing based on the routing entries for target path configured before receiving default BAP configurations. Alt 2: - The CU configures IP addresses and default BAP routing ID on each descendent node via RRC before migration. - After migration, the parent node of the descendent node performs the routing without a match of the routing ID or BAP address in the routing configuration, i.e. using the default BH RLC CH to forward ,UL traffic received from child nodes to the parent node corresponding to the default configuration until it receives the routing configuration via F1AP. - The CU configures a routing entry on each descendent node with child-node(s) via F1AP after migration. |
| Huawei |  | Suggest to Alt 1: - The CU configures IP addresses and default BAP routing ID on each descendent node via RRC before migration. - The CU configures a routing entry on each descendent node with child-node(s) via F1AP before migration. - After migration, the descendent node performs routing based on the routing entries for target path configured before receiving default BAP configurations. Alt 2: - The CU configures IP addresses and default BAP routing ID on each descendent node via RRC before migration. - After migration, the parent node of the descendent node performs the routing without a match of the routing ID or BAP address in the routing configuration, i.e. using the default BH RLC CH to forward ,UL traffic received from child nodes to the parent node corresponding to the default configuration until it receives the routing configuration via F1AP. - The CU configures a routing entry on each descendent node with child-node(s) via F1AP after migration. RAN3 think the Alt 1 may be supported by implementation, and Alt2 has impact on RAN2. RAN3 would like to know if Alt2 can be supported and whether it requires enhancements to current specifications. |

Summary:

Since companies still share different views on alt1 and alt2 based on the comments received yet. Moderator thinks we may need to converge on the LS in R3-224061 drafted based on the comments from all the companies, and we can discuss on the session to decide whether to send the LS to RAN2.

References

| 1 | R3-223116 | (CR TS 38.401): IAB Rel-17 Corrections (Ericsson) | CR0203r, TS 38.401 v17.0.0, Rel-17, Cat. F |
| --- | --- | --- | --- |
| 2 | R3-223219 | Discussion on IP address request for boundary node in inter-donor redundancy (Fujitsu) | discussion |
| 3 | R3-223220 | CR for 38.401 on inter-donor redundancy (Fujitsu) | CR0208r, TS 38.401 v17.0.0, Rel-17, Cat. F |
| 4 | R3-223250 | Corrections for IAB (Nokia, Nokia Shanghai Bell) | discussion |
| 5 | R3-223251 | Corrections for IAB (Stage-2) (Nokia, Nokia Shanghai Bell) | CR0212r, TS 38.401 v17.0.0, Rel-17, Cat. F |
| 6 | R3-223294 | Corrections on procedures for inter-donor topology adaptation, redundancy, and BH RLF recovery in TS 38.401 (ZTE) | CR0215r, TS 38.401 v17.0.0, Rel-17, Cat. F |
| 7 | R3-223308 | Correction on IP address allocation (Lenovo) | CR0216r, TS 38.401 v17.0.0, Rel-17, Cat. F |
| 8 | R3-223384 | Remaining issue for source IP selection and concurrent TNL migration (Huawei) | discussion |
| 9 | R3-223385 | Miscellaneous correction for IAB enhancement (Huawei) | CR0218r, TS 38.401 v17.0.0, Rel-17, Cat. F |
| 10 | R3-223541 | Correction on Rel-17 eIAB(Stage-2) (Samsung, Huawei) | CR0221r, TS 38.401 v17.0.0, Rel-17, Cat. F |
| 11 | R3-223673 | CR to 38.401 for Rel-17 IAB (Qualcomm Incorporated) | CR0231r, TS 38.401 v17.0.0, Rel-17, Cat. F Late contribution |