3GPP TSG-RAN WG3 Meeting # 116-e	R3-224075

# Online,  May 9th 2022 - May 19th 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | xxxx | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TS 38.300 corrections for CCO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-05-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 18) |  |
| Reason for change: |  | Section 15.5.5.1 (General) does not clearly indicate what a CCO issue is and that CCO function can be used to resolve (in addition to mitigate) a CCO issue. In section 15.5.5.3 (Dynamic coverage configuration changes): - there is no reference to CCO issue mitigation or resolution - the term “coverage state indicator” is not consistently used - there is a reference to CCO configuration, which is not a clear term - the wording “neighbor cell configuration” does not clearly indicate the coverage aspect |  |  |  |  |  |  |  |  |
| Summary of change: |  | Some text is added to section 15.5.5.1 (General) to introduce the term “CCO issue” and to indicate that CCO function can be used to resolve (in addition to mitigate) a CCO issue. In section 15.5.5.3 (Dynamic coverage configuration changes): - a clarification is added to indicate that actions taken at an NG-RAN node refer to mitigation or resolution of a CCO issue - the term “coverage state indicator” is consistently used - the reference to CCO configuration is replaced with “coverage configuration”, a term already used in the paragraph - it is clarified that neighbouring cell configurations refer to coverage. Impact Analysis The CR has no impact on the previous version of the specification. No ASN.1 impact. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear what a CCO issue is and that CCO function can be used to resolve it. It is also unclear what a CCO configuration is. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.5.5.1, 15.5.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: correct text in 15.5.5.3 |  |  |  |  |  |  |  |  |

-----------------------------------------------  Start of Changes ---------------------------------------------------------

### 15.5.5	Support for Coverage and Capacity Optimisation

#### 15.5.5.1	General

The objective of NR Coverage and Capacity Optimization (CCO) function is to detect and resolve or mitigate CCO issues, e.g. coverage and cell edge interference issues.

#### 15.5.5.2	OAM requirements

Each NG-RAN node may be configured with alternative coverage configurations by OAM. The alternative coverage configurations contain relevant radio parameters and may also include a range for how each parameter is allowed to be adjusted.

#### 15.5.5.3	Dynamic coverage configuration changes

An NG-RAN node may autonomously adjust within and switch between coverage configurations. When a change is executed, a NG-RAN node may notify its neighbour NG-RAN nodes using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells and SSBs with modified coverage included. The list contains the CGI of each modified cell with its coverage state indicator and optionally the SSB index of each modified SSB with its coverage state indicator.

The coverage state indicator may be used at the receiving NG-RAN node to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the coverage state indicator to retrieve a previously stored Mobility Robustness Optimisation state. The coverage state indicator may also be used at the receiving NG-RAN node to adopt coverage configurations matching with neighbouring cells coverage configurations.

If the list includes indication about planned reconfiguration and possibly a list of replacing cells, the receiving NG-RAN node may use this to avoid connection or re-establishment failures during the reconfiguration. Also, if the sending NG-RAN node adds cells in inactive state, the receiving NG-RAN node may use this information to avoid connection or re-establishment failures. The receiving NG-RAN node may also use the notification to reduce the impact on mobility. The receiving NG-RAN node should avoid triggering handovers towards cell(s) that are indicated to be inactive.

-----------------------------------------------  End of Changes ---------------------------------------------------------