3GPP TSG-RAN3 Meeting #116-e	R3-223016

Electronic meeting, 9 May - 19 May 2022

3GPP TSG-RAN WG2 Meeting #117 	R2-2204202

# eMeeting, 21st February – 3rd March, 2022

Title:	LS on RAN2 agreements on NR QoE

Response to: 	-

Release:	Rel-17

Work Item:	NR_QoE-Core

Source:	RAN2

To:	CT1, SA4, RAN3, SA5

Cc:

Contact Person:

#### Name:	Jun Chen

E-mail Address:	jun.chen@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	R2-2204218.zip

1. Overall Description:

RAN2 has discussed NR QoE in RAN2#116b-e and RAN2#117-e meetings, and the WI can be closed from RAN2 point of view. RAN2 agreed TS 38.331 CR is attached (R2-2204218).

The following RAN2 agreements may have impacts on AT-commands:

1.	AS layer indicates to application layer whether session start and end indication is required for the indicated application layer measurement configuration with measConfigAppLayerId

This indication in AT-command is set by transmissionOfSessionStartStop which is forwarded to upper layers. The relevant change in the 38.331 CR is listed as below:

2>	for each measConfigAppLayerId value included in the measConfigAppLayerToAddModList:

[partially omitted]

3> forward the transmissionOfSessionStartStop, if received, to upper layers;

2.	Application layer sends session start or end indication with measConfigAppLayerId for the required application layer measurement configuration if there is session start or end

This indication in AT-command is used to set applicationLayerSessionStatus. The relevant change in the 38.331 CR is listed as below:

2> if session start or stop information has been received from upper layers for the measConfigAppLayerId:

3> set the applicationLayerSessionStatus to the received value of the application layer measurement information;

3.	On RAN visible application layer measurement configuration setup: measConfigAppLayerId, RAN visible application layer measurement configuration (reporting periodicity and metrics)

These parameters in AT-command are set by RRC parameters which are forwarded to upper layers. The relevant change in the 38.331 CR is listed as below:

3> if ran-VisibleParameters is set to setup and the parameters have been received;

4> forward the measConfigAppLayerId, the ran-VisiblePeriodicity, the numberOfBufferLevelEntries and the reportInitialPlayOutDelay to upper layers considering the serviceType;

It is noted that the serviceType is considered by AS layer to forward to upper layers, and whether it needs to be included as AT-command is left to CT1.

4.	On RAN visible application layer measurement configuration release: measConfigAppLayerId. NG-RAN can release a list of RAN visible application layer measurement configurations while not releasing the corresponding legacy application layer measurement configurations.

This parameter in AT-command is set by RRC parameter (i.e., measConfigAppLayerId) which is forwarded to upper layers. The relevant change in the 38.331 CR is listed as below:

3> else if ran-VisibleParameters is set to release:

4> forward the measConfigAppLayerId and inform upper layers about the release of the RAN visible application layer measurement configuration;

5.	On RAN visible application layer measurement reporting: measConfigAppLayerId, RAN visible application layer measurement report (buffer level, initial playout delay and PDU-SessionID).

These parameters in AT-command are used to set measConfigAppLayerId, applicationLayerBufferLevelList, initialPlayoutDelay, and pdu-SessionIdList. The relevant change in the 38.331 CR is listed as below:

2> if RAN visible application layer measurement report has been received from upper layers:

3> for each applicationLayerBufferLevel value in the received RAN visible application layer measurement report:

4> set the applicationLayerBufferLevel values in the applicationlayerBufferLevelLIst to the buffer level values received from the upper layer in the order with the first applicationLayerBufferLevel value set to the newest received buffer level value, the second applicationLayerBufferLevel value set to the second newest received buffer level value, and so on until all the buffer level values received from the upper layer have been assigned or the configured maximum number of applicationLayerBufferLevel values have been set, if any;

3> set the initialPlayoutDelay to the received value in the RAN visible application layer measurement report, if any;

3> for each PDU session ID value indicated in the received RAN visible application layer measurement report, if any:

4> set the PDU-SessionID field in the pdu-SessionIdList to the indicated PDU session ID value;

CT1 may consider the most suitable way to implement these parameters in AT-commands.

2. Actions:

To CT1:

ACTION: 	RAN2 respectfully asks CT1 to consider RAN2 agreements in their future work, especially about the possible impacts on AT-commands listed above.

To SA4, RAN3, SA5:

ACTION: 	RAN2 respectfully asks SA4, RAN3, and SA5 to consider RAN2 agreements in their future work.

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 Meeting #118-e 	16 – 27 May 2022    Electronic

TSG-RAN WG2 Meeting #119 	August 2022    Electronic