3GPP TSG-RAN3 Meeting #116-e	R3-223028

Electronic meeting, 9 May - 19 May 2022

SA WG2 Meeting #S2-150E	S2-2203049

06 - 12 April, 2022, Electronic meeting

# Title:	Reply LS on Clarifications on Namf_MBSCommunication N2MessageTransfer service operation

# Response to:	S2-2201933/ C4-221427

# Release:	Rel-17

# Work Item:	5MBS, NR_MBS-Core

Source:	SA2

To:	CT4

Cc:	RAN3

Contact Person:

Name:	Thomas Belling

E-mail Address:	Thomas (dot) belling (at) nokia (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

The MB-SMF uses the Namf_MBSCommunication N2MessageTransfer service operation during the multicast MBS session activation and deactivation procedure and multicast session update procedure, as specified in clauses 7.2.5.2, 7.2.5.3, 7.2.6 and 9.3.3.2 of TS 23.247.

Upon receipt of this service request, the AMF distributes the N2 payload received from the MB-SMF towards the NG-RAN(s) serving the MBS service area of the multicast MBS session.

In all these 3 procedures, based on the description in the aforementioned clauses, CT4 assumes that:

- the AMF returns a successful response to the MB-SMF as soon as at least one NG-RAN returns a successful response to the AMF;

- the AMF returns an unsuccessful response to the MB-SMF, if the AMF cannot serve the request at all (e.g. unknown PDU Session ID) or if none of the NG-RANs returns a successful response;

and

- in either case, the AMF does never forward any successful or unsuccessful N2 payload(s) back to the MB-SMF.

CT4 kindly asks SA2 to:

- confirm whether CT4 understanding is correct;

SA2 confirms that understanding. The output of Namf_MBSCommunication service operation is without N2 container.

- clarify how the system should behave when a multicast session activation, deactivation or update request would fail for an NG-RAN; and

This is not specified in this Release.

2. Actions:

To CT4 group.

ACTION: 	SA2 asks CT4 to take the provided responses into consideration.

3. Date of Next SA2 Meetings:

SA2#151e	16th - 20th May 2022	electronic meeting