3GPP TSG-RAN WG3 #117-e								R3-224233

# Electronic Meeting, 15th - 24th August 2022

Agenda Item:	13.3

Source:  	KT, LG Uplus, SKT, ETRI

Title: 	Handling legacy UE in UAM using Mobile-IAB

Document for:	Discussion

# Introduction

Korea government plans to initiate UAM (Urban Aerial Mobility) service from the second half of 2024, which is aligned with the expected commercial timeline of Rel-18. In order to support the government’s commercialization plan, one issue should be resolved that legacy UE on board without Rel-18 features can also take a data service similar as on the ground, where at least voice quality should be maintained as sufficient level. In this contribution, the issues related with legacy device in aerial vehicle will be described and mobile IAB is introduced as a possible solution.

# Discussion

## Problems on legacy device riding aerial vehicle

Referring to LTE UAV (Uncrewed Aerial Vehicle) WI, aerial channel has higher LOS probability than ground channel [1]. On the same deployment scene, a UAV device can see radio signal from more cell than a ground device. DL and UL interference strength experienced by UAV would be stronger and, hence, throughput performance would decrease comparing to a ground device. To improve DL/UL throughput, a network needs to impose a solution, e.g. CoMP operation, beam direction change, etc. As similar as LTE UAV device, a Rel-18 device with NR UAV capability could send a measurement report message triggered by altitude-based measurement event. However, a legacy device such as Rel-15/16/17 device cannot send the measurement report since it does not support the feature. Thus, the network could not apply an appropriate solution to the legacy device.

Observation 1: A legacy device such as a Rel-15/16/17 device which rides aerial vehicle cannot send a measurement report message to indicate flying state although a proper solution needs to be applied so as to prevent DL/UL throughput performance degradation.

Also, aerial channel characteristic with higher LOS probability can induce neighbour relationship among cells to alter comparing to one on the ground. Especially, a cell which to be temporarily seen only by an aerial device should not be included into the NRT (Neighbour Relationship Table) by ANR (Autonomous Neighbour Relationship). However, since a legacy device will not be able to indicate flying situation and could report erroneous measurement result for the temporal strong cell, network would insert the wrong relationship information into the NRT, which would waste NRT storage.

Observation 2: The erroneous measurement report from a legacy device for the temporal strong cell on the flying would cause the distortion of NRT by ANR.

New spectrum for UAM might be a good approach to make a network deployment elaboration easier. A dedicated network deployment and configuration only for UAM could be possible depart from the existing one since they could be managed independently due to different spectrum utilization. However, it could have hardly been convinced that a legacy device would support the new spectrum.

Observation 3: The new spectrum utilization for UAM would be hard to be supported by a legacy device.

## Candidate solution using mobile-IAB

In this solution, a legacy device would connect to mobile IAB of aerial vehicle. MT of the mobile IAB should have an UAV feature such as measurement report triggered by altitude. As network receives the measurement report from mobile IAB, it can recognize by mapping between DU of the mobile IAB and UEs which legacy UEs are flying. Network could reconfigure the legacy UEs so as to stay in the serving mobile IAB DU.

Figure 1: Mobile-IAB based solution for supporting legacy NR UEs

## Analysis of candidate solution

Table 1 summarizes mobile-IAB based solution for supporting the legacy NR UE on UAM;

Table 1: Analysis of mobile-IAB solution handling legacy device riding aerial vehicle

| Solution | Redeeming throughput degradation | Preventing NRT distortion | UAM dedicated spectrum | UE Power Saving | Specification |
| --- | --- | --- | --- | --- | --- |
| Mobile IAB | Support | Support | Available | Support | - Measurement report triggered by altitude in mobile IAB MT - Provisioning mapping information between UE ID and flying state to AMF/MME/donor gNB - Preventing ground cell reselection |

Based on above analysis, we suggest to insert support for UAM into mobile-IAB WI.

Proposal 1: It is proposed to revise Mobile-IAB WID to support UAV features for UAM services as proposed in clause 5.

# Conclusion

In this contribution, the below observations as for handling a legacy device (Rel-15/16/17 device) in UAM are shown;

Observation 1: A legacy device such as a Rel-15/16/17 device which rides aerial vehicle cannot send a measurement report message to indicate flying state although a proper solution needs to be applied so as to prevent DL/UL throughput performance degradation.

Observation 2: The erroneous measurement report from a legacy device for the temporal strong cell on the flying would cause the distortion of NRT by ANR.

Observation 3: The new spectrum utilization for UAM would be hard to be supported by a legacy device.

Based on the above observations, we believe that issues on handling legacy device in UAM are essential. We kindly suggest as following;

Proposal 1: It is proposed to revise Mobile-IAB WID to support UAV features for UAM services as proposed in clause 5.

# References

- TR36.777, “Study on Enhanced LTE Support for Aerial Vehicles”

# Text Proposal

====================== <  Skipped > ======================

## 3	Justification

The support for Mobile Integrated Access and Backhaul (IAB) builds on the architecture and protocols derived in the Rel-17 WI NR_IAB_enh, which provided IAB improvements on various aspects such as robustness, load-balancing, spectral efficiency, and end-to-end performance.

The work on Mobile IAB in Rel-18 should focus on the scenario of mobile-IAB-nodes mounted on ground or aerial vehicles providing 5G coverage/capacity enhancement to onboard and/or surrounding UEs.

In Rel-18, mobile IAB supports the following functionality, applicable to FR1 and FR2:

- In-band and out-of-band backhauling.

- The mobile IAB-node should have no descendent IAB-nodes, i.e., it serves only UEs.

- Solutions should support UE HO and DC.

- The MT of a mobile IAB-node should support UAV features for UAM service.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The detailed objectives of the WI are listed as follows:

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2]

- Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2]

Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility.

- Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

- The MT of IAB-node equipped with UAV features [RAN2]

- As for UAM use case, define procedures for the DU of IAB-node to provide flying state information of a UE riding the aerial vehicle. [RAN3]

The following principles should be respected:

- Mobile IAB-nodes should be able to serve legacy UEs.

- Solutions providing optimization for Mobile IAB may entail Rel-18 UE enhancements, provided that such enhancements are backwards compatible

RAN4 is expected to study impact on RF and RRM requirements:

- Conduct co-existence study to assess the impact of moving cells. Based on the study outcome, specify RF and RRM requirements and mechanisms for the mobile IAB-node to enable co-existence, if needed.

- Specify RRM requirements for the mobile IAB-node to enable IAB-node mobility, if needed.

The involvement of RAN1 may be needed, depending on work progress.

====================== <  Skipped > ======================