3GPP TSG-RAN WG3 Meeting #117-e	R3-224250

# E-meeting, -, 15. - 24.08.2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of the missing descripition of SCG MRO handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure to enable analysis of the SCG failure cases is based on possible UE reporting, collecting and storing SN change information and using 2 separate procedures. Therefore, to enable proper interpretation, it has to be described well. Currently, the description is distributed in 3 specifications: TS 38.423, TS 37.340 and TS 38.300. While the stage-3 is quite clear, both stages-2 are very vague and say roughly the same. At RAN3 #116, it was proposed to add more description to TS 37.340, but it was not agreed (only the “FFS” was removed). On the other hand, stage-3 refers to TS 38.300, e.g. when mentioning that the SN Mobility Information and the Source PSCell CGI shall be stored after the SN Change procedure. However, the TS 38.300 does not mention handling of this information at all! Also, the use of the SCG Failure Transfer procedure is not mentioned in any stage-2! |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the description of the PSCell change failure handling, the use of the SCG Failure Transfer procedure is mentioned, as well as the handling of the SN Mobility Information and of the Source PSCell CGI. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change only clarifies the intended use of the procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | References in the stage-3 are not addressed in the referred specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.5.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change, ommited text not changed

#### 15.5.2.6	PSCell change failure

For analysis of PSCell change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN which is responsible for the PSCell change failures (see the clause 13.3 in TS 37.340 [21]). Otherwise, the MN transfers the SCG Failure Information to the last serving SN. If needed, the MN transfer the SCG Failure Information to the source SN (see the clause 13.3 in TS 37.340 [21]), which may respond using the SCG Failure Transfer procedure to inform the MN it is not responsible for the SCG failure.

When transferring the SCG Failure information to the SN, the MN shall, if supported, include the source PSCell CGI and the SN Mobility Information received in the last S-NG-RAN node initiated S-NG-RAN node Change procedure.

Remaining text not changed