3GPP TSG-RAN WG3 Meeting #117-e	R3-224251

E-meeting, 15 – 24 Aug 2022

Agenda item:	14.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Continuation of the work on CHO with DC and optimisation of the data forwarding

Document for:	Discussion

# 1	Introduction

One of the objectives of the WID, approved at RAN #96 in June, is further work on Rel.17 CHO with DC operation kept at the target node:

- To specify CHO including target MCG and target SCG [RAN3, RAN2].

Note 5: This is already being targeted for Rel-17, so this objective will be reviewed at RAN#97-e.

In this paper, we’re checking if there is anything left to be done.

# 2	Discussion

The solution agreed at RAN3 #115 enables the target MN to indicate, in the Addition preparation, that the UE may arrive later. It is also possible to include source MN’s IDs, so that the target SN may identify preparations related to the same UE, even if they are coming from different target MNs. Finally, the arrival probability may be passed to the target SN. All these features help the SN to optimise resource allocation related to the same UE.

However, data forwarding was not addressed separately in the Rel.17 solution. If early data forwarding is used, and there are SN-terminated bearers, the target SN is likely to receive the same data forwarded via different routes.

Observation 1: Since the source MN/SN is not aware there is a single target node in case of CHO with DC and SN-terminated bearers, the source will forward data to all prepared target MNs and, if SN-terminated bearers are configured, the target SN will receive the same data via multiple routes.

On one hand, the situation is not severe: if information on delivered data is provided, the multiplied data can be removed every now and then. On the other hand, sending the same via multiple routes, especially if indirect forwarding is used, abuses resources of the network.

Proposal 1: RAN3 shall consider if forwarding the same data over multiple paths to the same target SN is a problem.

This issue can be mitigated, if the target SN allocates the same TEID for all addition preparations and these TEIDs are passed to the source node for direct forwarding: the source can identify there is a single destination for given bearer(s) and send the data only once. However, providing the discarding information is still separated.

Proposal 2: If the issue requires RAN3’s attention, it shall be considered if existing implementation-based methods are sufficient, or if any signalling support is needed.

# 3	Conclusions

In this paper, we discuss briefly possible issue related to data forwarding when there is a common target SN:

Observation 1: Since the source MN/SN is not aware there is a single target node in case of CHO with DC and SN-terminated bearers, the source will forward data to all prepared target MNs and, if SN-terminated bearers are configured, the target SN will receive the same data via multiple routes.

Based on this, we make 2 proposals:

Proposal 1: RAN3 shall consider if forwarding the same data over multiple paths to the same target SN is a problem.

Proposal 2: If the issue requires RAN3’s attention, it shall be considered if existing implementation-based methods are sufficient, or if any signalling support is needed.