3GPP RAN WG3 meeting #117	R3-224253

E-meeting, 15 – 24 August 2022

Agenda item:	14.4

Source:	Nokia, Nokia Shanghai Bell

Title:	NR-DC with selective activation of cell groups

# 1	Introduction

CHO and CPAC functionality defined in Rel-16 and CPAC enhancements to support inter-SN scenarios in Rel. 17 focus only on defining the methods for conditional execution of cell change based on the execution condition for various single connectivity and dual connectivity scenarios. The candidate configurations, except the execution target configuration prepared for multiple cells, are released at the time of execution of conditional reconfiguration. However, the UE can still move quickly to one of the configured but released candidate cells or back to the source cell after the initial execution. Thus, new signalling procedures and optimisations are needed to minimise the signalling overhead associated with this scenario. The objective of Rel-18 WID for mobility enhancements is motivated based on the above scenario.

- To specify mechanism and procedures of NR-DC with the selective activation of the cell groups (at least for SCG) via L3 enhancements: - To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4]

This discussion is a very similar to the one provided in RAN2.

# 2	Discussion

## 2.1	The scope of the problem

The primary scenario for the above objective is illustrated in the following figures. Figure 1 illustrates the potential UE mobility across group of PSCells under the overlay coverage of NR PCell. In this scenario, the UE is configured with CPAC mobility for target cell PSCell 3. If the UE’s actual mobility is towards PSCell 2, the MN may trigger classic PSCell change through RRC-Reconfiguration to the target cell PSCell 2. If the PSCell 3 configuration is preserved after the mobility to PSCell 2, it may be possible to have the CPAC execution continued from PSCell 2.

Figure 1 : Classic PSCell-Change to new Target-cell when UE have stored CPAC configuration for PSCell 3

It is also possible that UE may have CPAC configurations for PSCell-2 and PSCell-3 stored at UE with initial CPAC execution to PSCell 2. If the CPAC configurations are stored after initial execution the UE can still further execute CPAC to PSCell 3.

NR-DC deployment scenarios with FR2 based PSCells or NR-DC deployments with secondary cell layers serving as capacity boosters with densely deployed secondary cells are expected to have frequent mobility across the PSCells. For these deployments, the mobility scenarios illustrated in Figure 1 are more relevant and can be considered as the main scenarios for selective activation.

Proposal 1: Keeping of CPAC configurations after RRC-Reconfiguration for cell change are considered as primary scenario for the objective.

Rel-17 specifications introduces CPA functionality where the UE can conditionally perform PSCell addition to move from single connectivity to dual connectivity. In this scenario also the UE may move to one of the prepared target PSCell after initial CPA execution. In such scenario, the UE should be able make use of stored configuration of target-PSCell for further execution of CPAC across the prepared PSCells for CPA.

Proposal 2: Objective also covers the scenario of keeping the initial CPA configuration after the initial CPA execution.

In case of CPC, intra-SN and SN-initiated inter-SN CPC scenarios are the potential mobility scenarios which can be benefit from the selective activation functionality. MN-initiated inter-SN CPC is mainly defined for offloading scenarios where MN identify better target PSCell suitable for switching the traffic. We don’t see major benefit to extend the selective activation functionality for MN-initiated CPC scenario.

Proposal 3: MN-initiated CPC scenario can be deprioritized for selective activation

While we agree to the current objective to prioritise the SCG mobility for selective activation, the key issues and solutions could be applicable to CHO for PCell also with very minor changes.  Frequent mobility across group of cells can be still valid scenario for NR single connectivity which operates at higher frequencies. We propose to consider the extension of the selective activation concept for MCG and re-use of the signalling procedure defined for SCG for MCG. Rel. 16 CHO can be used as baseline for selective activation of MCG.

One must remember also that the solution defined for lower-layer mobility, on which discussion starts in Rel.18, may be also relevant for possible selective activation for MCG.

Proposal 4: Discussion on enabling selective activation for MCG mobility (handover) could possibly be started only once the selective activation for SCG is addressed (as well as when details of lower-layer mobility are known).

After detecting SCG failure, UE stops evaluating the CPAC conditions to make sure it is up to network to decide which PSCell UE should access. As a result of the SCG failure procedure, UE either releases or establishes connection to a new PSCell and UE has to release all CPAC configurations afterwards. This is contradicting to the goal of the objective as UE cannot do cell group change without re-configuration and re-preparation in case of such a release. Objective should also cover SCG failure enhancements to enable PSCell addition and PSCell change after SCG failure.

Proposal 5: The WI objective also covers SCG failure enhancements to enable PSCell addition and PSCell change after SCG failure.

## 2.2	Key Issues for RAN3 to address

Most of the issues have to be decided in RAN2 first. These are, among others:

- The target configurations prepared for CPAC cannot be used after initial CPAC execution if the target node provides the target PSCell configuration as modification over current PSCell configuration.

- Use of full-configuration or Release/Addition of SCG will significantly impact the interruption time and delay in execution of selective activation with additional signalling and storage overhead.

- Execution condition for CPAC after initial CPAC execution and the reference measurement-configuration of each target PSCell requires additional coordination within MN and SN for SN-initiated intra/inter-SN CPAC scenarios.

- The target configurations are maintained for longer duration than current CPAC executions. Resource reservation related issues and release or pre-emption of configuration at target nodes needs to be addressed for the selective activation.

- A fresh SN counter value for the CPAC configurations after initial CPAC execution has to be provisioned to avoid a security issue.

Once these problems are decided, at least in an initial form, RAN3 will be able to decide upon the needed signalling.

Proposal 6: RAN3 shall wait with the overall signalling design for the first decisions of RAN2.

However, RAN3 may start discussion on some key issues specific to RAN3:

- Data forwarding overhead caused by maintaining the CPAC configurations for a longer time.

As the CPAC configurations are maintained for a longer time, in case of early data forwarding, higher overhead for the network might be caused.

- Data forwarding coordination for maintained CPAC configurations after PSCell change.

As the entity responsible for data forwarding may change while the CPAC configuration is maintained, the required coordination has to be discussed.

- Network release behaviour of CPAC preparation after the PSCell change/addition/release procedure.

The network always releases the CPAC preparations, as the UE releases the CPAC configurations after each PSCell change/addition/release. The objective requires modifications to such behavior.

- Preparation procedure modification related to the selective activation.

Network entities need to differentiate between Rel. 16, 17 CPC preparations with Rel. 18 CPC preparations.

- Coordination of incoming probability.

Incoming probability for each CPAC configuration would be assumed 0 after PSCell change for pre Rel. 18 behavior. Rel. 18 will change this and the incoming probability after PSCell change has to be coordinated for each maintained PSCell.

Proposal 7: RAN3 may start considering some aspects of the signalling that are independent from RAN2’s decisions.

# 4	Conclusion

In this paper, we start the discussion on the selective activation for NR-DC. We make following proposals:

Proposal 1: Keeping of CPAC configurations after RRC-Reconfiguration for cell change are considered as primary scenario for the objective.

Proposal 2: Objective also covers the scenario of keeping the initial CPA configuration after the initial CPA execution.

Proposal 3: MN-initiated CPC scenario can be deprioritized for selective activation

Proposal 4: Discussion on enabling selective activation for MCG mobility (handover) could possibly be started only once the selective activation for SCG is addressed (as well as when details of lower-layer mobility are known).

Proposal 5: The WI objective also covers SCG failure enhancements to enable PSCell addition and PSCell change after SCG failure.

Proposal 6: RAN3 shall wait with the overall signalling design for the first decisions of RAN2.

Proposal 7: RAN3 may start considering some aspects of the signalling that are independent from RAN2’s decisions.