3GPP TSG-RAN WG3 Meeting #117-e	R3-224256

15 August 2022 – 25 August 2022

Online

Agenda Item:	12.2.2

Source:	InterDigital

Title:	Discussion on Mobility Optimization Model Outputs

Document for:	Discussion and Agreement

# 1	Introduction

In December RAN plenary approved the work item for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

The main objective of the WI is:

Specify data collection enhancements and signalling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3).

This is the follow up to the study of these topics the outcome of which is captured in 37.817.

The study item made a number of agreements on how to proceed but left a number of topics open to be resolved during the specification phase.

This contribution tries to look at the outputs of the mobility optimization model and discuss whether the output needs to go outside of the local RAN node and thus have standards impact.

# 2. Discussion

Below are agreed outputs of the mobility optimization model:

UE trajectory prediction (Latitude, longitude, altitude, cell ID of UE over a future period of time) Note: Whether the UE trajectory prediction is an external output to the node hosting the Model Inference function should be discussed during the normative work phase.

Estimated arrival probability in CHO and relevant confidence interval

Predicted handover target node, candidate cells in CHO, may together with the confidence of the predication

Priority, handover execution timing, predicted resource reservation time window for CHO.

UE traffic prediction (will be used by the RAN node internally and the details are left to normative work phase)

Model output validity time will be discussed during R18 normative work per inference output.

These all need a little further discussion.

## 2.1 UE Trajectory Prediction

There are inherent privacy issues with network nodes knowing the location of particular UEs, which needs to be explored by SA3 in their work on AI/ML. However, trajectory prediction just based on cell IDs would be useful for the RAN and not be too private. For example, a prediction that this UE is in cell A, and will move to cell B and then cell C would be useful in the target gNB for optimizing future handovers and would require very limited extensions to existing IEs, e.g., UE Mobility History IE.

## 2.2 Estimated Arrival Probability in CHO and Priority, handover execution timing, predicted resource reservation time window for CHO.

These related outputs are relevant to the target gNB to optimize both resource reservation when preparing handovers and to optimize data forwarding, particularly early data forwarding. In CHO, the UE is pre-configured for a HO that will happen in the future so predicting aspects these related to CHO is an obvious enhancement that perfectly fits the scope of the AI/ML work. However, more discussion is probably needed on the need for priority.

## 2.3 Predicted Handover target node

This output is directly used to determine the target gNB and cell for the optimized handover, thus it may not need or lead to message enhancement. One possibility is to allow target node to potentially prepare resources in the future, tying perfectly with the idea of a neighbouring RAN node predicting its own resources. This would lead to a possible message enhancement to be an indication that the HO preparation is based on a prediction.

## 2.4 UE traffic prediction

It is clear that UE traffic prediction will be used by the local node, but the admission control in the target gNB could very much make use of the expected UE traffic that is predicted which would help scheduling resources for reception of that UE. On the other hand, the UE probably will not benefit from being sent the prediction. In fact for uplink data the UE would have a better understanding of the predicted traffic anyway.

## 2.5 Model output validity time

This is covered in another paper in more detail, in our view, if it applies, it is only relevant to outputs that are sent over standardized interfaces.

# 3	Proposal

Based on the discussion above, the following proposal should be agreed:

Proposal: The following mobility optimization model outputs should be added to the Handover Preparation Procedure (HANDOVER REQUEST), exact IE structure is FFS:

Estimated Arrival Probability in CHO

Priority (FFS)

handover execution timing/predicted resource reservation time window for CHO

UE Traffic prediction

FFS UE Trajectory Prediction (at the cell id level at least) dependent on validation of SA3.