3GPP TSG-RAN WG3 Meeting #117-e	R3-224258

15 August 2022 – 25 August 2022

Online

Agenda Item:	12.2.2

Source:	InterDigital

Title:	QoS Feedback

Document for:	Discussion and Agreement

# 1	Introduction

In December RAN plenary approved the work item for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN

The main objective of the WI is:

Specify data collection enhancements and signaling support within existing NG-RAN interfaces and architecture (including non-split architecture and split architecture) for AI/ML-based Network Energy Saving, Load Balancing and Mobility Optimization. (RAN3).

This is the follow up to the study of these topics the outcome of which is captured in 37.817.

The study item made a number of agreements on how to proceed but left a number of topics open to be resolved during the specification phase.

This contribution looks at one of the open issues namely QoS Feedback.

From the TR:

Energy savings

- UE performance affected by the energy saving action (e.g., handed-over Ues), including bitrate, packet loss, latency.

-	System KPIs (e.g., throughput, delay, RLF of current and neighbouring NG-RAN node)

Load balancing

-	UE performance information from target NG-RAN (for those Ues handed over from the source NG-RAN node)

-	System KPIs (e.g., throughput, delay, RLF of current and neighbours)

Mobility

- QoS parameters such as throughput, packet delay of the handed-over UE, etc.

- Performance information from target NG-RAN. The details of performance information are to be discussed during normative work phase.

# 2	Discussion

The topic of feedback on optimization actions is particularly important for AI/ML based decision-making engines because, in many instances, these rely on feedback information for self improvement of ML models and general improvement of processes, in the case of more intelligent engines. The feedback aspects become even more important knowing that users served are never or almost never in the same situation when it is time to perform a mobility action for one or more users, i.e., user position, radio conditions, cell load, service requirements, etc. constantly change, leaving an AI/ML based intelligent engine even more permeable to not making the optimal decision, and creating a bigger need for a decision making node to receive feedback. This applies to all kinds of decision making, independently of the target optimization and has been the case in all earlier SON mechanisms, so it only makes sense to consider it as well in the scope of the AI/ML work.

For the particular case of QoS related feedback, in TR 37.817, companies considered it as an important aspect, as all three use cases consider feedback information for both system and UE QoS related metrics exchanges. However, from inspecting the feedback information in the report, there is a misalignment between the terminology and parameter examples given per use case.

The misalignment in itself, is not a problem. The feedback information needs to be closely tied with the target optimization. Different target optimizations for a use case may therefore not require the same feedback information as an action for another use case. If we talk about one UE only, system level KPIs may be of less importance (not null importance). In another example, if we handover a group of UEs for Energy Savings for example, we will want to consider, on top of user service disruption, system KPI information exchanges. However, the case is different for the UE QoS, as an example, when handing over a UE based on AI/ML decisioning from RAN node A to RAN node B, we likely want to focus on whether the service for that UE has been disrupted this is true for all use cases equally, therefore it is probably desirable to align the UE QoS feedback parameters for all use cases.

In addition, we also highlight that UE QoS related feedback is the most important. All three use cases have mobility as the underlying mechanism for optimization and the QoS impacts from mobility are interruption time, establishment of bearers that can fail, RACH that can fail, etc. Energy savings and load balancing are NW processes with excellent benefits. However, good engineering solutions need to account for user service because users should not even notice these procedures are happening and they cannot happen at an excessive cost of user experience. Both the benefits of the use cases and user experience can be achieved at the same time. This should in fact be the sole purpose of the AI/ML work, to leverage on the ML mechanisms to anticipate events and be able with the extra time to make proper decisions that achieve the benefits with the seamless experience for the users.

There is a lot of discussion around which parameters to use for this UE QoS feedback and it is clear that there are some basic “measurements” that are absolutely necessary and others that should continue to be discussed. Having a report from the target gNB to the source gNB on the UE performance in the target cell after handover is interruption time, and UL and DL data rates. The amount of time to use to “measure” this could be a fixed period of time or something configurable (for example at handover preparation).

Realizing all the above arguments, we make in this contribution the following points:

Observation 1: Feedback information is of extreme importance to improve AI/ML decision making process and should be included for all use cases and decision making, just like it has been for SON;

Observation 2: Feedback information will need further discussion to fine tune per use case, and it is clear that different use cases and different actions resulting from AI/ML decisioning require different feedback information

Observation 3: Both UE and system level performance information will be required. UE QoS should be aligned between use cases since each involves one or more UEs moved because of the model output. System level performance can and should be different between use cases, since there are different network effects for multiple UEs moved for energy savings or moved for load balancing and when single UEs are moved due to mobility optimization.

Proposal 1: System level KPIs/QoS parameters are FFS and need distinct discussions for each use case

Proposal 2: Since all 3 use cases involve moving UEs, the UE QoS parameters shared for feedback should be a common set measured over either a fixed or configurable timeframe after handover and should include at least:

Handover interruption time

DL Data Rate

UL Data Rate

# 3	Proposal

Proposal 1: System level KPIs/QoS parameters are FFS and need distinct discussions for each use case

Proposal 2: Since all 3 use cases involve moving UEs, the UE QoS parameters shared for feedback should be a common set measured over either a fixed or configurable timeframe after handover and should include at least:

Handover interruption time

DL Data Rate

UL Data Rate