3GPP TSG-RAN WG3 Meeting #117-e	R3-224276

Online, 15th – 24th August 2022

Agenda Item:	9.2.8

Source:	Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei

Title:	Discussion on coordination of CHO and intra-SN CPC

Document for:	Approval

# 1	Introduction

RAN2 has discussed the coexistence and confirmed the followings in last meeting.

Confirm the working assumption and agree the following: when one conditional reconfiguration is executed, the other conditional reconfigurations are released.

Confirm the working assumption and agree the following: MN indicates (via CG-ConfigInfo) how many conditional reconfigurations Source SN is allowed to initiate.

In this contribution, we provide analysis on the related RAN3 impacts.

# 2	Discussion

## 2.1 	Background

As being agreed in RAN2, from Rel-16 the conditional reconfigurations are released in the UE upon execution of any conditional reconfiguration or upon execution of reconfigurationWithSync in the MCG or at reconfigurationWithSync in the SCG if CPC or CPA is configured. Once the UE releases the conditional reconfigurations, the network should also release the conditional reconfigurations in order to avoid any mismatch in the configurations between the UE and the network and also to free up network resources. However, the network configurations are done by both the MN and the SN, also one node may not be aware of what reconfigurations have been done in the other node.

Taking Figure 1 below as an example, while both CHO and CPC are configured, the MN is not aware of that the SN has configured the UE with CPC, as the RRCReconfiguration message is transparent to the MN and directly forwarded to the UE. Also, when the CPC is executed, the RRCReconfigurationComplete message within ULInformationTransferMRDC is forwarded to the SN as a container, i.e. it is transparent to the MN. The UE till thus releases the conditional reconfigurations, but the MN will not be aware of this.

Figure 1: Example problematic scenario: CHO and CPC configurations, SRB1 is used

Current MN is not able to know whether the other conditional reconfigurations have been released by the UE when one of the conditional reconfiguration is executed.

## 2.2 	Use cases

In this section the possible cases are given below.

Case 1: CHO + R16 CPC

Case 2: MN initiated CPC + R16 CPC

Case 3: SN initiated CPC + R16 CPC

Case 4: Others, e.g., combination with legacy PSCell change

## Case 1: CHO + R16 CPC

Preparation phase: Both CHO and R16 CPC are configured to the UE.

Execution phase:

If CHO is executed, S-MN sends SN RELEASE REQUEST message to S-SN, then R16 CPC is cancelled.

If R16 CPC is executed, S-SN would need to tell the S-MN about CPC execution, so that S-MN sends CONDITIONAL HANDOVER CANCEL message to all of candidate T-MN, then CHO is cancelled.

## Case 2: MN initiated CPC + R16 CPC

Preparation phase: Both MN initiated CPC and R16 CPC are configured to the UE.

Execution phase:

If CPC is executed, S-MN sends the SN RELEASE REQUEST message to S-SN, then R16 CPC is cancelled.

If R16 CPC is executed, S-SN would need to indicate R16 CPC execution to S-MN, so that S-MN sends the SN RELEASE REQUEST message to all of candidate T-SN and sends CONDITIONAL PSCELL CHANGE CANCEL message to S-SN, then MN initiated CPC is cancelled.

## Case 3: SN initiated CPC + R16 CPC

Preparation phase: Both SN initiated CPC and R16 CPC are configured to the UE.

Execution phase:

If CPC is executed, S-MN sends the SN CHANGE CONFIRM message to S-SN, then R16 CPC is cancelled.

If R16 CPC is executed, S-SN sends CHANGE REQUIRED message (with CPC-cancellation indicator), in that case SN initiated CPC is cancelled.

## Case 4: Others, e.g., combination with legacy PSCell change

There is also a problem if the UE is configured with CHO + CPC, if a legacy PSCell change is triggered, as the UE will also in this case release all the conditional reconfigurations. PSCell change may be performed in the SN without any information of this being sent to the MN.

In another case when SRB3 is used, the MN is not aware of that the UE has been configured with CPC. There is no message passed via the MN, thus it is impossible for the MN to has any knowledge of that the UE is configured with CPC. The MN is also unaware of other reconfigurations in the SN, such as reconfigurationWithSync.

One specific case is that CHO is configured and legacy PSCell change is performed while SRB3 is used. In legacy MN is not involved, however SN is possible to tell MN that RRCReconfiguration occurs, though the conditional reconfiguration is not released in the UE for CHO. The above new indicator can be reused and how the MN behaves accordingly will leave for implementation, e.g., check cause value to understand which case is applied.

The listed cases should be supported considering the coexistence of CHO and other conditional reconfigurations.

## 2.3 	Network Solution

As analyzed above, a simple indicator over Xn would be a straightforward way to make S-MN be aware of the release of conditional reconfiguration in the UE. Another possible solution will be introducing new signaling, though the efforts would be a bit too much. Therefore we propose to introduce a new indicator to the SN MODIFICATION REQUIRED message then it can be sent from the SN to MN after the SN receives the RRCReconfigurationComplete message from the UE, i.e., UE has executed CHO or CPC and released all the conditional configurations.

Furthermore, one remaining point would be how the MN informs SN about configured CHO or MN initiated CPC. In stage-2 signaling flows, the following description is given for MN initiated CPC. Naturally it makes sense to update the Xn-U Address Indication message (for MR-DC) and Data Forwarding Address Indication (for EN-DC) to mandatory in the charts.

4a.	Upon receiving the MN RRCReconfigurationComplete message from the UE, the MN informs the source SN that the CPC has been configured via Xn-U Address Indication procedure, the source SN, if applicable, together with the Early Status Transfer procedure, starts early data forwarding. The PDCP SDU forwarding may take place during early data forwarding.

Corresponding stage-2 and stage-3 CRs are provided in [2][3][4] to capture the new indicator and related descriptions.

RAN3 agrees to introduce new indicator within SN initiated modification procedure to support coexistence of CHO and other conditional reconfigurations.

Agree stage-2 and stage-3 CRs in [1][2][3].

# Conclusion

In the previous sections we made the following observations:

Observation 1	Current MN is not able to know whether the other conditional reconfigurations have been released by the UE when one of the conditional reconfiguration is executed.

Observation 2	The listed cases should be supported considering the coexistence of CHO and other conditional reconfigurations.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN3 agrees to introduce new indicator within SN initiated modification procedure to support coexistence of CHO and other conditional reconfigurations.

Proposal 2	Agree stage-2 and stage-3 CRs in [1][2][3].

# References

R3-224268, Coordination of CHO and intra-SN CPC for TS37.340, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Ericsson

R3-224248, Coordination of CHO and intra-SN CPC, TS 38.423, Nokia, Nokia Shanghai Bell, ZTE, Huawei, Ericsson

R3-224320, Coordination of CHO and intra-SN CPC, TS 36.423, Huawei, Nokia, Nokia Shanghai Bell, ZTE,  Ericsson