3GPP TSG-RAN WG3 #117-e						R3-224289

E-meeting, 15 – 24 August 2022

Agenda Item:  9.2.8

Source:       NEC, ZTE

Title:		Handling of PDCP COUNT reset in CU-UP for inter-gNB-DU Handover

Document for:	dicision

- Introduction

Last RAN3#116e meeting discussed the issue with related to the use case i.e. the inter-gNB-DU handover, when target gNB-DU generated CellGroupConfig using full configuration, while gNB-CU decide to keep the same gNB-CU-UP that need to reset the PDCP COUNT of the existing DRBs. (refer to R3-213181)

Some solutions were raised and discussed, resulted in the chair-minuted that:

“For the use case of inter-DU handover that target gNB-DU has taken full configuration decision while gNB-CU decide to keep the same gNB-CU-UP that need to reset the PDCP COUNT of the existing DRBs, it is confirmed that the existing signalling with two times the E1AP: Bearer Context Modification procedures (first to release DRBs + adding the same DRBs, second to give Target DU’s DL TNL address) can work but not optimal as it take longer time to complete handover.”

With this chair minute, this contribution continue to study/discuss the possible ways to reduce the handover time.

(last RAN3#116e the e-mail discussion was summarised in R3-223762.)

- 2. Discussion

The possible solutions to reduce the handover time for the use case are simply to execute one time of signalling procedure instead of two times (i.e. the E1AP: Bearer Context Modification procedure).

The remaining solutions of the discussion are:

Solution 1: In BEARER CONTEXT MODIFICATION REQUEST message the PDU Session Resource To Modify List IE to release the relevant DRBs (set the DRB To Remove List IE)and also to setup the same DRBs (set the DRB To Setup List IE). Furthermore, because of the need to give the target gNB-DU’s TNL information, then need to newly add the DL UP Parameter IE in the DRB To Setup List IE.

Solution 3: Use of the PDCP SN Status Information IE i.e. in BEARER CONTEXT MODIFICATION REQUEST message the PDU Session Resource To Modify List IE, set the PDCP SN and HFN as “0” in the PDCP SN Status Information IE of the DRB To Modify List IE .

Solution 4: A dedicated IE to indicate reset PDCP COUNT in the DRB to Modify List IE. i.e. in BEARER CONTEXT MODIFICATION REQUEST message the PDU Session Resource To Modify List IE to introduce e.g. new PDCP COUNT Reset IE under the DRB To Modify List IE.

(Note that in last meeting the Solution 2 is the one that has been confirmed that can work but take longer handover time)

Discussion of each solution:

The solution 3 that use of the existing PDCP SN Status Information IE, this IE is for particular function as in the semantic description of the IE that say “Provides the PDCP SN Status to the target gNB-CU-UP.”,

| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status to the target gNB-CU-UP. | - | - |
| --- | --- | --- | --- | --- | --- | --- |

This PDCP SN Status Information IE in the DRB To Modify List IE is used e.g. in the change of gNB-CU-UP (chapter 8.9.5 of 38.401): (summary in below)

- The gNB-CU-CP request the source gNB-CU-UP to provide the PDCP UL/DL status by the PDCP SN Status Request IE in the Bearer Context Modification Request message

- the gNB-CU-UP provide the PDCP UL/DL status in PDCP SN Status Information IE in the Bearer Context Modification Response message;

- the gNB-CU-CP then provide to the target gNB-CU-UP the PDCP UL/DL status in PDCP SN Status Information IE in the Bearer Context Modification Request message.

Therefore this PDCP SN Status Information IE is used for the data forwarding purpose, and it is to provide to the target gNB-CU-UP in the Bearer Context Modification Request message.

For this reason, the solution 3 that use the existing PDCP SN Status Information IE does not fit into the use case of the issue discussed here, as the use case here we are discussing does not change the gNB-CU-UP.

Observation 1: The solution 3 that use the existing PDCP SN Status Information IE does not fit into the use case of the issue discussed here, as the use case here we are discussing does not change the gNB-CU-UP.

For the solution 1 and the Solution 4 received almost equally support from companies in the last RAN3#116e meeting.

The solution 4 (a dedicated IE to indicate reset PDCP COUNT) may be straightforward and cleaner that indicate directly the purpose i.e. to reset the PDCP COUNT. In comparison, the solution 1 needs to understand from the combination of the IEs presence in the signalling message. but solution 1 is nothing wrong and also acceptable solution.

Therefore, either solution 1 or solution 4 can be taken as the way to solve the issue.

Proposal : either solution 1 or solution 4 can be taken as the way to solve the issue to reduce the handover time in the use case i.e. the inter-gNB-DU handover, when target gNB-DU generated CellGroupConfig using full configuration, while gNB-CU decide to keep the same gNB-CU-UP that need to reset the PDCP COUNT of the existing DRB

Regarding which release to apply, since the inter-gNB-DU handover and full configuration chosen by the target gNB-DU is likely to be a normal use case, we are thinking to provide the solution from Rel-16.

We provide two set of CRs,

- 3. proposal

Proposal : either solution 1 or solution 4 can be taken as the way to solve the issue to reduce the handover time in the use case i.e. the inter-gNB-DU handover, when target gNB-DU generated CellGroupConfig using full configuration, while gNB-CU decide to keep the same gNB-CU-UP that need to reset the PDCP COUNT of the existing DRB.

References:

R3-213181 Handling of PDCP COUNT reset in CU-UP for inter-gNB-DU Handover

R3-223762 Summary of Offline Discussion on CB: # 57_PDCPCount