3GPP TSG-RAN WG3 #117-e	R3-224291

# Online, 15  – 24 August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 37.483 | CR | 0025 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Handling of PDCP COUNT reset in CU-UP for inter-gNB-DU Handover |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, ZTE, Huawei, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CPUP_Split-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2022-08-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the inter-gNB-DU handover when CellGroupConfig is generated using full configuration by the target gNB-DU, while eNB-CU decide to keep the same gNB-CU-UP that need to reset the PDCP COUNT of the existing DRBs, two time of the signalling procedus is needed.i.e. the E1AP: Bearer Context Modification procedures (first to release DRBs + adding the same DRBs, second to give Target DU’s DL TNL address), this takes longer time to complete the whole hanover procedure. An optimal way to do is similar to the intra-cell handover case that the gNB-CU-CP indicate to the gNB-CU-UP to release of DRBs and establish the same DRBs by the PDU Session Resource To Modify List IE in one BEARER CONTEXT MODIFICATION REQUEST message. However in the current DRB To Setup List IE within the PDU Session Resource To Modify List IE has no way to indicate new DL TNL of the target DU to the gNB-CU-UP, i.e. currently there is no DL UP Parameter IE in the DRB To Setup List IE. Further, If the same DRB ID exists in both DRB to Setup List IE and DRB to Remove List IE of the PDU Session Resource To Modify List IE, the gNB-CU-UP shall not handle this as abnormal condition but shall process the DRB to remove first then process DRB to setup. |  |  |  |  |  |  |  |  |
| Summary of change: |  | DL UP Parameter IE is added in the DRB To Setup List IE within PDU Session Resource To Modify List IE of the BEARER CONTEXT MODIFICATION REQUEST message. Adding text in procedure text. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because the correction only related with Bearer Context Modification procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to do the PDCP COUNT reset in a single Bearer Context Modification procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.2, 9.3.3.11, ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.463 CR 0702 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is the mirror change of the CR to the previous release of 38.463 This content of this CR is same as R3-223936 (RAN3#116e), only updated based on latest version of specification |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Change >>>>>>>>>>>>>>>>>>>>

8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT MODIFICATION REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to modify the bearer context, it replies to the gNB-CU-CP with the BEARER CONTEXT MODIFICATION RESPONSE message.

The gNB-CU-UP shall report to the gNB-CU-CP, in the BEARER CONTEXT MODIFICATION RESPONSE message, the result for all the requested resources in the following way:

For E-UTRAN:

-	A list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	A list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	A list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	A list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

For NG-RAN:

-	A list of PDU Session Resources which are successfully established shall be included in the PDU Session Resource Setup List IE;

-	A list of PDU Session Resources which failed to be established shall be included in the PDU Session Resource Failed List IE;

-	A list of PDU Session Resources which are successfully modified shall be included in the PDU Session Resource Modified List IE;

-	A list of PDU Session Resources which failed to be modified shall be included in the PDU Session Resource Failed To Modify List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which are successfully established shall be included in the DRB Setup List IE;

-	For each successfully established or modified PDU Session Resource, a list of DRBs which failed to be established shall be included in the DRB Failed List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which are successfully modified shall be included in the DRB Modified List IE;

-	For each successfully modified PDU Session Resource, a list of DRBs which failed to be modified shall be included in the DRB Failed To Modify List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which are successfully established shall be included in the Flow Setup List IE;

-	For each successfully established or modified DRB, a list of QoS Flows which failed to be established shall be included in the Flow Failed List IE;

When the gNB-CU-UP reports the unsuccessful establishment of a PDU Session Resource, DRB or QoS Flow the cause value should be precise enough to enable the gNB-CU-CP to know the reason for the unsuccessful establishment.

If the Security Information IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the UE DL Aggregate Maximum Bit Rate IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the UE DL Maximum Integrity Protected Data Rate IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Bearer Context Status Change IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall consider the UE RRC state and act as specified in TS 38.401 [2]. If the Bearer Context Status Change IE is set to "ResumeforSDT", the gNB-CU-UP shall consider that DRBs configured with SDT are resumed only and the other DRBs remain suspended.

If SDT Continue ROHC IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message and the value is set to “true”, the gNB-CU-UP shall, if supported, continue the ROHC for the SDT bearers for the UE.

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDU Session Data Forwarding Information IE is included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, consider that data forwarding is applicable for the indicated QoS flows for the concerned PDU session.

If the PDCP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information, except for the PDCP SN UL Size IE, the PDCP SN DL Size IE and the RLC mode IE which shall be ignored.

If the E-UTRAN QoS IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the PDCP SN Status Request IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall act as specified in TS 38.401 [2] and include the UL COUNT Value IE and the DL COUNT Value IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDCP SN Status Information IE is contained in the DRB To Setup List IE or the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the DL UP Parameters IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the same DRB ID exists in both DRB to Setup List E and DRB to Remove List IE in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall process the DRB to remove first then process DRB to setup. And if the DL UP Parameter IE is contained within the DRB To Setup List IE for a DRB of the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, configure the corresponding information.

If the Cell Group To Add IE or the Cell Group To Modify IE or the Cell Group To Remove IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall add or modify or remove the corresponding cell group.

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall replace the information in the UE context and use it when enforcing downlink traffic policing for the non GBR QoS flows for the concerned UE, as specified in TS 23.501 [20].

If the PDU Session Resource DL Aggregate Maximum Bit Rate IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the SDAP Configuration IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

If the Flow Mapping Information IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall update the corresponding information.

For each requested DRB, if the PDCP Duplication IE or Additional PDCP duplication Information IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, then the gNB-CU-CP shall include two or more UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION REQUEST message, and the gNB-CU-UP shall, if supported, also include two or more UP Transport Layer Information IEs in the BEARER CONTEXT MODIFICATION RESPONSE message to support packet duplication. If only one cell group is included in the Cell Group Information IE for the concerned DRB, then the gNB-CU-UP shall consider that the first UP Transport Layer Information IE of these UP Transport Layer Information IEs is for the primary path. If more than one cell group is included in the Cell Group Information IE, then the gNB-CU-UP shall consider that the number of duplication tunnels for each cell group is indicated by the Number of tunnels IE, and that the first UP Transport Layer Information IE for each cell group is for the primary path or the split secondary path.

For a certain DRB which was allocated with two or more GTP-U tunnels, if such DRB is modified and given one GTP-U tunnel via the Bearer Context Modification (gNB-CU-CP initiated) procedure, i.e. only one UP Transport Layer Information per Cell Group ID is present in DL UP Parameters IE for the concerned DRB, then the gNB-CU-UP shall consider that PDCP duplication is deconfigured for this DRB. If such Bearer Context Modification (gNB-CU-CP initiated) procedure occurs, the Duplication Activation IE shall not be included for the concerned DRB.

If the New UL TNL Information Required IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the new UP Transport Layer Information in the BEARER CONTEXT MODIFICATION RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE or the Security Indication Modify IE is included in the PDU Session Resource To Modify List IE of the BEARER CONTEXT MODIFICATION REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "preferred", then the gNB-CU-UP should, if supported, perform user plane integrity protection or ciphering, respectively, for the concerned PDU session and shall notify whether it performed the user plane integrity protection or ciphering by including the Integrity Protection Result IE or Confidentiality Protection Result IE, respectively, in the PDU Session Resource Setup List IE or the PDU Session Resource Modified List IE of the BEARER CONTEXT MODIFICATION RESPONSE message.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE or the Security Indication Modify IE is included in the PDU Session Resource To Modify List IE of the BEARER CONTEXT MODIFICATION REQUEST message, and the Integrity Protection Indication IE or Confidentiality Protection Indication IE is set to "required", then the gNB-CU-UP shall perform user plane integrity protection or ciphering, respectively, for the concerned PDU Session. If the gNB-CU-UP cannot perform the user plane integrity protection or ciphering, it shall reject the setup of the PDU Session Resources with an appropriate cause value.

For each PDU session for which the Security Indication IE is included in the PDU Session Resource To Setup List IE or the Security Indication Modify IE is included in the PDU Session Resource To Modify List IE of the BEARER CONTEXT MODIFICATION REQUEST message:

-	if the Integrity Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane integrity protection for the concerned PDU session;

-	if the Confidentiality Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane ciphering for the concerned PDU session.

For E-UTRAN:

-	For each DRB for which the Security Indication IE is included in the DRB To Setup List IE of the BEARER CONTEXT MODIFICATION REQUEST message, and the Integrity Protection Indication IE is set to "preferred", then the gNB-CU-UP should, if supported, perform user plane integrity protection for the concerned DRB and notify whether it performed the user plane integrity protection by including the Integrity Protection Result IE in the DRB Setup List IE of the BEARER CONTEXT MODIFICATION RESPONSE message.

-	For each DRB for which the Security Indication IE is included in the DRB To Setup List IE of the BEARER CONTEXT MODIFICATION REQUEST message, and the Integrity Protection Indication IE is set to "required", then the gNB-CU-UP shall, if supported, perform user plane integrity protection for the concerned DRB. If the gNB-CU-UP cannot perform the user plane integrity protection, it shall reject the setup of the DRB with an appropriate cause value.

-	For each DRB for which the Security Indication IE is included in the DRB To Setup List IE of the BEARER CONTEXT MODIFICATION REQUEST message and the Integrity Protection Indication IE is set to "not needed", then the gNB-CU-UP shall not perform user plane integrity protection for the concerned DRB.

For each PDU Session Resource, if the Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message and the Common Network Instance IE is not included, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Common Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource as specified in TS 23.501 [20].

For each PDU session, if the Redundant NG UL UP Transport Layer Information IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, include the Redundant NG DL UP Transport Layer Information IE in the PDU Session Resource Setup List IE or the PDU Session Resource Modified List IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the Redundant Common Network Instance IE is included in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, use it when selecting transport network resource for the redundant transmission as specified in TS 23.501 [20].

For each PDU session for which the Redundant QoS Flow Indicator IE is included in QoS Flows Information To Be Setup IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if support, shall store and use it as specified in TS 23.501 [20].

For each PDU session, if the Redundant QoS Flow Indicator IE is set to false for all QoS flows, the gNB-CU-UP shall, if supported, stop the redundant transmission and release the redundant tunnel for the concerned PDU session as specified in TS 23.501 [20].

If the QoS Flow Mapping Indication IE is contained in the QoS Flow QoS Parameters List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, replace any previously received value and take it into account that only the uplink or downlink QoS flow is mapped to the DRB.

If the Data Discard Required IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message and the value is set to “Required”, the gNB-CU-UP shall consider that a RAN Paging Failure occurred for that UE. The gNB-CU-UP shall discard the user plane data for that UE and consider that the bearer context is still suspended.

If UE Inactivity Timer IE or PDU session Inactivity Timer IE or DRB Inactivity Timer IE is contained in BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account when perform inactivity monitoring.

If the S-NSSAI IE is contained in the PDU Session Resource To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store the corresponding information and replace any existing information.

If the DRB QoS IE is contained within the DRB To Setup List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, take it into account for each DRB, as specified in TS 28.552 [22].

If the DRB QoS IE is contained within the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, replace any previously received value and take it into account for each DRB, as specifed in TS 28.552 [22].

If the gNB-DU-ID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store and replace any previous information received.

If the RAN UE ID IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store and replace any previous information received.

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message including Activity Notification Level IE and its value does not match the current bearer context, the gNB-CU-UP shall ignore the Activity Notification Level IE and also the requested modification of inactivity timer.

For each successfully established DRB, the gNB-CU-UP shall provide, in the respective UL UP Parameters IE of the BEARER CONTEXT MODIFICATION RESPONSE, one UL UP Transport Layer Information Item per cell group entry contained in the respective Cell Group Information IE of the BEARER CONTEXT MODIFICATION REQUEST message.

If the Old QoS Flow List - UL End Marker expected IE is included in the PDU Session Resource To Modify List IE of the BEARER CONTEXT MODIFICATION REQUEST message for a DRB to be modified, the gNB-CU-UP shall consider that the source NG-RAN node has initiated QoS flow re-mapping and has not yet received SDAP end markers, as described in TS 38.300 [8]. The gNB-CU-UP shall consider that the Old QoS Flow List - UL End Marker expected  IE only contains UL QoS flow information for QoS flows for which no SDAP end marker has been yet received on the source side.

For EN-DC, if the Subscriber Profile ID for RAT/Frequency priority IE is included in the BEARER CONTEXT MODIFICATION REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25]. If the Additional RRM Policy Index IE is included in the BEARER CONTEXT MODIFICATION REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25].

If there is at least one DRB removed by the gNB-CU-UP, the gNB-CU-UP shall, if supported, include the Retainability Measurements Information IE in the BEARER CONTEXT MODIFICATION RESPONSE message, providing information on the removed DRB(s) for retainability measurements in the gNB-CU-CP, as described in TS 32.425 [26] and TS 28.552 [22].

If the TSC Traffic Characteristics IE is included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, take into account the corresponding information received in the TSC Traffic Characteristics IE.

For each QoS flow whose DRB has been successfully established or modified and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [20]. If the QoS Monitoring Reporting Frequency IE was included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall store this information, and, if supported, use it for RAN part delay reporting.

For each requested DRB, if the QoS Mapping Information IE is contained in the DL UP Parameters IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall use it to set DSCP and/or flow label fields in the downlink IP packets which are transmitted through the GTP tunnels indicated by the UP Transport Layer Information IE. The Diffserv code point (DSCP) marking is performed as specified in TS 38.474 [28].

If the Early Forwarding COUNT Request IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall act as specified in TS 38.401 [2] and include the requested FIRST DL COUNT Value IE or DISCARD DL COUNT Value IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the Early Forwarding COUNT Information IE is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account and act as specified in TS 38.401 [2].

If the Ignore Mapping Rule Indication IE is contained within the DRB To Setup List IE for a DRB in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, ignore the QoS flow mapping information indicated by the QoS Flows Information To Be Setup IE for the concerned DRB.

If the DAPS Request Information IE is included for a DRB to be modified in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall consider that the request concerns a DAPS handover for that DRB and, if admitted, act as specified in TS 38.300 [4].

If the Early Data Forwarding Indicator IE set to “stop” is contained in the DRB To Modify List IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported and if already initiated, stop the early data forwarding for the concerned DRB. If the DRB Data forwarding information IE containing the DL Data Forwarding IE is included together in the DRB To Modify List IE, the gNB-CU-UP shall consider that the stop is only for the early data forwarding initiated toward that forwarding TNL.

If the MDT Polluted Measurement Indicator IE is included in the BEARER CONTEXT MODIFICATION REQUEST, the gNB-CU-UP shall take this information into account as specified in TS 38.401 [2].

If the UE Slice Maximum Bit Rate List IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, store and replace the previously provided UE Slice Maximum Bit Rate List by the received UE Slice Maximum Bit Rate List in the UE context, and use the received UE Slice Maximum Bit Rate List for the downlink traffic policing for each concerned slice as specified in TS 23.501 [20].

If the SCG Activation Status IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall take it into account when handling DL data transfer as specified in TS 37.340 [19].

If the UDC parameters IE is included in the PDCP Configuration IE in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, take these parameters into account to perform appropriate uplink data compression for the concerned DRB.

If the Data Forwarding Source IP Address IE is included in the DRB To Setup Modification List E-UTRAN IE or in the QoS Flow Level QoS Parameters IE within the PDU Session Resource To Setup Modification List IE and the PDU Session Resource To Modify List IE contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, store this information in the UE context and use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the Data Forwarding Source IP Address IE is included in the DRB Setup Modification List E-UTRAN IE or in the Flow Setup List IE within the PDU Session Resource Setup Modification List IE and the PDU Session Resource Modified List IE of the BEARER CONTEXT MODIFICATION RESPONSE message, the gNB-CU-CP shall, if supported, store this information in the UE context and use it as part of its ACL functionality configuration actions, if such ACL functionality is deployed.

If the Management Based MDT PLMN Modification List IE is contained in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, overwrite any previously stored Management Based MDT PLMN List information in the UE context and use the received information to determine subsequent selection of the UE for management based MDT defined in TS 32.422 [24].

Interaction with the Bearer Context Modification (gNB-CU-CP initiated)

If the BEARER CONTEXT MODIFICATION REQUEST message includes for a DRB in the DRB To Modify List IE the PDCP SN Status Request IE set to “requested” and if the gNB-CU-UP has not yet received a SDAP end marker packet for a QoS flow which has been previously re-configured to another DRB by means of a gNB-CU-CP initiated Bearer Context Modification procedure, the gNB-CU-UP shall includes the QoS Flow Identifier of that QoS flow in the Old QoS Flow List - UL End Marker expected IE in the PDU Session Resource Modified List IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

8.3.2.3	Unsuccessful Operation

Figure 8.3.2.3-1: Bearer Context Modification procedure: Unsuccessful Operation.

If the gNB-CU-UP cannot successfully perform any of the requested bearer context modifications, or cannot handle SCG with the indicated activated or deactivated status, it shall respond with a BEARER CONTEXT MODIFICATION FAILURE message and appropriate cause value.

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message containing the Security Indication Modify IE in the PDU Session Resource To Modify List IE for a PDU session that may result in the change of security status that has been applied but the DRBs that have been established for that PDU session are not requested to be released via the DRB To Remove List IEs as specified in TS 38.331 [10], then the gNB-CU-UP shall respond with a BEARER CONTEXT MODIFICATION FAILURE message and appropriate cause value.

8.3.2.4	Abnormal Conditions

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message containing a E-UTRAN QoS IE in the DRB To Setup List or the DRB To Modify List IE for a GBR QoS DRB but where the GBR QoS Information IE is not present, the gNB-CU-UP shall report the addition or the modification of the corresponding DRB as failed in the DRB Failed List IE or the DRB Failed To Modify List IE of the BEARER CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

If the gNB-CU-UP receives a BEARER CONTEXT MODIFICATION REQUEST message containing a QoS Flow Level QoS Parameters IE in the PDU Session Resource To Setup List IE or the PDU Session Resource To Modify List IE for a GBR QoS Flow but where the GBR QoS Flow Information IE is not present, the gNB-CU-UP shall report the addition or the modification of the corresponding QoS Flow as failed in the corresponding  Flow Failed List IE of the BEARER CONTEXT MODIFICATION RESPONSE message with an appropriate cause value.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.3.11	PDU Session Resource To Modify List

This IE contains PDU session resource to modify related information used at Bearer Context Modification Request

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| PDU Session Resource To Modify Item |  | 1..<maxnoofPDUSessionResource> |  |  | - | - |
| >PDU Session ID | M |  | 9.3.1.21 |  | - | - |
| >Security Indication | O |  | 9.3.1.23 | This IE is not used in this release. | - | - |
| >PDU Session Resource DL Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.20 |  | - | - |
| >NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >PDU Session Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >PDU Session Data Forwarding Information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >PDU Session Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to PDU Session. | - | - |
| >Network Instance | O |  | 9.3.1.62 | This IE is ignored if the Common Network Instance IE is included. | YES | ignore |
| >Common Network Instance | O |  | 9.3.1.66 |  | YES | ignore |
| >DRB To Setup List |  | 0..1 |  |  | - | - |
| >>DRB To Setup Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | M |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | M |  | 9.3.1.38 |  | - | - |
| >>>Cell Group Information | M |  | 9.3.1.11 |  | - | - |
| >>>QoS Flow Information To Be Setup | M |  | QoS Flow QoS Parameters List 9.3.1.25 |  | - | - |
| >>>DRB Data Forwarding Information Request | O |  | Data Forwarding Information Request 9.3.2.5 | Requesting forwarding information from the target gNB-CU-UP. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status at setup after Resume to the target gNB-CU-UP. | - | - |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >>>DAPS Request Information | O |  | 9.3.1.91 | This IE is not used in this version of the specification | YES | ignore |
| >>>Ignore Mapping Rule Indication | O |  | ENUMERATED (True, …) | Included if the QoS flow mapping rule for the DRB has not been decided by gNB-CU-CP. | YES | reject |
| >>>QoS Flows Remapping | O |  | ENUMERATED (update, source configuration, …) | This IE is not used in this version of the specification. | YES | reject |
| >>>SDT Indicator Setup | O |  | ENUMERATED (true, …) | Indicates that the DRB is for SDT. | YES | reject |
| >>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  | YES | ignore |
| >DRB To Modify List |  | 0.. 1 |  |  | - | - |
| >>DRB To Modify Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >>>SDAP Configuration | O |  | 9.3.1.39 |  | - | - |
| >>>PDCP Configuration | O |  | 9.3.1.38 |  | - | - |
| >>>DRB Data forwarding information | O |  | Data Forwarding Information 9.3.2.6 | Providing forwarding information to the source gNB-CU-UP. | - | - |
| >>>PDCP SN Status Request | O |  | ENUMERATED (requested, …) | The gNB-CU-CP requests the gNB-CU-UP to provide the PDCP SN Status in the response message. | - | - |
| >>>PDCP SN Status Information | O |  | 9.3.1.58 | Provides the PDCP SN Status to the target gNB-CU-UP. | - | - |
| >>>DL UP Parameters | O |  | UP Parameters 9.3.1.13 |  | - | - |
| >>>Cell Group To Add | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Modify | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Cell Group To Remove | O |  | Cell Group Information 9.3.1.11 |  | - | - |
| >>>Flow Mapping Information | O |  | QoS Flow QoS Parameters List 9.3.1.25 | Overrides previous mapping information. | - | - |
| >>>DRB Inactivity Timer | O |  | Inactivity Timer 9.3.1.54 | Included if the Activity Notification Level is set to DRB. | - | - |
| >>>Old QoS Flow List - UL End Marker expected | O |  | QoS Flow List 9.3.1.12 | Indicates that the source NG-RAN node has initiated QoS flow re-mapping and has not yet received SDAP end markers, as described in TS 38.300 [8]. | YES | reject |
| >>>DRB QoS | O |  | 9.3.1.26 | Indicates the DRB QoS when more than one QoS Flow is mapped to the DRB | YES | ignore |
| >>>Early Forwarding COUNT Request | O |  | ENUMERATED (First DL count, DL discarding, …) | Requests early data forwarding information from the source gNB-CU-UP | YES | reject |
| >>>Early Forwarding COUNT Information | O |  | 9.3.1.92 | Provides early data forwarding information to the target gNB-CU-UP. | YES | reject |
| >>>DAPS Request Information | O |  | 9.3.1.91 | Used to request intra-gNB-CU-UP DAPS HO | YES | ignore |
| >>>Early Data Forwarding Indicator | O |  | ENUMERATED (stop, …) |  | YES | ignore |
| >>>SDT Indicator Modify | O |  | ENUMERATED (true, false, …) | Indicates that the DRB is for SDT or not. | YES | reject |
| >DRB To Remove List |  | 0.. 1 |  |  | - | - |
| >>DRB To Remove Item |  | 1..<maxnoofDRBs> |  |  | - | - |
| >>>DRB ID | M |  | 9.3.1.16 |  | - | - |
| >S-NSSAI | O |  | 9.3.1.9 |  | YES | reject |
| >Redundant NG UL UP Transport Layer Information | O |  | UP Transport Layer Information 9.3.2.1 |  | YES | ignore |
| >Redundant Common Network Instance | O |  | Common Network Instance 9.3.1.66 |  | YES | ignore |
| >Data Forwarding to E-UTRAN Information List |  | 0.. 1 |  | Contains a list of DL Data Forwarding tunnels and the associated QoS Flows to be forwarded on each tunnel | YES | ignore |
| >>Data Forwarding to E-UTRAN Information List Item |  | 1..<maxnoofDataForwardingTunneltoE-UTRAN> |  |  | - | - |
| >>>Data forwarding tunnel information | M |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| >>>QoS Flows to be forwarded List |  | 1 |  |  | - | - |
| >>>>QoS Flows to be forwarded Item |  | 1..<maxnoofQoSflows> |  |  | - | - |
| >>>>>QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.24 |  | - | - |
| >Security Indication Modify | O |  | Security Indication 9.3.1.23 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofDRBs | Maximum no. of DRBs for a UE. Value is 32. |
| maxnoofPDUSessionResource | Maximum no. of PDU Sessions for a UE. Value is 256. |
| maxnoofDataForwardingTunneltoE-UTRAN | Maximum no. of Data Forwarding Tunnels to E-UTRAN for a UE. Value is 256. |
| maxnoofQoSflows | Maximum no. of QoS flows in a PDU Session. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-QoSMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-QoSMonitoringDisabled,

id-PDCP-StatusReportIndication,

id-RedundantCommonNetworkInstance,

id-redundant-nG-UL-UP-TNL-Information,

id-redundant-nG-DL-UP-TNL-Information,

id-RedundantQosFlowIndicator,

id-TSCTrafficCharacteristics,

id-ExtendedPacketDelayBudget,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-AdditionalPDCPduplicationInformation,

id-RedundantPDUSessionInformation,

id-RedundantPDUSessionInformation-used,

id-QoS-Mapping-Information,

id-MDTConfiguration,

id-TraceCollectionEntityURI,

id-EHC-Parameters,

id-DAPSRequestInfo,

id-EarlyForwardingCOUNTReq,

id-EarlyForwardingCOUNTInfo,

id-AlternativeQoSParaSetList,

id-MCG-OfferedGBRQoSFlowInfo,

id-Number-of-tunnels,

id-DataForwardingtoE-UTRANInformationList,

id-DataForwardingtoNG-RANQoSFlowInformationList,

id-MaxCIDEHCDL,

id-ignoreMappingRuleIndication,

id-EarlyDataForwardingIndicator,

id-QoSFlowsDRBRemapping,

id-SecurityIndicationModify,

id-DataForwardingSourceIPAddress,

id-M4ReportAmount,

id-M6ReportAmount,

id-M7ReportAmount,

id-PDUSession-PairID,

id-SurvivalTime,

id-UDC-Parameters,

id-SecurityIndication,

id-SecurityResult,

id-SDTindicatorSetup,

id-SDTindicatorMod,

id-DiscardTimerExtended,

id-DL-UP-Parameter,

maxnoofMBSAreaSessionIDs,

maxnoofSharedNG-UTerminations,

maxnoofMRBs,

maxnoofMBSSessionIDs,

maxnoofQoSParaSets,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofSPLMNs,

maxnoofMDTPLMNs,

maxnoofExtSliceItems,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofExtNRCGI,

maxnoofECGI,

maxnoofSMBRValues

FROM E1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM E1AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-SingleContainer{},

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers;

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

DRB-To-Setup-List-NG-RAN	::= SEQUENCE (SIZE(1.. maxnoofDRBs)) OF DRB-To-Setup-Item-NG-RAN

DRB-To-Setup-Item-NG-RAN	::=	SEQUENCE {

dRB-ID										DRB-ID,

sDAP-Configuration							SDAP-Configuration,

pDCP-Configuration							PDCP-Configuration,

cell-Group-Information						Cell-Group-Information,

qos-flow-Information-To-Be-Setup					QoS-Flow-QoS-Parameter-List,

dRB-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

dRB-Inactivity-Timer						Inactivity-Timer	OPTIONAL,

pDCP-SN-Status-Information								PDCP-SN-Status-Information								OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { DRB-To-Setup-Item-NG-RAN-ExtIEs } }	OPTIONAL,

...

}

DRB-To-Setup-Item-NG-RAN-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DRB-QoS						CRITICALITY ignore	EXTENSION QoSFlowLevelQoSParameters		PRESENCE optional}|

{ID id-DAPSRequestInfo				CRITICALITY ignore	EXTENSION DAPSRequestInfo					PRESENCE optional}|

{ID id-ignoreMappingRuleIndication	CRITICALITY reject	EXTENSION IgnoreMappingRuleIndication	PRESENCE optional}|

{ID id-QoSFlowsDRBRemapping			CRITICALITY reject	EXTENSION QoS-Flows-DRB-Remapping			PRESENCE optional}|

{ID id-SDTindicatorSetup			CRITICALITY reject	EXTENSION SDTindicatorSetup					PRESENCE optional}|

{ID id-DL-UP-Parameter			CRITICALITY ignore	EXTENSION 	UP-Parameters								PRESENCE optional	},

...

}

<<<<<<<<<<<<<<<<<<<< Next part no Change (only showing the corresponding place, for convenience)>>>>>>>>>>>>>>>>>>>>

PDU-Session-Resource-To-Modify-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Modify-Item

PDU-Session-Resource-To-Modify-Item	::=	SEQUENCE {

pDU-Session-ID										PDU-Session-ID,

securityIndication									SecurityIndication						OPTIONAL,

pDU-Session-Resource-DL-AMBR						BitRate									OPTIONAL,

nG-UL-UP-TNL-Information							UP-TNL-Information						OPTIONAL,

pDU-Session-Data-Forwarding-Information-Request		Data-Forwarding-Information-Request		OPTIONAL,

pDU-Session-Data-Forwarding-Information	Data-Forwarding-Information	OPTIONAL,

pDU-Session-Inactivity-Timer						Inactivity-Timer						OPTIONAL,

networkInstance										NetworkInstance							OPTIONAL,

dRB-To-Setup-List-NG-RAN							DRB-To-Setup-List-NG-RAN				OPTIONAL,

dRB-To-Modify-List-NG-RAN							DRB-To-Modify-List-NG-RAN				OPTIONAL,

dRB-To-Remove-List-NG-RAN					DRB-To-Remove-List-NG-RAN		OPTIONAL,

iE-Extensions										ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Modify-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Modify-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-SNSSAI										CRITICALITY reject	EXTENSION SNSSAI												PRESENCE optional}|

{ID id-CommonNetworkInstance						CRITICALITY ignore	EXTENSION CommonNetworkInstance								PRESENCE optional	}|

{ID id-redundant-nG-UL-UP-TNL-Information			CRITICALITY ignore	EXTENSION 	UP-TNL-Information								PRESENCE optional	}|

{ID id-RedundantCommonNetworkInstance				CRITICALITY ignore	EXTENSION 	CommonNetworkInstance							PRESENCE optional	}|

{ID id-DataForwardingtoE-UTRANInformationList		CRITICALITY ignore	EXTENSION 	DataForwardingtoE-UTRANInformationList	PRESENCE optional	}|

{ID id-SecurityIndicationModify						CRITICALITY ignore	EXTENSION 	SecurityIndication								PRESENCE optional	},

...

}

PDU-Session-Resource-To-Remove-List	::= SEQUENCE (SIZE(1.. maxnoofPDUSessionResource)) OF PDU-Session-Resource-To-Remove-Item

PDU-Session-Resource-To-Remove-Item	::=	SEQUENCE {

pDU-Session-ID							PDU-Session-ID,

iE-Extensions							ProtocolExtensionContainer	{ { PDU-Session-Resource-To-Remove-Item-ExtIEs } }	OPTIONAL,

...

}

PDU-Session-Resource-To-Remove-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-Cause		CRITICALITY ignore	EXTENSION Cause		PRESENCE optional},

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

#### -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

id-gNB-CU-UP-E1Setup											ProcedureCode ::= 3

id-gNB-CU-CP-E1Setup											ProcedureCode ::= 4

id-gNB-CU-UP-ConfigurationUpdate								ProcedureCode ::= 5

id-gNB-CU-CP-ConfigurationUpdate								ProcedureCode ::= 6

id-e1Release													ProcedureCode ::= 7

id-bearerContextSetup											ProcedureCode ::= 8

id-bearerContextModification									ProcedureCode ::= 9

id-bearerContextModificationRequired							ProcedureCode ::= 10

id-bearerContextRelease											ProcedureCode ::= 11

id-bearerContextReleaseRequest									ProcedureCode ::= 12

id-bearerContextInactivityNotification							ProcedureCode ::= 13

id-dLDataNotification											ProcedureCode ::= 14

id-dataUsageReport												ProcedureCode ::= 15

id-gNB-CU-UP-CounterCheck										ProcedureCode ::= 16

id-gNB-CU-UP-StatusIndication									ProcedureCode ::= 17

id-uLDataNotification											ProcedureCode ::= 18

id-mRDC-DataUsageReport											ProcedureCode ::= 19

id-TraceStart													ProcedureCode ::= 20

id-DeactivateTrace												ProcedureCode ::= 21

id-resourceStatusReportingInitiation							ProcedureCode ::= 22

id-resourceStatusReporting										ProcedureCode ::= 23

id-iAB-UPTNLAddressUpdate										ProcedureCode ::= 24

id-CellTrafficTrace												ProcedureCode ::= 25

id-earlyForwardingSNTransfer									ProcedureCode ::= 26

id-gNB-CU-CPMeasurementResultsInformation						ProcedureCode ::= 27

id-iABPSKNotification											ProcedureCode ::= 28

id-BCBearerContextSetup											ProcedureCode ::= 29

id-BCBearerContextModification									ProcedureCode ::= 30

id-BCBearerContextModificationRequired							ProcedureCode ::= 31

id-BCBearerContextRelease										ProcedureCode ::= 32

id-BCBearerContextReleaseRequest								ProcedureCode ::= 33

id-MCBearerContextSetup											ProcedureCode ::= 34

id-MCBearerContextModification									ProcedureCode ::= 35

id-MCBearerContextModificationRequired							ProcedureCode ::= 36

id-MCBearerContextRelease										ProcedureCode ::= 37

id-MCBearerContextReleaseRequest								ProcedureCode ::= 38

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofTLAs									INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofTNLAddresses							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofExtNRCGI 							INTEGER	::= 16384

maxnoofPSKs									INTEGER	::= 256

maxnoofECGI 								INTEGER	::= 512

maxnoofSMBRValues							INTEGER	::= 8

maxnoofMBSAreaSessionIDs					INTEGER ::= 256

maxnoofSharedNG-UTerminations				INTEGER ::= 8

maxnoofMRBs									INTEGER ::= 32

maxnoofMBSSessionIDs						INTEGER ::= 512

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired											ProtocolIE-ID ::= 70

id-OldQoSFlowMap-ULendmarkerexpected							ProtocolIE-ID ::= 71

id-DRB-QoS														ProtocolIE-ID ::= 72

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID														ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance										ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication										ProtocolIE-ID ::= 80

id-TraceActivation												ProtocolIE-ID ::= 81

id-TraceID														ProtocolIE-ID ::= 82

id-SubscriberProfileIDforRFP									ProtocolIE-ID ::= 83

id-AdditionalRRMPriorityIndex									ProtocolIE-ID ::= 84

id-RetainabilityMeasurementsInfo								ProtocolIE-ID ::= 85

id-Transport-Layer-Address-Info									ProtocolIE-ID ::= 86

id-QoSMonitoringRequest											ProtocolIE-ID ::= 87

id-PDCP-StatusReportIndication									ProtocolIE-ID ::= 88

id-gNB-CU-CP-Measurement-ID										ProtocolIE-ID ::= 89

id-gNB-CU-UP-Measurement-ID										ProtocolIE-ID ::= 90

id-RegistrationRequest											ProtocolIE-ID ::= 91

id-ReportCharacteristics										ProtocolIE-ID ::= 92

id-ReportingPeriodicity											ProtocolIE-ID ::= 93

id-TNL-AvailableCapacityIndicator								ProtocolIE-ID ::= 94

id-HW-CapacityIndicator											ProtocolIE-ID ::= 95

id-RedundantCommonNetworkInstance								ProtocolIE-ID ::= 96

id-redundant-nG-UL-UP-TNL-Information							ProtocolIE-ID ::= 97

id-redundant-nG-DL-UP-TNL-Information							ProtocolIE-ID ::= 98

id-RedundantQosFlowIndicator									ProtocolIE-ID ::= 99

id-TSCTrafficCharacteristics									ProtocolIE-ID ::= 100

id-CNPacketDelayBudgetDownlink									ProtocolIE-ID ::= 101

id-CNPacketDelayBudgetUplink									ProtocolIE-ID ::= 102

id-ExtendedPacketDelayBudget									ProtocolIE-ID ::= 103

id-AdditionalPDCPduplicationInformation							ProtocolIE-ID ::= 104

id-RedundantPDUSessionInformation								ProtocolIE-ID ::= 105

id-RedundantPDUSessionInformation-used							ProtocolIE-ID ::= 106

id-QoS-Mapping-Information										ProtocolIE-ID ::= 107

id-DLUPTNLAddressToUpdateList									ProtocolIE-ID ::= 108

id-ULUPTNLAddressToUpdateList									ProtocolIE-ID ::= 109

id-NPNSupportInfo												ProtocolIE-ID ::= 110

id-NPNContextInfo												ProtocolIE-ID ::= 111

id-MDTConfiguration												ProtocolIE-ID ::= 112

id-ManagementBasedMDTPLMNList									ProtocolIE-ID ::= 113

id-TraceCollectionEntityIPAddress								ProtocolIE-ID ::= 114

id-PrivacyIndicator												ProtocolIE-ID ::= 115

id-TraceCollectionEntityURI										ProtocolIE-ID ::= 116

id-URIaddress													ProtocolIE-ID ::= 117

id-EHC-Parameters												ProtocolIE-ID ::= 118

id-DRBs-Subject-To-Early-Forwarding-List						ProtocolIE-ID ::= 119

id-DAPSRequestInfo												ProtocolIE-ID ::= 120

id-CHOInitiation												ProtocolIE-ID ::= 121

id-EarlyForwardingCOUNTReq										ProtocolIE-ID ::= 122

id-EarlyForwardingCOUNTInfo										ProtocolIE-ID ::= 123

id-AlternativeQoSParaSetList									ProtocolIE-ID ::= 124

id-ExtendedSliceSupportList								ProtocolIE-ID ::= 125

id-MCG-OfferedGBRQoSFlowInfo									ProtocolIE-ID ::= 126

id-Number-of-tunnels											ProtocolIE-ID ::= 127

id-DRB-Measurement-Results-Information-List						ProtocolIE-ID ::= 128

id-Extended-GNB-CU-CP-Name										ProtocolIE-ID ::= 129

id-Extended-GNB-CU-UP-Name										ProtocolIE-ID ::= 130

id-DataForwardingtoE-UTRANInformationList						ProtocolIE-ID ::= 131

id-QosMonitoringReportingFrequency								ProtocolIE-ID ::= 132

id-QoSMonitoringDisabled										ProtocolIE-ID ::= 133

id-AdditionalHandoverInfo										ProtocolIE-ID ::= 134

id-Extended-NR-CGI-Support-List									ProtocolIE-ID ::= 135

id-DataForwardingtoNG-RANQoSFlowInformationList					ProtocolIE-ID ::= 136

id-MaxCIDEHCDL													ProtocolIE-ID ::= 137

id-ignoreMappingRuleIndication									ProtocolIE-ID ::= 138

id-DirectForwardingPathAvailability								ProtocolIE-ID ::= 139

id-EarlyDataForwardingIndicator									ProtocolIE-ID ::= 140

id-QoSFlowsDRBRemapping											ProtocolIE-ID ::= 141

id-DataForwardingSourceIPAddress								ProtocolIE-ID ::= 142

id-SecurityIndicationModify										ProtocolIE-ID ::= 143

id-IAB-Donor-CU-UPPSKInfo										ProtocolIE-ID ::= 144

id-ECGI-Support-List											ProtocolIE-ID ::= 145

id-MDTPollutedMeasurementIndicator								ProtocolIE-ID ::= 146

id-M4ReportAmount 												ProtocolIE-ID ::= 147

id-M6ReportAmount 												ProtocolIE-ID ::= 148

id-M7ReportAmount 												ProtocolIE-ID ::= 149

id-UESliceMaximumBitRateList									ProtocolIE-ID ::= 150

id-PDUSession-PairID											ProtocolIE-ID ::= 151

id-SurvivalTime													ProtocolIE-ID ::= 152

id-UDC-Parameters												ProtocolIE-ID ::= 153

id-SCGActivationStatus											ProtocolIE-ID ::= 154

id-GNB-CU-CP-MBS-E1AP-ID										ProtocolIE-ID ::= 155

id-GNB-CU-UP-MBS-E1AP-ID										ProtocolIE-ID ::= 156

id-GlobalMBSSessionID											ProtocolIE-ID ::= 157

id-BCBearerContextToSetup										ProtocolIE-ID ::= 158

id-BCBearerContextToSetupResponse								ProtocolIE-ID ::= 159

id-BCBearerContextToModify										ProtocolIE-ID ::= 160

id-BCBearerContextToModifyResponse								ProtocolIE-ID ::= 161

id-BCBearerContextToModifyRequired								ProtocolIE-ID ::= 162

id-BCBearerContextToModifyConfirm								ProtocolIE-ID ::= 163

id-MCBearerContextToSetup										ProtocolIE-ID ::= 164

id-MCBearerContextToSetupResponse								ProtocolIE-ID ::= 165

id-MCBearerContextToModify										ProtocolIE-ID ::= 166

id-MCBearerContextToModifyResponse								ProtocolIE-ID ::= 167

id-MCBearerContextToModifyRequired								ProtocolIE-ID ::= 168

id-MCBearerContextToModifyConfirm								ProtocolIE-ID ::= 169

id-MBSMulticastF1UContextDescriptor								ProtocolIE-ID ::= 170

id-gNB-CU-UP-MBS-Support-Info									ProtocolIE-ID ::= 171

id-SecurityIndication											ProtocolIE-ID ::= 172

id-SecurityResult												ProtocolIE-ID ::= 173

id-SDTContinueROHC												ProtocolIE-ID ::= 174

id-SDTindicatorSetup											ProtocolIE-ID ::= 175

id-SDTindicatorMod												ProtocolIE-ID ::= 176

id-DiscardTimerExtended											ProtocolIE-ID ::= 177

id-ManagementBasedMDTPLMNModificationList						ProtocolIE-ID ::= 178id-DL-UP-Parameter												ProtocolIE-ID ::= 1xx

END

-- ASN1STOP