3GPP TSG-RAN WG3 #117-e						      R3-224297

E-meeting, 15-24 August 2022

Agenda Item:  14.4

Source:       NEC

Title:		NR-DC with selective activation of the cell groups

Document for:	decision

Introduction

The Rel-18 WID Further NR Mobility Enhancements [1] has the following objective that may impact on RAN3 protocols:

- To specify mechanism and procedures of NR-DC with selective activation of the cell groups (at least for SCG) via L3 enhancements: - To allow subsequent cell group change after changing CG without reconfiguration and re-initiation of CPC/CPA [RAN2, RAN3, RAN4] Note 4: A harmonized RRC modelling approach for objectives 1 and 2 could be considered to minimize the workload in RAN2.

This contribution gives an initial analysis from RAN3 perspective.

Discussion

This selective activation of the cell groups is coming from the justification in the WID [1]:

In Rel-17 Conditional PSCell change (CPC)/Conditional PSCell addition (CPA), a CPC/CPA-configured UE has to release the CPC/CPA configurations when completing random access towards the target PSCell. Hence the UE doesn’t have a chance to perform subsequent CPC/CPA without prior CPC/CPA reconfiguration and re-initialization from the network. This will increase the delay for the cell change and increase the signaling overhead, especially in the case of frequent SCG changes when operating FR2. Therefore, MR-DC with selective activation of cell groups aims at enabling subsequent CPC/CPA after SCG change, without reconfiguration and re-initialization on the CPC/CPA preparation from the network. This results in a reduction of the signalling overhead and interrupting time for SCG change.

It can be understood from the justification that the selective activation of cell groups is to keep the previous prepared cell groups after the execution and completion of the PScell add/change from CPA/CPC, then these prepared cell groups can be used for subsequent SCG Change.

While this item is led by RAN2, and it may be beneficial to wait for RAN2 until an initial assumption is taken by RAN2, we give some analysis and observations from RAN3 perspective.

From RAN3 perspective, it may be important to see first that either MN or SN will take the decision to apply the selective activation of cell group. We see the following cases:

Case 1) CPA, the MN might prepare multiple candidate PScells towards multiple T-SNs, after the UE meet the condition and the PScell adding is completed, the other prepared candidate PScells in other T-SNs can be kept. For this CPA case, it may be too much if T-SN to take the decision because if multiple T-SNs were prepared, each T-SN may have different decision then this will complicate the handling. Therefore for the CPA case it may be beneficial the MN to decide to apply the selective activation of cell group instead of T-SN.

Case 2) MN initiated CPC, the MN might prepare multiple candidate PScells towards multiple T-SN, after the UE meet the condition and the PScell change is completed, the other prepared candidate PScells in other T-SNs and the source SN may be kept. Since this is the MN initiated, then it may be logical the MN to decide to apply the selective activation of cell group, while the S-SN may be able also to take the decision.

Case 3) SN initiated CPC, the S-SN might prepare multiple candidate PScells towards multiple T-SN, after the UE meet the condition and the PScell change is completed, the other prepared candidate PScells in other T-SNs and the source SN may be kept. This is the SN initiated, then it may be logical that the S-SN take the decision to apply the selective activation of cell group, while the MN may be able also to take the decision.

As can be seen from the analysis above, depend on the scenario (CPA, MN initiated CPC, SN initiated CPC) , either MN or SN can take the decision to apply the selective activation of cell group.

It is observed that:

Observation 1: it may be not persuaded to let the T-SN to take the decision whether to apply the selective activation of cell group.

Observation 2: for the CPA, it is the MN to take the decision whether to apply selective activation of cell group.

Observation 3: for the MN/SN initiated CPC, it is logical either the MN or the S-SN to take decision to apply the selective activation of cell group.

We will design our RAN3 signalling protocol to support the selective activation of the cell group, depend on the node who take the decision. Since in general the MN is the centralized node to control overall handling, and in order to have a common signalling design to support all cases, it may be beneficial that the MN to take the decision to apply the selective activation of cell groups.

conclusion

We give our initial view from RAN3 perspective for the NR-DC with selective activation of cell group, it may be beneficial for the MN to take the decision to apply the selective activation of cell group.

However, this is led by RAN2 and it is beneficial to wait first RAN2 reach some basic assumption, then RAN3 can have detail discussion.

References

[1] RP-221799, WID Further NR Mobility Enhancements