3GPP TSG-RAN3 Meeting #117-e	R3-224304

# Online, 15 - 24 August, 2022

Agenda Item:	17.2

Source: 	Qualcomm Incorporated

Title: 		XN Enhancements for NTN Mobility and Feeder Link Switch Over

Document for:	Discussion/Decision

# Introduction

Basic procedures needed for NTN mobility were specified in Release 17. However, there are few open issues with respect to NTN Cell Identifier over XN and NG interfaces and multiple TAC support for NTN cells over XN, which needs to be clarified. This contribution discusses the clarifications needed.

This contribution also discusses, supporting additional NTN information over XN for NTN mobility and feeder link switch over as Release 18 enhancements.

In Release 18 the WI includes the following objectives for NTN-NTN mobility and NTN-TN mobility. The highlighted bullets from the WI below are discussed in this paper.

This work considers existing methods from NR TN as well as outcome of Rel-17 NR NTN WI outcome as baseline for NTN-TN mobility. - Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2, RAN3, RAN4] - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4] - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3] - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4] Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

# Discussion

## Clarification on Cell Identifier over XN and NG

The Cell identifiers to be used for different NTN scenarios over NG and XN interface is specified in 38.300 Section 16.14.5.

On the Cell Identifier for Handover messages, the 38.300 Section 16.14.5 states the below.

TS 38.300 Section 16.14.5 “The Cell Identity included within the target identification of the handover messages allows identifying the correct target cell.”

However, it is not clear from the above text, if Mapped Cell ID or Uu Cell ID should be used for XN and NG based Handover message. In the similar context, it is also not clear, if the cell identifiers used for exchanging Served Cell Information and Neighbour Information via XN Setup procedure are Mapped Cell Id or Uu Cell ID.

Hence our proposal is to use Uu Cell ID for NG and XN based handover messages and in the Served cell and Neighbour cell information in XN Setup procedure to enable ANR and inter vendor operability. Based on the RAN3 decision clarification shall be provided in 38.300 Section 16.14.5.

Proposal 1: Uu Cell ID shall be used in XN and NG based Handover procedures for NTN.

Proposal 2: Uu Cell ID shall be used in Served Cell Information and Neighbour Information IEs in XN Setup procedure for NTN.

Proposal 3: NG-RAN Signalling Section 16.14.5 in TS 38.300 shall provide clarification on the Proposal 1 and 2.

## Clarification on TAC for NTN Cell Information exchanged via XN Setup Procedure

In Rel17 RAN3#112-e it was decided to exchange the NTN Cell information between neighbouring gNBs via XN procedures.

Serving/neighbor NTN cell information, if any, may be exchanged between gNBs via Xn.

NTN Cells are associated with more than one TAC since NTN cells covers a large area.

However, the Served Cell Information and Neighbour Information IEs over XN Setup procedure allows only a single TAC per Cell. For NTN cells, Served Cell Information and Neighbour Information IEs over XN Setup procedure shall allow multiple TACs per cell.

Proposal 4:  Served Cell Information and Neighbour Information IEs in both XN Setup procedure and Configuration Update procedure shall allow multiple TACs for NTN cells.

## NTN Cells Coverage Stop Time and Ephemeris Information over XN

In Rel17, it was decided that NTN cell related Satellite position and ephemeris information shall be available via OAM. The NTN cell coverage stop time and ephemeris information are essential for both CHO and HO to find and configure the UE with appropriate target cells. OAM option is available by default. However, in order to allow dynamicity of the NTN cells and inter vendor operability, our proposal is to share the NTN Coverage Stop Time and Ephemeris Information per NTN Cell via XN Setup and Configuration Update procedure in Release 18.

Proposal 5:  NTN Cell Coverage Stop Time and Ephemeris information shall be exchanged via XN Setup procedure and Config Update procedure between the neighbouring gNBs to assist NTN mobility and ANR in Release 18.

## Feeder Link Switch Over

Rel 17 supports both hard and soft Feeder Link Switch over. It was agreed in Release 17 that Feeder Link switch over related information shall be provided by OAM as shown below.

TS 38.300 Section 16.14.4.1 “Both hard and soft feeder link switch over are applicable to NTN.” TS 38.300 Section 16.14.7 “Additional information to enable gNB operation for feeder/service link switch overs.”

We would like to re-discuss if the XN based NTN Cell information exchange can be enhanced in Release 18 by providing basic information on Feeder Link Switch over.

The NTN Cell Coverage Stop Time and ephemeris information, if exchanged via XN as per proposal 5, will assist the neighboring gNBs to be aware of the next available target after the switch over. Hence if proposal 5 is acceptable, it will be useful for Feeder Link Switch over related information as well.

Along with Proposal 5, an explicit indication of Hard or soft feeder link switch over XN interface will assist the neighboring gNBs for timely handovers.

Proposal 6: Proposal 5 shall be applicable for Feeder Link Switch over scenario as well.

Proposal 7: Hard or Soft Feeder link Switch over indication shall be exchanged via XN Setup procedure and Config Update procedure between the neighbouring gNBs.

The following options may be available at the network to handle a hard feeder link switch over.

- CHO with time and location-based measurement events

- RRC Release with Redirection

- RRC Reestablishment

While CHO and RRC Release with Redirection can be planned by the source network based on the Feeder Link switch over information available, RRC Re-establishment is the default procedure during a hard switch over, if the network has not configured the target cell information in the UE via RRC Release or CHO.

In the above scenario where the NTN gateways are connected to different gNBs and the NTN Payload undergoes hard switch over between the gateways, UEs will experience RLF and perform RRC Reestablishment when the target is available, if the UEs are not preconfigured by the source with CHO or RRC Release. In this scenario, the source may transfer all the available UE Context via XN to the target, so that when UEs perform RRC Reestablishment at the target, the target has the UE context available. Else the target may ask the UEs to perform RRC Setup as a response to RRC Reestablishment, which is not preferred. Hence the proposal is to enable group transfer to UE Context from source gNB to target gNB via XN during a Feeder Link Switch Over.

Proposal 8: Allow group transfer of UE Context via XN between the source and target gNBs for feeder link switch over.

# Summary

Based on the discussion, we have following proposals:

Proposal 1: Uu Cell ID shall be used in XN and NG based Handover procedures for NTN.

Proposal 2: Uu Cell ID shall be used in Served Cell Information and Neighbour Information IEs in XN Setup procedure for NTN.

Proposal 3: NG-RAN Signalling Section 16.14.5 in TS 38.300 shall provide clarification on the Proposal 1 and 2.

Proposal 4:  Served Cell Information and Neighbour Information IEs in both XN Setup procedure and Configuration Update procedure shall allow multiple TACs for NTN cells.

Proposal 5:  NTN Cell Coverage Stop Time and Ephemeris information shall be exchanged via XN Setup procedure and Config Update procedure between the neighbouring gNBs to assist NTN mobility and ANR in Release 18.

Proposal 6: Proposal 5 shall be applicable for Feeder Link Switch over scenario as well.

Proposal 7: Hard or Soft Feeder link Switch over indication shall be exchanged via XN Setup procedure and Config Update procedure between the neighbouring gNBs

Proposal 8: Allow group transfer of UE Context via XN between the source and target gNBs for feeder link switch over.