3GPP TSG-RAN WG3 Meeting #117-e	R3-224334

# E-meeting, 15 – 24 August 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.401 | CR | 0240 | rev | - | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Multicast Session Establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, CATT, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | F1AP UE specific UE context management procedures were updated to include MBS related information at RAN3#116, but the call flow of Multicast MBS Session Context establishment procedure did not capture the corresponding part yet. In order to make decision on whether to trigger the UE specific context management procedure or not by taking MBS session status into account, we also moved the NG-U distribution setup procedure before the related F1AP procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add missing UE context management procedures in Multicast MBS Session Context establishment procedure. - Move the NG-U distribution setup procedure before the UE reated context management procedures. Impact Analysis: This CR has an isolated impact towards the previous version of the specification, and will not cause any non-backward compatible issues. This CR only have impacts on the MBS multicast context setup function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE context management procedure in Multicast MBS Session Context establishment procedure is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.15.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of the first change--------------------

#### 8.15.1.2	Multicast MBS Session Context Establishment

Figure 8.15.1.2-1 illustrates an examplified interaction of NGAP, E1AP, F1AP and RRC protocol functions for Multicast MBS Session Context Establishment.

Figure 8.15.1.2-1: Multicast MBS Session Context establishment

1.	A multicast session context is established by the 5GC.

2/3. If not yet existing, the gNB-CU-CP establishes the multicast bearer context at the gNB-CU-UP, in order to retrieve for unicast NG-U transport the GTP DL TEID, a shared resource address (GTP DL TEID).

4/5. The gNB-CU CP triggers the NGAP Distribution Setup procedure. For unicast transport, DL/UL GTP TEIDs are exchanged, for multicast transport, multicast address information is fetched from the 5GC.

6/7.	If applicable, the gNB-CU-CP establishes the Multicast Context at the DU, providing MRB configuration. It may contain MBS Area Session ID information.

8/9. If applicable,	the gNB-CU-CP provides the MRB information for the joined UE to the gNB-DU via F1 UE Context Management procedures.

NOTE 1:	The gNB-DU may request to establish multiple F1-U tunnels for the MRB, then the step 10~13 should be triggerred multiple times.

NOTE 2:	The interactions between the gNB-CU-CP and gNB-CU-UP for UE specific Berarer Context management before and after step 8/9 are not illustrated in this Figure.

10.	Dependent on e.g. joined UEs, the gNB-DU triggers the establishment of an F1-U tunnel, which is established either per DU or per cell or per MBS Area Session ID. The receiving gNB-CU-CP may need to fetch a gNB side NG-U TNL address information for the gNB-CU-UP by means of a E1AP MC Bearer Context Modification procedure.

7/8. The gNB-CU CP triggers the NGAP Distribution Setup procedure. For unicast transport, DL/UL GTP TEIDs are exchanged, for multicast transport, multicast address information is fetched from the 5GC.

11/12. The gNB-CU-UP side of the F1-U and NG-U UP entity is established by means of the E1AP MC Bearer Context Modification procedure, providing the DU side F1-U TNL address and the 5GC NG-U TNL address to the gNB-CU-UP, which provides the gNB-CU-UP side F1-U TNL address in return.

13.	The gNB-CU-UP side F1-U TNL address is provided to the gNB-DU.

Note 3: The step 10-13 may be triggered earlier than step 8 and 9.

14.	In case of NG-U multicast transport, the gNB-CU-UP joins the NG-U multicast group.

15.	The gNB-CU-CP RRC-configures each UE which has joined the multicast group.

16.	The gNB successfully terminates the NGAP procedure for establishing the multicast session context.

17.	The multicast MBS media stream is provided to the UEs.

On NG-U, in case of location dependent multicast MBS Sessions, multiple shared NG-U transport tunnels may need to be setup, one per Area Session ID served by the gNB.

In case of shared NG-U termination,

-	the gNB-CU-UP may provide the gNB-CU-CP at E1 setup or configuration update about established shared NG-U terminations, indicated by one or several MBS Session IDs.

-	at establishment of the MC bearer context in the gNB-CU-UP, the gNB-CU- CP may request the gNB-CU-UP to either apply the available MRB configuration of the shared NG-U termination, or to apply the MRB configuration requested by the gNB-CU-CP. The gNB-CU-UP provides the MRB configuration to the gNB-CU-CP if the MRB configuration requested by the gNB-CU-CP and the available MRB configuration of the shared NG-U termination are different.

--------------End of the changes--------------------