3GPP TSG-RAN WG2 Meeting #117-e	R3-224345

Online, 15th – 24th Aug 2022

Agenda Item:	16.4

Source: 	vivo

Title:         	Authorization and mobility for Multi-path UE aggregation

Document for: 	Discussion and Decision

# Introduction

In the WID of R18 SL Relay [1], the following objectives had been included:

- Study the benefit and potential solutions for multi-path support to enhance reliability and throughput (e.g., by switching among or utilizing the multiple paths simultaneously) in the following scenarios [RAN2, RAN3]: - A UE is connected to the same gNB using one direct path and one indirect path via 1) Layer-2 UE-to-Network relay, or 2) via another UE (where the UE-UE inter-connection is assumed to be ideal), where the solutions for 1) are to be reused for 2) without precluding the possibility of excluding a part of the solutions which is unnecessary for the operation for 2). Note 3A: Study on the benefit and potential solutions are to be completed in RAN#98 which will decide whether/how to start the normative work. Note 3B: UE-to-Network relay in scenario 1 reuses the Rel-17 solution as the baseline. Note 3C: Support of Layer-3 UE-to-Network relay in multi-path scenario is assumed to have no RAN impact and the work and solutions are subject to SA2 to progress.

In this contribution, we will discuss the above multi-path/UE aggregation authorization and mobility for ideal-link backhaul scenario.

# Discussion

There are two potential multi-path/UE aggregation solutions, i.e. PC5 relaying multi-path/UE aggregation architecture (SL L2 U2N relay architecture) and ideal-link relaying multi-path/UE aggregation architecture (with ideal-link backhaul between two UEs). For multi-path/UE aggregation one UE being connected to gNB on Uu can at the same time be connected to the same gNB through a secondary UE it is inter-connected to by mean of PC5 or ideal-link. For simplicity the UE connected to gNB by direct path Uu and indirect path through the secondary UE can be called Primary UE (PUE), i.e., remote UE in case of PC5 link. And the secondary UE assisting the PUE accessing to gNB, for multi-path/UE aggregation purpose, on the indirect path can be called Secondary UE (SUE), i.e., relay UE in case of PC5 link. From the perspective of PUE, there are two paths for data delivery, one is direct path and another is indirect path via a SUE as shown in Figure 1, Figure 2 and Figure 3.

  - Scenarios for ideal-link and PC5

For multi-path/UE aggregation solutions, considering UP path and CP path only from the perspective of PUE (remote UE for PC5), there are the following scenarios as shown in the Figure 1, Figure 2 and Figure 3 (i.e. UE 1 is PUE (remote UE for PC5) and UE 2 is SUE (relay UE for PC5):

Baseline principle is that separate RRC, RLC and MAC procedures for PUE (UE1) and SUE (UE2).

    - CP over direct path only & UP over both paths

In this scenario (see Figure 1), signalling data or SRB(s) has only one path, i.e. direct path, and user data or DRB(s) may have two paths, i.e. direct path and indirect path.

A typical connection establishment procedure has two key steps:

- In the first step, remote UE establishes direct link with its serving gNB via legacy procedures and achieves both CP and UP path over direct link firstly. This is legacy Uu link establishment procedure.

- In the second step, when a suitable SUE is discovered or a SUE with ideal-link is available, another indirect UP path can be added and finally user data or DRBs may have two paths to achieve enhanced reliability and throughput performance.

    - CP over indirect path only & UP over both paths

In this scenario (see Figure 2), signalling data or SRB(s) has only one path, i.e. indirect path, and user data or DRB(s) may have two paths, i.e. direct path and indirect path.

A typical connection establishment procedure has two key steps:

- In the first step, PUE UE establishes indirect link with its serving gNB via a suitable SUE, e.g. when the legacy RSRP thresholds for remote UE and relay UE are met or PUE and SUE are connected using ideal link (e.g., PUE and SUE connected with ideal wire link), and achieves both CP and UP path over indirect link firstly.  For PC5 case, this is legacy R17 U2N relay link establishment procedure. For ideal-link case, similar procedure can be FFS.

- In the second step, when PUE moves closer to gNB and Uu link becomes available, another UP direct path can be added and finally user data or DRB may have two paths to achieve enhanced reliability and throughput performance.

    - CP&UP over both paths

In this scenario (see Figure 3), both SRB(s) and DRB(s) may have two paths, i.e. direct path and indirect path. And current Scenario (Scenario 3) can be based on either Scenario 1(Section 2.1.1) or Scenario 2 (Section 2.1.2) to add another path not only for DRB but also for SRB. The main benefit of this scenario is higher signalling reliability.

Furthermore, potential conditions for multi-path/UE aggregation establishment may include the followings and details are FFS:

- Upper layer policy and authorization information allow multi-path;

- PC5 and/or Uu RSRP above threshold;

- QoS requirement e.g., reliability above threshold;

- Data buffer size above threshold;

From our understanding, the scenario depicted in Figure 2 of section 2.1.2 is mainly for those remote UEs located in the coverage edge of serving cells and the scenario depicted in Figure 1 of section 2.1.1 is mainly for normal coverage cases. This scenario in current section 2.1.3 is a supplementary for higher signalling reliability. We think all these 3 scenarios should be included in the scope of the SI phase.

- The following three multi-path/UE aggregation scenarios are considered:

- 1: CP over direct path only & UP over both paths;

- 2: CP over indirect path only & UP over both paths;

- 3: CP&UP over both paths.

  - PUE&SUE authorization for multi-path

For multi-path establishment as briefly discussed in section 2.1, to establish the second path (especially in scenario depicted in clause 2.1.1) PUE and SUE need to be authorized for second path establishment.

- SL Relay PC5 case

For SL relay architecture, Rel-17 relay discovery and authorization procedure can be reused for multi-path establishment.

- Ideal-link case

For Ideal-link there is no need to discover between PUE and SUE. However, authorization is needed for both UEs. Similar with SL relay architecture, for ideal-link, PUE and SUE need to be authorized by CN to be allowed provide multi-path establishment.

- In case of ideal-link multi-path scenarios, consider PUE and SUE authorization procedure

Two potential alternatives can be considered for PUE and SUE authorization:

  - Alt1: PUE and SUE are in RRC_CONNECTED mode. For multi-path purpose, AMF authorizes and indicates to gNB, e.g, in initial UE context setup. gNB can get authorization of PUE and SUE from CN (AMF) separately. PUE can report SUE identity to gNB, based on PUE/SUE, authorization from AMF, gNB can configure SUE for multi-path establishment with PUE.

  - Alt2: PUE and SUE are correlated and bind at CN when PUE and SUE subscription. This is possible in case PUE and SUE belong to one subscriber or PUE and SUE are collocated on the same machine, see Figure 7. In such case, when PUE and/or SUE connects to network, CN can indicate to gNB, e.g, in initial UE context setup, that the two UEs are correlated and SUE is authorized for multi-path establishment with PUE

Both Alt1 and Alt2 are viable solution for PUE and SUE authorization. But, Alt2 works in some restricted PUE and SUE correlated scenario. On the contrary, Alt1 works all potential PUE and SUE scenario, as far as PUE and SUE have ideal-link between them.

- Discuss and agree PUE and SUE authorization in case of ideal-link multi-path scenarios based on above authorization Alt1, i.e., gNB gets authorization results of PUE and SUE from CN(AMF) separately.

  - Multi-path/UE aggregation Mobility

For both SL relay scenario and ideal-link scenario Multi-path/UE aggregation mobility may be characterized in three-fold:

- Mobility from multi-path (MP) to single path (SP)

- Mobility from SP to MP

- Mobility from MP to MP

## Mobility from MP to SP

Figure 4: Multi-path/UE aggregation mobility from MP to SP

As depicted in Figure 4, multi-path/UE aggregation mobility from MP to SP may be due to:

- Figure 4.A: Primary UE (PUE), i.e., the UE (Remote UE in SL relay case) which needs multi-path/UE aggregation connection to source gNB, mobility from source cell to target cell.

- Figure 4.B: Secondary UE (SUE), i.e., the UE (Relay UE in SL relay case) which assists providing PUE with multi-path/UE aggregation connection source gNB, mobility from source cell to target cell.

- Multi-path /UE aggregation mobility from MP to SP may be due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE (Relay UE in SL relay case) which assists providing PUE with multi-path/UE aggregation) HO from source cell to target cell.

- Mobility from MP to SP due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE which assists providing multi-path/UE aggregation connectivity for PUE) HO from source cell to target cell is valid multi-path /UE aggregation mobility scenario.

In both cases, multi-path/UE aggregation connection on the indirect link needs to be released, for PUE to fallback to single connectivity to gNB, during PUE HO or SUE HO to target cell.

Thus,

- During mobility from MP to SP due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE which assists providing multi-path/UE aggregation connectivity for PUE) HO from source cell to target cell the indirect link needs to be released.

## Mobility from SP to MP

Figure 5: Multi-path/UE aggregation mobility from SP to MP

As depicted in Figure 5, multi-path/UE aggregation mobility from MP to SP may be due to:

- Figure 5.A: Primary UE (PUE), i.e., the UE (Remote UE in SL relay case) which needs multi-path/UE aggregation connection to source gNB, moves from source cell to target cell.

- Figure 5.B: Secondary UE (SUE), i.e., the UE (Relay UE in SL relay case) which assists providing PUE with multi-path/UE aggregation connection source gNB, moves from source cell to target cell.

- Multi-path /UE aggregation mobility from SP to MP may be due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE (Relay UE in SL relay case) which assists providing PUE with multi-path/UE aggregation) HO from source cell to target cell.

- Mobility from SP to MP due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE which assists providing multi-path/UE aggregation connectivity for PUE) HO from source cell to target cell is valid multi-path /UE aggregation mobility scenario.

In both cases, multi-path/UE aggregation connection on the indirect link needs to be added, for PUE to establish multi-path/UE aggregation connectivity to gNB, during PUE HO or SUE HO to target cell.

Thus,

- Mobility from MP to SP due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE which assists providing multi-path/UE aggregation connectivity for PUE) HO from source cell to target cell is valid multi-path /UE aggregation mobility scenario.

## Mobility from MP to MP

Figure 6: Multi-path/UE aggregation mobility from SP to MP

As depicted in Figure 6, multi-path/UE aggregation mobility from MP to MP may be due to:

- Figure 6.A: PUE, i.e., the UE (Remote UE in SL relay case) which needs multi-path/UE aggregation connection to source gNB and SUE), i.e., the UE (Relay UE in SL relay case) which assists providing PUE with multi-path/UE aggregation connection moving altogether from source cell to target cell.

This scenario is mostly frequent in industry environment where both PUE and SUE, in UE aggregation mode, are collocated on a moving machine, as shown in Figure 7.

Figure 7: Scenario of moving PUE and SUE

- Multi-path /UE aggregation mobility from MP to MP may be due to PUE and SUE HO from source cell to target cell.

- Mobility from MP to MP due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO and SUE (i.e., the UE which assists providing multi-path/UE aggregation connectivity for PUE) HO from source cell to target cell is valid multi-path /UE aggregation mobility scenario.

To allow both PUE and SUE HO from source cell to target simultaneous RRC reconfiguration of both UE may be applied.

- Figure 6.B: PUE using multi-path/UE aggregation connectivity with SUE1 at source cell may HO from source cell to target cell with SUE2 assisting with multi-path/UE aggregation connectivity.

This mobility scenario would require addition of SUE2 at target cell. This case is similar to Figure X.A with addition of a new UE for multi-path/UE aggregation connectivity at target cell.

- Multi-path /UE aggregation mobility from MP to MP may be due to PUE HO from source cell to target cell with addition of new UE as SUE for Multi-path /UE aggregation connectivity is similar to MP to SP due to PUE mobility.

Thus,

- Mobility from MP to MP scenario may require simultaneous/combined RRC reconfiguration during HO for both UE at target cell

# Conclusion

In this paper, the following observations and proposal are given:

- The following three multi-path/UE aggregation scenarios are considered:

- 1: CP over direct path only & UP over both paths;

- 2: CP over indirect path only & UP over both paths;

- 3: CP&UP over both paths.

- In case of ideal-link multi-path/UE aggregation scenarios, Consider SUE authorization procedure

- Discuss and agree SUE authorization in case of ideal-link multi-path/UE aggregation scenarios based on above authorization Alt1, i.e., PUE report SUE identity to gNB, then gNB sends SUE authorization request to AMF

- Multi-path /UE aggregation mobility from MP to SP may be due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE (Relay UE in SL relay case) which assists providing PUE with multi-path/UE aggregation) HO from source cell to target cell.

- Mobility from MP to SP due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE which assists providing multi-path/UE aggregation connectivity for PUE) HO from source cell to target cell is valid multi-path /UE aggregation mobility scenario.

- During mobility from MP to SP due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE which assists providing multi-path/UE aggregation connectivity for PUE) HO from source cell to target cell the indirect link needs to be released.

- Multi-path /UE aggregation mobility from SP to MP may be due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE (Relay UE in SL relay case) which assists providing PUE with multi-path/UE aggregation) HO from source cell to target cell.

- Mobility from SP to MP due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE which assists providing multi-path/UE aggregation connectivity for PUE) HO from source cell to target cell is valid multi-path /UE aggregation mobility scenario.

- Mobility from MP to SP due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO or SUE (i.e., the UE which assists providing multi-path/UE aggregation connectivity for PUE) HO from source cell to target cell is valid multi-path /UE aggregation mobility scenario.

- Multi-path /UE aggregation mobility from MP to MP may be due to PUE and SUE HO from source cell to target cell.

- Mobility from MP to MP due to PUE (i.e., the UE which needs multi-path/UE aggregation connectivity) HO and SUE (i.e., the UE which assists providing multi-path/UE aggregation connectivity for PUE) HO from source cell to target cell is valid multi-path /UE aggregation mobility scenario.

- Multi-path /UE aggregation mobility from MP to MP may be due to PUE HO from source cell to target cell with addition of new UE as SUE for Multi-path /UE aggregation connectivity is similar to MP to SP due to PUE mobility.

- Mobility from MP to MP scenario may require simultaneous/combined RRC reconfiguration during HO for both UE at target cell

# References

- RP-221262, NR sidelink relay enhancements, LG Electronics

[2]