3GPP TSG-RAN WG3 Meeting #117-e	R3-224355

E-meeting, 15th – 24th August, 2022

Title: 	Discussion on group mobility for mobile IAB and UEs

Source: 	Huawei

Agenda item:	13.3

Document Type:	Discussion

1 Introduction

In Rel-18 IAB, the enhancement for mobility of an IAB-node together with its served UEs is an objective in Rel-18 IAB WID [1]:

- Define Procedures for migration/topology adaptation to enable IAB-node mobility, including inter-donor migration of the entire mobile IAB-node (full migration) [RAN3, RAN2]

- Enhancements for mobility of an IAB-node together with its served UEs, including aspects related to group mobility. No optimizations for the targeting of surrounding UEs. [RAN3, RAN2]

Note: Solutions should avoid touching upon topics where Rel-17 discussions already occurred and where the topic was excluded from Rel-17, except for enhancements that are specific to IAB-node mobility.

- Mitigation of interference due to IAB-node mobility, including the avoidance of potential reference and control signal collisions (e.g. PCI, RACH). [RAN3, RAN2]

Note: At the beginning of the work period, RAN3, RAN2 should discuss the potential complexity of a scenario where a mobile IAB node connects to a stationary (intermediate) IAB node, with respect to the scenario where a mobile IAB node connects directly to an IAB-donor.

In this paper, we mainly focus on the issues about the group mobility for mobile IAB and UEs.

2 Discussion

As discussed in Rel-17 IAB WI, the flowing sequences {full-nested, gradual bottom-up, gradual top-down} were proposed to support full migration.

Figure 1 Candidate sequences for full migration

In gradual top-down, the IAB-MT switches first. It is worth noting that all the connected UEs traffic will be switched to the target CU also, after the IAB-DU performs migration. So, the target CU should consider the traffic load of the migrating IAB-DU’s served UEs to make the admission control decision, when receives handover request for the IAB-MT. Moreover, when the target CU performs F1AP configuration in the target path, it will be beneficial if the target CU has knowledge of all the UE’s traffic to be switched to its topology, otherwise, frequent F1AP signalling need be used if the target CU configure target path for one UE’s traffic at one time.

Observation 1: In “MT’s HO Cmd first” sequence (Gradual Top-down), the Xn HO procedure for mobile-IAB at least needs to indicate some context of its serving UEs, so that the target CU can determine whether to accept them all and do the related F1AP configuration in the target path all together.

Similarly, in full nested and gradual bottom-up sequence, the UEs switch first, and the target CU may also treat the UEs as a group but not individuals when making the admission control decision and perform F1AP configuration in target path.

Observation 2: In “UE’s HO Cmd first” sequences (Full Nested, or Gradual Bottom-up), the Xn HO procedure for one UE needs to indicate the context of other UEs in this group, for the similar motivation as observation 1.

In gradual top-down, to indicate some UEs context in the MT handover preparation procedure, the target logical DU must have been activated so that the UEs can report to the source CU the RSRP measurement result of the cells under the target logical DU. But the target logical DU activation should be after the MT handover procedure if without any enhancement, since the MT’s handover preparation procedure is the first step to do in gradual top-down. In this sense, the combination of MT’s handover and the UE’s handover seems not feasible for gradual top-down sequence.

Observation 3: There is a UE’s measurement/report procedure of target cell in target logical DU, which should between “target DU cell activation” and “UE’s HO procedure”, then the top-down sequence seems cannot support the “group” HO for IAB-MT and connected UEs if without any enhancement.

Based on the observations above, we suggest:

Proposal 1: R18 to support group/combined HO at least for all connected UEs of a mobile IAB.

Proposal 2：RAN3 discuss whether to support the combined HO of mobile IAB-node and the connected UEs.

3 Conclusion

This paper mainly discusses the potential issues on the group mobility for mobile IAB and UEs, then we provide the following observations and proposals:

Observation 1: In “MT’s HO Cmd first” sequence (Gradual Top-down), the Xn HO procedure for mobile-IAB at least needs to indicate some context of its serving UEs, so that the target CU can determine whether to accept them all and do the related F1AP configuration in the target path all together.

Observation 2: In “UE’s HO Cmd first” sequences (Full Nested, Gradual Bottom-up), the Xn HO procedure for one UE needs to indicate the context of other UEs in this group, for the similar motivation as observation 1.

Observation 3: There is a UE’s measurement/report procedure of target cell in target logical DU, which should between “target DU cell activation” and “UE’s HO procedure”, then the top-down sequence seems cannot support the “group” HO for IAB-MT and connected UEs if without any enhancement.

Proposal 1: R18 to support group/combined HO at least for all connected UEs of a mobile IAB.

Proposal 2：RAN3 discuss whether to support the combined HO of mobile IAB-node and the connected UEs.

Reference

- RP-221815, New WID on Mobile IAB. Qualcomm.