3GPP TSG-RAN WG3 Meeting #117-e	R3-224367

# Online, August 15th – 24th 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for RVQoE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_QoE-Core |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current text of TS 38.300, chapter 21.4 states that “If there is no reporting periodicity defined in the RAN visible QoE configuration, RAN visible QoE reports are sent together with the non-RAN visible QoE reports.”. This is not aligned with the RAN3 agreeement from RAN3#115-e which states: “If the reporting periodicity of RVQoE is not explicitly indicated in the RVQoE configuration, RVQoE reports can be sent together with the legacy QoE reports.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | Aligned the text in clause 21.4, concerning the sending of RAN visible QoE reports, with the above RAN3 agreement. Impact Analysis - No functional impacts, since the change simply aligns the text with the TS 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification text does not reflect RAN3 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------Start of changes-----------------------------------------------

## 21.4	RAN Visible QoE Measurements

RAN visible QoE measurements are configured by the gNB, where a subset of QoE metrics is reported from the UE as an explicit IE readable by the gNB. The RAN visible QoE measurements can be utilized by the gNB for network optimization. The RAN visible QoE measurements are supported for the DASH streaming and VR services. The gNB configures the RAN visible QoE measurement to collect all or some of the available RAN visible QoE metrics, where the indication of metric availability is received from the OAM or CN. The set of available RAN visible QoE metrics is a subset of the metrics which are already configured as part of QoE measurement configuration encapsulated in the transparent container. The PDU session ID(s) corresponding to the service that is subject to QoE measurements can also be reported by the UE along with the RAN visible QoE measurement results.

The RAN visible QoE measurements can be reported with a reporting periodicity different from the one of regular QoE measurements. If there is no reporting periodicity defined in the RAN visible QoE configuration, RAN visible QoE reports can be sent together with the non-RAN visible QoE reports.

Multiple simultaneous RAN visible application layer measurements configuration and reports can be supported for RAN visible application layer measurement, and each RAN visible application layer measurement configuration and report is identified by the same RRC identifier as the application layer measurement configuration and measurement report. After receiving the RAN visible application layer measurement configuration, the UE RRC layer forwards the configuration to the application layer, indicating the service type, the RRC identifier and the periodicity. RAN visible application layer configuration can only be configured if there is a corresponding application layer measurement configuration for the same service type configured at the UE. The application layer sends the RAN visible application layer measurement report associated with the RRC identifier to the UE's AS layer. UE can send both RAN visible application layer measurement reports and the application layer measurement reports to the gNB in the same MeasurementReportAppLayer message. The gNB can release one or multiple RAN visible application layer measurement configurations from the UE in one RRCReconfiguration message at any time.

During RAN overload, the UE continues to report the configured RAN visible application layer measurements, when the corresponding non RAN visible application layer measurement reporting is paused.

-----------------------------------------------End of changes-----------------------------------------------