3GPP TSG-RAN WG3 #117-e 					                                                      R3-224385

15-24 August 2022

Online

Agenda item:	18.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on the support for discontinuous coverage

Document for:	Discussion & decision

# 1	Introduction

This contribution discusses the RAN3 impact to support discontinuous coverage in IOT NTN.

# Discussion

WID ([1]) describe the following objective for RAN3:

4.1.3	Further enhancement to discontinuous coverage

-	Study and specify, if needed, mobility management enhancements and power saving enhancements for discontinuous coverage, taking into account the conclusions from the SA2 study FS_5GSAT_Ph2.  [RAN2, RAN3].

There are two aspects to be considered:

- A RRC CONNECTED UE may lose the coverage.

For a RRC CONNECTED UE, it will be out of the service area when the satellite moves out of the area. The eNB need to initiate a UE Context Release procedure to inform the MME. The MME need to be aware that the UE context release request procedure is triggered by the discontinuous coverage. The Rel-17 NAS cause value “UE not in PLMN serving area” is not appropriate for this case because the UE remains in the area (I.e. PLMN ID is not changed). The new cause value may also be used for MME initiated UE Context Release. So a new Radio Network Layer cause value is needed.

Propose 1: Introduce a new Radio Network Layer cause value for discontinuous coverage

The draft S1AP CR can be found in ([2])

- Coverage window prediction.

- For a UE: to reduce power consumption, the UE need to know whether it is (or will be) out of coverage. The UE may predict the future coverage window based on SIB. Due to the limitation of SIB, the UE may only predict the coverage window for a limited number of satellites, e.g. for the 4 satellites.

- For a MME: the MME need to know whether the UE is (or will be) out of coverage, so the MME can set the related NAS timer accordingly, and the MME should not initiate the Network Triggered Service Request (or MT-SDT, etc) when the UE is out of coverage.

SA2 ([3]) is discussing this issue. One option is shown below:

Figure 6.6.2-1: UE negotiates Unreachability Period with the AMF using satellite ephemeris information

1.	NG-RAN informs the UE of the satellite ephemeris of the serving cell in RRC signalling. The UE determines how long the UE is still covered by the serving cell.

2.	UE includes Unreachability Period in its Registration Request. The Unreachability Period tells the AMF the foreseen unreachability period caused by discontinuous satellite coverage in the UE location.

NOTE:	The encoding of the unreachability period in terms of start and end time or start time and duration is left for stage 3 specifications.

3.	The AMF may request for UE Unreachability Period estimate from the NWDAF. If the NWDAF has got sufficient history of UE trajectory and the ephemeris of the satellite constellation, it returns UE Unreachability Period estimate to the AMF.

4.	AMF stores the Unreachability Period as part of the UE context in AMF and considers the UE as "not reachable" during that period. The AMF echoes back the UE requested Unreachability Period to acknowledge it has taken notice of it.

Any procedures that apply on UE that is not reachable due to e.g. MICO mode or eDRX, apply also during Unreachability Period, so that the AMF does not page the UE, notifies the UE reachability as "unreachable" to those NFs that have subscribed to UE reachability notifications. Extended DL data buffering in the CN applies if it is supported in the network.

The AMF assigns a Periodic Update Timer that allows the UE to omit periodic updates until the next foreseen connectivity period.

5.	UE is not reachable due to coverage gap caused by discontinuous coverage. If the UE finds coverage before the end of Unreachability Period, it may initiate UL signalling or send UL data. The UE may initiate an additional registration update procedure if it finds unpredicted coverage before the end of Unreachability Period.

6.	DL data targeted for the UE arrives while the UE is not reachable due to discontinuous coverage.

7.	Upon receiving DL Data Notification, the SMF requests DL data transfer from the AMF

8.	The AMF knows based on Unreachability Period received from the UE in step 2 in the previous registration procedure that the UE is not reachable. The AMF includes the corresponding Estimated Maximum Wait time as specified in clause 4.2.3.3 of TS 23.502 [3].

9.	If extended DL data buffering is supported in the SMF, then the SMF initiates data DL data buffering as specified in TS 23.502 [3]

The exchange of the coverage window prediction is performed between the UE and the CN via NAS. The CN know when the UE is reachable, so the CN only initiate a paging procedure when there is a DL data to be delivered to the UE and the UE is in the coverage. The impact to RAN3 is unclear at current stage. It is preferred to wait for SA2 progress, e.g. the prediction for coverage window, and the usage for the prediction.

Proposal 2: RAN3 wait for SA2 progress on support discontinuous coverage.

# 3	Conclusions

In this contribution, we briefly analyzed the support for discontinuous coverage. Our proposals are:

Propose 1: Introduce a new Radio Network Layer cause value for discontinuous coverage

The draft S1AP CR can be found in ([2])

Proposal 2: RAN3 wait for SA2 progress on support discontinuous coverage.

# References

- RP-221806, WID on IoT NTN enhancements

- R3-224386, draft S1AP CR for IOT NTN

- TR 23.700-28, Study on Integration of satellite components in the 5G architecture; Phase 2