3GPP TSG-RAN WG3 Meeting #117-e	R3-224413

Online, 15 – 24 August 2022

Agenda item:	 10.2

Source:		Lenovo

Title:		MRO for fast MCG link recovery

Document for:		Discussion and Decision

1	Introduction

In [1], MRO for fast MCG recovery would be supported in R18:

- Support of SON/MDT enhancements for [RAN3, RAN2]:

•	MR-DC CPAC

•	Successful PScell change report

•	Successful Handover Report (e.g. inter-RAT)

•	NPN

•	RACH report

•	fast MCG recovery

•	NR-U (MRO and UL MLB)

In this paper, we would discuss the details of MRO for fast MCG link recovery.

2	Discussion

In R16, fast MCG link recovery procedure was introduced to inform the network about an MCG failure that the UE has experienced i.e. MCG radio link failure, in order to continue the RRC connection without RRC re-establishment. If T316 is configured, upon RLF happening in MCG, the UE transmits the MCGFailureInformation message to MN via SN while SCG is not suspended. The MN may decide to reconfigure or change or release MCG based on the received MCG failure information. The MN may provide the corresponding response message (e.g. RRC Reconfiguration message with reconfigurationwithSync for MCG, MobilityFromNRCommand, or RRC Release message) to the UE via SN.

Observation: Fast MCG link recovery procedure was introduced to inform MN about MCG failure via SN and recover the MCG link without RRC re-establishment in R16.

Even fast MCG link recovery procedure is supported in MR-DC cases in R16, to reduce work load in R18 SON/MDT topic, NR-NR DC should be prioritised similarly as when we discuss MRO for SCG failure in R17 SON/MDT. If time is allowed, other MR-DC cases can be further discussed.

Proposal 1: MRO for fast MCG link recovery in NR-NR DC should be prioritized in R18.

There may be following cases after MCGFailureInformation message is triggered:

- Case 1: The UE performs RRC re-establishment procedure when the timer T316 expires;

- Case 2: The UE performs RRC re-establishment procedure when SCG failure happens while T316 is running;

- Case 3: The UE stops the timer T316 when the response message is received;

Both Case 1 and Case 2 are failure cases for fast MCG link recovery. In Case 3, since response message is successfully received, fast MCG link recovery procedure is considered to be successful even the UE may fail to perform handover after receiving the response message for MCG reconfiguration for fast MCG link recovery purpose, this handover failure case can be treated as a legacy MRO case since handover procedure is a separate procedure after successfully receiving the HO command for fast MCG link recovery. Therefore, Case 1 and Case 2 shoule be considered for MRO for fast MCG link recovery.

Proposal 2: RAN3 should consider following cases for MRO for fast MCG link recovery:

- The UE performs RRC re-establishment procedure when the timer T316 expires;

- The UE performs RRC re-establishment procedure when SCG failure happens while T316 is running.

For Case 1 and Case 2, the UE experiences failed fast MCG link recovery before RRC re-establishment procedure. The failure type of fast MCG link recovery could be T316 expiry or SCG failure during fast MCG link recovery. It is beneficial for the UE to report fast MCG recovery failure to the network, furthemore, failure type of fast MCG link recovery can be included in the RLF-Report.

Proposal 3: The UE reports fast MCG recovery failure information in the RLF-Report.

The RLF-Report including fast MCG recovery failure related information may be transferred to MN by the receiving node, then MN can underatand why fast MCG link recovery fails based on the failure type of fast MCG link recovery, and perform configuration update to aviod such a failure, for example, if failure is due to the timer T316 expires, MN may enlarge the duration of timer T316.

Proposal 4: MN performs root cause analysis for fast MCG recovery failure based on the received fast MCG recovery failure information in the RLF-Report.

3	Conclusion

In this contribution, the details of MRO for fast MCG link recovery are discussed. We have the following observation and proposals:

Observation: Fast MCG link recovery procedure was introduced to inform MN about MCG failure via SN and recover the MCG link without RRC re-establishment in R16.

Proposal 1: MRO for fast MCG link recovery in NR-NR DC should be prioritized in R18.

Proposal 2: RAN3 should consider following cases for MRO for fast MCG link recovery:

- The UE performs RRC re-establishment procedure when the timer T316 expires;

- The UE performs RRC re-establishment procedure when SCG failure happens while T316 is running.

Proposal 3: The UE reports fast MCG recovery failure information in the RLF-Report.

Proposal 4: MN performs root cause analysis for fast MCG recovery failure based on the received fast MCG recovery failure information in the RLF-Report.

Reference

[1] RP-221825, Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity), CMCC