3GPP TSG-RAN WG3 Meeting #117-e	R3-224415

Online, 15 – 24 August 2022

Agenda item:	 10.3

Source:		Lenovo

Title:		(TP for SON BLCR for 38.300) MRO for inter-system handover for voice fallback

Document for:		Discussion and Decision

1	Introduction

MRO enhancement for inter-system handover for voice fallback would be supported in R18 [1]:

- Support of data collection for SON features, including, MRO for MR-DC SCG failure scenario, and MRO enhancement for inter-system handover voice fallback,

- Specification of the UE reporting necessary to enhance the mobility parameter tuning [RAN2]

- Specification of the inter-node information exchange, including possible enhancements to interfaces    
[RAN3]

In this paper, we would discuss details of MRO for inter-system handover for voice fallback.

2	Discussion

In R15 legacy inter-system inter-RAT handover from NR to E-UTRA, the UE will revert back to the configuration used in the source NR PCell and initiate re-establishment procedure if inter-system inter-RAT handover from NR failure happens.

In R16 to enable the UE understand the handover is triggered by EPS fallback for IMS voice and prioritize to establish RRC connection to the E-UTRA cell, the voiceFallbackIndication IE is introduced in the MobilityFromNRCommand message. As captured in Section 5.4.3.5 in TS38.331, when MobilityFromNRCommand message including the voiceFallbackIndication IE is sent to the UE, the UE shall attempt to select a E-UTRA cell for IMS voice in priority if failing in inter-system inter-RAT handover from NR to E-UTRA for the purpose of voice fallback. If a suitable E-UTRA cell is selected, the UE performs the actions upon going to RRC_IDLE with release cause 'RRC connection failure'. But for some certain coverage of the cell, UE may not able to find a suitable E-UTRA cell, if so, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure.

#### 5.4.3.5 Mobility from NR failure 2> if voiceFallbackIndication is included in the MobilityFromNRCommand message: 3> attempt to select an E-UTRA cell: 4> if a suitable E-UTRA cell is selected: 5> perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'; 4> else: 5> revert back to the configuration used in the source PCell; 5> initiate the connection re-establishment procedure as specified in clause 5.

After failure of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, there may be two cases which should be taken into account for MRO:

- Case 1: a suitable E-UTRA cell is selected, and the UE tries RRC connection setup procedure for the voice service in the E-UTRA cell.

- Case 2: none suitable E-UTRAN cell can be selected, the UE reverts back to the configuration of the source PCell and initiates RRC re-establishment procedure in NR.

Proposal 1: RAN3 consider the following cases for MRO enhancement for inter-system inter-RAT handover for voice fallback:

- after failure of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE establishes RRC connection to E-UTRA when a suitable E-UTRA cell is selected;

- after failure of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE reverts back to the configuration in source PCell and initiates RRC re-establishment procedure in NR when no suitable E-UTRA cell is found.

Similar as R15 inter-system inter-RAT handover failure, RLF report may also be triggerred after failure of inter-system inter-RAT handover from NR to E-UTRA for voice fallback. In legacy MRO for inter-system inter-RAT handover failure which may be caused by radio link problem, network may detect the failure type, and modify mobility parameters or perform coverage optimisation for MRO purpose. However, since inter-system inter-RAT handover for voice fallback is a service based handover, and it is totally different from legacy inter-system inter-RAT handover which is a radio link quality based handover, network may configure different parameters for triggering inter-system inter-RAT handover for voice fallback compared with legacy inter-system inter-RAT handover. Additionally, the motivation of inter-system inter-RAT handover for voice fallback is prioritizing the UE to establish RRC connection to the E-UTRA cell to enable voice service, even if the inter-system inter-RAT handover procedure fails.  Therefore, it is necessary for the network to distinguish failure case due to inter-system inter-RAT handover triggered by EPS fallback for IMS voice from legacy failure case in inter-system inter-RAT handover for the sake of proper MRO analysis and modification, e.g. based on different failure cases, network may update the parameters related with triggering handover correspondingly to ensure the successful completion of inter-system inter-RAT handover with specific motivations separately.

Observation: It is necessary for the network to distinguish failure case due to inter-system inter-RAT handover triggered by EPS fallback for IMS voice from legacy failure case in inter-system inter-RAT handover.

Since the UE can keep the RLF-Report as long as 48 hours after the failure is detected, there is a possibility that the network has released the UE context when receving the RLF-Report. Storing UE context is up to network implementation, so it is not reasonable to mandate the network to always keep the UE context. In order to enable the network distinguish the failure case due to inter-system inter-RAT handover triggered by EPS fallback for IMS voice, it is needed for the UE to report an indication on whether the failed handover was triggered by EPS fallback for IMS voice in the RLF report.

Proposal 2: The UE reports an indication on whether the failed inter-system inter-RAT handover was triggered by EPS fallback for IMS voice in the RLF report.

For Case1, since the UE establishes RRC connection to a E-UTRA cell, the RLF report including the indication on whether the failed inter-system inter-RAT handover was triggered by EPS fallback for IMS voice may be transmitted to the E-UTRA cell by the UE. Then, the E-UTRA cell may transfer the UE RLF Report Container which includes the indication to the source NR cell over S1 and NG interfaces. For example, the existing ENB CONFIGURATION TRANSFER message can be reused to transfer the UE RLF Report Container including the indication from the E-UTRA node to MME, MME shall transparently transfer the UE RLF Report Container towards the AMF serving the source NG-RAN node, then AMF shall transfer the UE RLF Report Container to the source NG-RAN node via the existing DOWNLINK RAN CONFIGURATION TRANSFER message.

Proposal 3: The UE RLF Report Container including the indication on whether the failed inter-system inter-RAT handover was triggered by EPS fallback for IMS voice may be transferred from the re-connected E-UTRA node to the source NG-RAN node via the existing ENB CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

For Case 2, the UE connects to a NR cell after RRC re-establishment, the RLF report including the indication on whether the failed inter-system inter-RAT handover was triggered by EPS fallback for IMS voice may be transmitted to the NR cell. Then, the NR cell may transfer the UE RLF Report Container which includes the indication to the source NR cell directly via Xn interface, e.g. the existing FAILURE INDICATION message can be reused. Also, if there is no direct Xn interface, the NR cell may transfer the UE RLF Report Container to the source NR cell via NG interface, e.g. the existing UPLINK RAN CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message can be reused.

Proposal 4: The UE RLF Report Container including the indication on whether the failed inter-system inter-RAT handover was triggered by EPS fallback for IMS voice may be transferred from the re-established NG-RAN node to the source NG-RAN node via the existing FAILURE INDICATION message, or UPLINK RAN CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

For both Case1 and Case2, based on the received indication on whether the failed inter-system inter-RAT handover was triggered by EPS fallback for IMS voice, the source NG-RAN node may distinguish failure case due to inter-system inter-RAT handover for voice fallback from legacy failure case in inter-system inter-RAT handover when performing MRO analysis after receiving the UE RLF Report Container, furthermore, source NG-RAN node may modify handover related parameters to enable the UE perfrom a successful inter-system inter-RAT handover for voice fallback.

Proposal 5: Source NG-RAN node may distinguish failure case due to inter-system inter-RAT handover for voice fallback from failure case in inter-system inter-RAT handover for MRO, based on the received indication on whether the failed inter-system inter-RAT handover was triggered by EPS fallback for IMS voice.

In general, a failure in inter-system inter-RAT handover for voice fallback are different from a failure in legacy inter-system inter-RAT handover, so, to distinguish from the existing failure type definition and detection for Too Early or Too Late inter-system handovers in TS38.300, we need to have a separate failure type definition and detection for inter-system inter-RAT HO from NR to E-UTRA for voice fallback in stage 2.

Proposal 6: Introduce a separate failure type definition and detection for inter-system inter-RAT HO from NR to E-UTRA for voice fallback in stage 2.

The corresponding TP for TS38.300 is attached in the Annex.

3	Conclusion

In this contribution, we discussed MRO for inter-system inter-RAT handover for voice fallback, the following observation and proposals are given:

Observation: It is necessary for the network to distinguish failure case due to inter-system inter-RAT handover triggered by EPS fallback for IMS voice from legacy failure case in inter-system inter-RAT handover.

Proposal 1: RAN3 consider the following cases for MRO enhancement for inter-system inter-RAT handover for voice fallback:

- after failure of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE establishes RRC connection to E-UTRA when a suitable E-UTRA cell is selected;

- after failure of inter-system inter-RAT handover from NR to E-UTRAN for voice fallback, the UE reverts back to the configuration in source PCell and initiates RRC re-establishment procedure in NR when no suitable E-UTRA cell is found.

Proposal 2: The UE reports an indication on whether the failed inter-system inter-RAT handover was triggered by EPS fallback for IMS voice in the RLF report.

Proposal 3: The UE RLF Report Container including the indication on whether the failed inter-system inter-RAT handover was triggered by EPS fallback for IMS voice may be transferred from the re-connected E-UTRA node to the source NG-RAN node via the existing ENB CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

Proposal 4: The UE RLF Report Container including the indication on whether the failed inter-system inter-RAT handover was triggered by EPS fallback for IMS voice may be transferred from the re-established NG-RAN node to the source NG-RAN node via the existing FAILURE INDICATION message, or UPLINK RAN CONFIGURATION TRANSFER message and DOWNLINK RAN CONFIGURATION TRANSFER message.

Proposal 5: Source NG-RAN node may distinguish failure case due to inter-system inter-RAT handover for voice fallback from failure case in inter-system inter-RAT handover for MRO, based on the received indication on whether the failed inter-system inter-RAT handover was triggered by EPS fallback for IMS voice.

Proposal 6: Introduce a separate failure type definition and detection for inter-system inter-RAT HO from NR to E-UTRA for voice fallback in stage 2.

Reference

[1] RP-221825, Revised WID: Further enhancement of data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity), CMCC

Annex: TP for TS38.300

15.5.2.2.x	Connection failure due to inter-system inter-RAT HO from NR to E-UTRA for voice fallback

One of the functions of Mobility Robustness Optimization is to detect connection failures that occurred due to inter-system inter-RAT HO from NR to E-UTRA for voice fallback. The problem is defined as follow:

-	A handover failure occurs during the handover procedure from a cell belonging to an NG-RAN node to a cell belonging to an E-UTRAN node; the UE tries RRC connection setup procedure for the voice service in a cell belonging to an E-UTRAN node or the UE attempts to re-connect to a cell belonging to an NG-RAN node.

Detection mechanism

A failure indication may be sent to the node last serving the UE when the NG-RAN node fetches the RLF REPORT from UE by triggering:

-	The Failure Indication procedure over Xn;

-	The Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer procedure over NG.

In case the last serving node is an NG-RAN node, the detection mechanisms for inter-system inter-RAT HO failure from NR to E-UTRA for voice fallback is carried out through the following:

-	There is a recent handover procedure from a cell belonging to an NG-RAN node to a cell belonging to an E-UTRAN node for the UE prior to the connection failure, and, the successful re-connect cell is a cell belonging to an E-UTRAN node where the UE attempts a RRC connection setup procedure, or the re-establishment attempt cell is a cell belonging to an NG-RAN node where the UE attempts a RRC re-establishment procedure.

An indication on whether a failure in a recent handover procedure from a cell belonging to an NG-RAN node to a cell belonging to an E-UTRAN node was triggered by EPS fallback for IMS voice may be included in the RLF Report.

In case the re-establishment attempt cell is a cell belonging to an NG-RAN node where the UE attempts a RRC re-establishment procedure, the UE may make the RLF Report available to an NG-RAN node. The NG-RAN node may forward the information using the FAILURE INDICATION message over Xn or by means of the Uplink RAN configuration transfer procedure and Downlink RAN configuration transfer over NG to the node that served the UE before the reported connection failure.

In case the re-connect cell is a cell belonging to an E-UTRAN node where the UE attempts a RRC connection setup procedure, the UE may make the RLF Report available to an E-UTRAN node. The E-UTRAN node may forward the information by means of the ENB configuration transfer procedure over S1 and Downlink RAN configuration transfer over NG to the node that served the UE before the reported connection failure.