3GPP TSG-RAN WG3 #117-e	R3-224421

# Online, 15th – 24th August 2022

Agenda Item:	12.2.1

Source:	Lenovo

Title: 	        (TP for TS38.300 TS38.401) Discussion on general framework

Document for:	Discussion and Approval

# 1	Introduction

In this paper we discuss issues related to high level framework which should be clarified. Also, we propose to add description related to AI training/inference to TS38.300 and TS38.401.

# 2	Discussion

During study item discussion, the scenario, wherein the AI model is trained at OAM and deployed at the NG-RAN node, is supported. However, the specification impact is unclear considering the OAM supporting AI model management depends on SA5 progress and in Rel18 SA5 does plan to study the relevant issues as explained in SP-211443. Thus, RAN3 is suggested to focus on the scenario wherein both AI model training and inference are at NG-RAN node CU.

RAN3 is suggested to focus on the scenario wherein both AI model training and inference are at NG-RAN node CU. RAN3 only considers AI model provided from OAM after enough progress from SA5.

Another aspect needs discussion is which signalling design principle/preference shall be adopted by RAN3 with respect to AI data collection and inference output transfer etc.:

- Option 1: RAN3 introduces a new and unified procedure that can be used for different kinds of AI data collection and inference output transfer (e.g., resource status prediction, trajectory prediction)

- Option 2: RAN3 tries to reuse the existing procedure as much as possible with necessary enhancements.

In our view, both ways can work with pros and cons. Option 1 would require more significant specification changes, while it is more future proof considering possible new type of data in the future. Option 2 has less specification impacts but needs to modify different procedures case by case. We are open to go for either option. It would be good for RAN3 to decide soon enough to facilitate relevant discussion.

RAN3 discusses which principle shall be taken for Stage 3 signalling design

  - Introduce AI dedicated procedure for different kinds of AI data collection and inference output transfer when possible.

  - Reuse existing procedure as much as possible with necessary enhancements.

# 3	Conclusion

Based on the discussion above, we propose:

Proposal 1	RAN3 is suggested to focus on the scenario wherein both AI model training and inference are at NG-RAN node CU. RAN3 only considers AI model provided from OAM after enough progress from SA5.

Proposal 2	RAN3 discusses which principle shall be taken for Stage 3 signalling design

a.	Introduce AI dedicated procedure for different kinds of AI data collection and inference output transfer when possible.

b.	Reuse existing procedure as much as possible with necessary enhancements.

# TP for TS 38.300

**********************************************	    Change Starts    ******************************************************

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

5GC	5G Core Network

5GS	5G System

5QI	5G QoS Identifier

A-CSI	Aperiodic CSI

AGC	Automatic Gain Control

AI	Artificial Intelligence

AKA	Authentication and Key Agreement

AMBR	Aggregate Maximum Bit Rate

AMC	Adaptive Modulation and Coding

AMF	Access and Mobility Management Function

ARP	Allocation and Retention Priority

BA	Bandwidth Adaptation

BCCH	Broadcast Control Channel

BCH	Broadcast Channel

BFD	Beam Failure Detection

BH	Backhaul

BL	Bandwidth reduced Low complexity

BPSK	Binary Phase Shift Keying

C-RNTI	Cell RNTI

CAG	Closed Access Group

CAPC	Channel Access Priority Class

CBRA	Contention Based Random Access

CCE	Control Channel Element

CD-SSB	Cell Defining SSB

CFRA	Contention Free Random Access

CG	Configured Grant

CHO	Conditional Handover

CIoT	Cellular Internet of Things

CLI	Cross Link interference

CMAS	Commercial Mobile Alert Service

CORESET	Control Resource Set

CP	Cyclic Prefix

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

DAG	Directed Acyclic Graph

DAPS	Dual Active Protocol Stack

DFT	Discrete Fourier Transform

DCI	Downlink Control Information

DCP	DCI with CRC scrambled by PS-RNTI

DL-AoD	Downlink Angle-of-Departure

DL-SCH	Downlink Shared Channel

DL-TDOA	Downlink Time Difference Of Arrival

DMRS	Demodulation Reference Signal

DRX	Discontinuous Reception

E-CID	Enhanced Cell-ID (positioning method)

EHC	Ethernet Header Compression

ETWS	Earthquake and Tsunami Warning System

FS	Feature Set

FSA ID	Frequency Selection Area Identity

G-CS-RNTI	Group Configured Scheduling RNTI

G-RNTI	Group RNTI

GFBR	Guaranteed Flow Bit Rate

GIN	Group ID for Network selection

GSO	Geosynchronous Orbit

H-SFN	Hyper System Frame Number

HAPS	High Altitude Platform Station

HRNN	Human-Readable Network Name

IAB	Integrated Access and Backhaul

IFRI	Intra Frequency Reselection Indication

I-RNTI	Inactive RNTI

INT-RNTI	Interruption RNTI

KPAS	Korean Public Alarm System

L2	Layer-2

L3	Layer-3

LDPC	Low Density Parity Check

LEO	Low Earth Orbit

MBS	Multicast/Broadcast Services

MCE	Measurement Collection Entity

MCCH	MBS Control Channel

MDBV	Maximum Data Burst Volume

MEO	Medium Earth Orbit

MIB	Master Information Block

MICO	Mobile Initiated Connection Only

MFBR	Maximum Flow Bit Rate

ML	Machine Learning

MMTEL	Multimedia telephony

MNO	Mobile Network Operator

MPE	Maximum Permissible Exposure

MRB	MBS Radio Bearer

MT	Mobile Termination

MTCH	MBS Traffic Channel

MTSI	Multimedia Telephony Service for IMS

MU-MIMO	Multi User MIMO

Multi-RTT	Multi-Round Trip Time

MUSIM	Multi-Universal Subscriber Identity Module

NB-IoT	Narrow Band Internet of Things

NCD-SSB	Non Cell Defining SSB

NCGI	NR Cell Global Identifier

NCR	Neighbour Cell Relation

NCRT	Neighbour Cell Relation Table

NGAP	NG Application Protocol

NGSO	Non-Geosynchronous Orbit

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

NSAG	Network Slice AS Group

NTN	Non-Terrestrial Network

P-MPR	Power Management Maximum Power Reduction

P-RNTI	Paging RNTI

PCH	Paging Channel

PCI	Physical Cell Identifier

PDC	Propagation Delay Compensation

PDCCH	Physical Downlink Control Channel

PDSCH	Physical Downlink Shared Channel

PEI	Paging Early Indication

PH	Paging Hyperframe

PLMN	Public Land Mobile Network

PNI-NPN	Public Network Integrated NPN

PO	Paging Occasion

PRACH	Physical Random Access Channel

PRB	Physical Resource Block

PRG	Precoding Resource block Group

PRS	Positioning Reference Signal

PS-RNTI	Power Saving RNTI

PSS	Primary Synchronisation Signal

PTM	Point to Multipoint

PTP	Point to Point

PTW	Paging Time Window

PUCCH	Physical Uplink Control Channel

PUSCH	Physical Uplink Shared Channel

PWS	Public Warning System

QAM	Quadrature Amplitude Modulation

QFI	QoS Flow ID

QMC	QoE Measurement Collection

QoE	Quality of Experience

QPSK	Quadrature Phase Shift Keying

RA	Random Access

RA-RNTI	Random Access RNTI

RACH	Random Access Channel

RANAC	RAN-based Notification Area Code

REG	Resource Element Group

RIM	Remote Interference Management

RLM	Radio Link Monitoring

RMSI	Remaining Minimum SI

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RNTI	Radio Network Temporary Identifier

RQA	Reflective QoS Attribute

RQoS	Reflective Quality of Service

RS	Reference Signal

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSSI	Received Signal Strength Indicator

RSTD	Reference Signal Time Difference

RTT	Round Trip Time

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SDT	Small Data Transmission

SFI-RNTI	Slot Format Indication RNTI

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

SMC	Security Mode Command

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

SNPN	Stand-alone Non-Public Network

SNPN ID	Stand-alone Non-Public Network Identity

SPS	Semi-Persistent Scheduling

SR	Scheduling Request

SRAP	Sidelink Relay Adaptation Protocol

SRS	Sounding Reference Signal

SRVCC	Single Radio Voice Call Continuity

SS	Synchronization Signal

SSB	SS/PBCH block

SSS	Secondary Synchronisation Signal

SSSG	Search Space Set Group

SST	Slice/Service Type

SU-MIMO	Single User MIMO

SUL	Supplementary Uplink

TA	Timing Advance

TB	Transport Block

TCE	Trace Collection Entity

TNL	Transport Network Layer

TPC	Transmit Power Control

TRP	Transmit/Receive Point

TRS	Tracking Reference Signal

U2N	UE-to-Network

UCI	Uplink Control Information

UDC	Uplink Data Compression

UE-Slice-MBR	UE Slice Maximum Bit Rate

UL-AoA	Uplink Angles of Arrival

UL-RTOA	Uplink Relative Time of Arrival

UL-SCH	Uplink Shared Channel

UPF	User Plane Function

URLLC	Ultra-Reliable and Low Latency Communications

VR	Virtual Reality

V2X	Vehicle-to-Everything

Xn-C	Xn-Control plane

Xn-U	Xn-User plane

XnAP	Xn Application Protocol

# x	Support for network optimization using AI/ML

## x.1	General

In this release, NG-RAN may use a trained AI/ML model for network optimization in case of network energy saving, load balancing, and mobility optimization.

The following high-level principles should be applied for AI-enabled NG-RAN intelligence:

-	The detailed AI/ML algorithms and models for use cases are implementation specific and out of 3GPP scope.

-	An AI/ML model used in a Model Inference function has to be initially trained, validated and tested by the Model Training function before deployment.

-	User data privacy and anonymisation should be respected during AI/ML operation.

## x.2	Functional Framework

Figure x.2-1. Functional Framework for RAN Intelligence

This section introduces the common terminologies related to the functional framework for NG-RAN intelligence illustrated in Figure 22.2-1. For the functions and data/information flows shown in the Figure 22.2-1. In this release, AI/ML Model Training and AI/ML Model Inference are both located in the gNB. In case of CU DU split architecture, AI/ML Model Training and Model Inference are both located in the gNB-CU, which is further specified in TS 38.401.

-	Data Collection is a function that provides input data to Model training and Model inference functions. AI/ML algorithm specific data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) is not carried out in the Data Collection function.  
Examples of input data may include measurements from UEs or different network entities, feedback from Actor, output from an AI/ML model.

-	Training Data: Data needed as input for the AI/ML Model Training function.

-	Inference Data: Data needed as input for the AI/ML Model Inference function.

-	Model Training is a function that performs the AI/ML model training, validation, and testing which may generate model performance metrics as part of the model testing procedure. The Model Training function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Training Data delivered by a Data Collection function, if required.

-	Model Deployment/Update: Used to initially deploy a trained, validated, and tested AI/ML model to the Model Inference function or to deliver an updated model to the Model Inference function.

-	Note: Details of the Model Deployment/Update process as well as the use case specific AI/ML models transferred via this process are out of RAN3 Rel-17 study scope. The feasibility to single-vendor or multi-vendor environment has not been studied in RAN3 Rel-17 study.

-	Model Inference is a function that provides AI/ML model inference output (e.g., predictions or decisions). Model Inference function may provide Model Performance Feedback to Model Training function when applicable. The Model Inference function is also responsible for data preparation (e.g., data pre-processing and cleaning, formatting, and transformation) based on Inference Data delivered by a Data Collection function, if required.

-	Output: The inference output of the AI/ML model produced by a Model Inference function.

Note:	Details of inference output are use case specific.

-	Model Performance Feedback:  It may be used for monitoring the performance of the AI/ML model, when available.

Note:	Details of the Model Performance Feedback process are out of RAN3 scope.

-	Actor is a function that receives the output from the Model Inference function and triggers or performs corresponding actions. The Actor may trigger actions directed to other entities or to itself.

-	Feedback: Information that may be needed to derive training data, inference data or to monitor the performance of the AI/ML Model and its impact to the network through updating of KPIs and performance counters.

## x.3	AI/ML model training and inference using collected data

## Figure x.3-1: AI/ML model training and inference using collected data

Step 0: the neighbour gNB is assumed to have an AI/ML model optionally, which can provide the serving gNB with input information.

Step 1: The serving gNB configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report(s) to the serving gNB including the required measurement result.

Step 4: The neighbour gNB sends the required input data to serving gNB for model training.

Step 5: The serving gNB trains AI/ML model based on collected data. The neighbour gNB is assumed to have AI/ML model, which can also generate predicted results/actions.

Step 6: The neighbour gNB sends the required input data to the serving gNB for model inference.

Step 7: UE sends the UE measurement report(s) to the serving gNB.

Step 8: Based on local inputs of the serving gNB and received inputs from the neighbour gNB, the serving gNB generates model inference output (e.g., energy saving strategy, handover strategy, etc).

Step 9: The serving gNB executes actions for network energy saving, load balancing, or mobility optimization/handover according to the model inference output.

Step 10: The neighbour gNB provides feedback to the serving gNB.

**********************************************	    Change Ends    ******************************************************

# TP for TS 38.401

**********************************************	    Change Starts    ******************************************************

## 8.x	Overall procedures for network optimization using AI/ML

### 8.x.1	General

The following clauses describe the overall procedures for network optimization involving E1 and F1.

#### 8.x.2	AI/ML model training and inference using collected data

Figure 8.20.2-1. Model Training and Model Inference at NG-RAN

Step 0: gNB CU-CP is assumed to have an AI/ML model optionally, which can provide NG-RAN node 1 with input information.

Step 1: gNB CU-CP configures the measurement information on the UE side and sends configuration message to UE to perform measurement procedure and reporting.

Step 2: The UE collects the indicated measurement(s), e.g., UE measurements related to RSRP, RSRQ, SINR of serving cell and neighbouring cells.

Step 3: The UE sends the measurement report(s) to gNB CU-CP including the required measurement result.

Step 4-5: gNB CU-UP and gNB DU send the required input data to gNB CU-CP for model training.

Step 6: gNB CU-CP trains AI/ML model based on collected data.

Step 7: UE sends the UE measurement report(s) to NG-RAN node 1.

Step 8-9: gNB CU-UP and gNB DU send the required input data to gNB CU-CP for model inference.

Step 10: Based on local inputs of gNB CU-CP and received inputs from gNB CU-UP and gNB DU, gNB CU-CP generates model inference output (e.g., energy saving strategy, handover strategy, etc).

Step 11: gNB CU-CP executes actions for network energy saving, load balancing, or mobility optimization/handover according to the model inference output.

**********************************************	    Change Ends    ******************************************************