3GPP TSG-RAN WG3 #117-e	R3-224423

# Online, 15th – 24th August 2022

Agenda Item:	12.2.2

Source:	Lenovo

Title: 	        Discussion on UE traffic prediction

Document for:	Discussion and Approval

# 1	Introduction

As captured in TR 37.817, for load balancing and mobility optimization, the local NG-RAN node can measure and predict the traffic for a UE. So far, it is assumed the UE traffic prediction is used by a NG-RAN node internally. In this paper, we further analyse the possible specification impact to support the resource status prediction and transfer.

| Network Energy Saving | Load Balancing: | Mobility Optimization |
| --- | --- | --- |
| Input information from local node: - UE mobility/trajectory prediction - Current/Predicted Energy efficiency - Current/Predicted resource status Input from neighbouring NG-NG-RAN nodes: - Current/Predicted energy efficiency - Current/Predicted resource status - Current energy state (e.g., active, high, low, inactive) Output: - Energy saving strategy, such as recommended cell activation/deactivation. - Handover strategy, including recommended candidate cells for taking over the traffic - Predicted energy efficiency - Predicted energy state (e.g., active, high, low, inactive) - Model output validity time will be discussed during R18 normative work per inference output. | Input Information from the local node: - Current and predicted own resource status - UE trajectory prediction - Current and predicted UE traffic - Predicted resource status information of neighbouring NG-NG-RAN node(s) Input Information from neighbouring NG-NG-RAN nodes: - Current and predicted resource status - UE performance measurement at traffic offloaded neighbouring cell Output: - Selection of target cell for load balancing - Predicted own resource status information - Predicted resource status information of neighbouring NG-RAN node(s) - Model output validity time will be discussed during R18 normative work per inference output. - The predicted UE(s) selected to be handed over to target NG-RAN node (will be used by RAN node internally) | Input Information from the local node: - UE trajectory prediction - Current/predicted resource status - Current/predicted UE traffic Input Information from the neighbouring NG-RAN nodes: - UE’s history information from neighbour - Position, QoS parameters and the performance information of historical HO-ed UE (e.g., loss rate, delay, etc.) - Current/predicted resource status - UE handovers in the past that were successful and unsuccessful, including too-early, too-late, or handover to wrong (sub-optimal) cell, based on existing SON/RLF report mechanism. Output: - UE trajectory prediction (Latitude, longitude, altitude, cell ID of UE over a future period of time) - Estimated arrival probability in CHO and relevant confidence interval - Predicted handover target node, candidate cells in CHO, may together with the confidence of the predication - Priority, handover execution timing, predicted resource reservation time window for CHO. - UE traffic prediction (will be used by the RAN node internally and the details are left to normative work phase) - Model output validity time will be discussed during R18 normative work per inference output. |

# 2	Discussion

## 2.1	General

During the Rel17 study item discussion, it is common understanding among companies that UE traffic measurement and prediction could help AI based load balancing and mobility optimization. For example, for better load balancing and mobility optimization decision, a NG-RAN node needs to know

- if the source cell has no longer enough resources to serve the UE traffic as demanded, or

- if any target cell has enough resources to serve the UE as demanded

However, it is not crystal clear what is the meaning of UE traffic. Intuitively, it could be beneficial for a NG-RAN node to know everything about current and future UE traffic including data volume, QoS requirement, and traffic characteristics (e.g., period, jitter).

In legacy, during the QoS flow and data radio bearer establishment, NG-RAN node can understand the QoS requirement of UE traffic and some traffic characteristics as well, such as delay budget, maximum burst, etc. In Rel18 XR work item discussion, companies will study and specify additional traffic characteristics that can be aware by the NG-RAN node such as jitter, importance level, and size of belonging PDU set etc. On the other hand, how can NG-RAN node predict the future UE traffic QoS requirement and characteristic is unclear since that strongly depends on what kind of new application or new service the UE is going to initiate, which NG-RAN node normally doesn’t have the best knowledge.

Comparatively, NG-RAN node could predict the data volume at the UE based on historical measurement conducted at the NG-RAN node, such as the measured amount of data (in bits) in UL/DL in a given time window. Therefore, as a start point, RAN3 could work on supporting the UE traffic data volume prediction.

New QoS requirement and characteristics of future UE traffic depends on UE application layer behavior, for which NG-RAN node doesn’t have the best knowledge.

For UE traffic prediction, as a start point, RAN3 is suggested to discuss the data volume prediction for a certain UE.

## 2.1	Impacts on E1 interface

In legacy, DC scenario, the SN CU-UP could measure and report the data usage for one UE to SN CU-CP over E1 interface and then from SN CU-CP to MN CU-CP over Xn interface for charging purpose as specified in TS 23.501. The usage data reporting is triggered either by OAM, some specified events, or periodically as instructed by OAM.

TS 23.501 ### 5.12.2 Usage Data Reporting for Secondary RAT When NG-RAN is deployed in dual connectivity configuration, the HPLMN or VPLMN operator may wish to record the data volume sent and received on the Secondary RAT. In order to reduce the complexity of this procedure, the following principles are used in this release: a) The PLMN locally activates the Secondary RAT Usage Data Reporting by NG-RAN OAM. The activation is based on configuration in NG-RAN and NG-RAN determines whether the data volume report will contain data volumes consumed for the whole PDU Session or for selected QoS flows or both as described in TS 38.413 [34]. The activation can happen separately for Data Volume Reporting of NR in licensed or unlicensed spectrum and E-UTRA in licensed or unlicensed spectrum. If the PLMN uses this feature, it should ensure that this functionality is supported by all NG-RAN nodes that support NR or E-UTRA as a Secondary RAT. b) Depending on its configuration the NG-RAN reports uplink and downlink data volumes to the 5GC for the Secondary RAT (including the using of unlicensed spectrum for NR or E-UTRA) for the PDU Session or for selected QoS flows or both and per time interval. c) During Xn handover and N2 handover, the source NG-RAN node reports the data volume to the 5GC. The reported data volume excludes data forwarded to the target RAN node. d) At the time of NG connection release, Secondary Node change/release, deactivation of UP connection for a PDU Session, the NG-RAN node reports the data volumes to the 5GC. e) To assist "partial CDR" generation, NG-RAN OAM can instruct the NG-RAN to also make periodic reports (as described in clause 5.12.3) if no event has triggered a report before the period expires. NOTE 2: The timing of these periodic NG-RAN reports is not expected to align with the timing of partial CDR generation. Hence the frequency of NG-RAN reports might be greater than that of partial CDR generation. NOTE 3: RAN needs to be able to partition the measurements in a report to indicate usage that occurred before and after an absolute time. An example of the absolute time is that RAN is configured to partition data usage reports that occurred before and after midnight. ### 5.12.3 Secondary RAT Periodic Usage Data Reporting Procedure Periodic reporting of the Secondary RAT usage data is an optional function. When NG-RAN, as defined in bullet e) of clause 5.12.12, is configured with a "time interval for Secondary RAT usage data reporting", the NG-RAN shall send a RAN Usage Data Report message for periodic reporting purposes to the SMF only when the timer expires for a UE for which Secondary RAT usage data reporting is ongoing. The timer runs from the last usage reporting for the UE. ### TS 37.483 ### 8.3.11 MR-DC Data Usage Report #### 8.3.11.1 General This procedure is initiated by the gNB-CU-UP to report data volume served at the gNB-CU-UP, where the UE is connected to the 5GC or EPC. The procedure uses UE-associated signalling. #### 8.3.11.2 Successful Operation Figure 8.3.11.2-1: MR-DC Data Usage Report procedure: Successful Operation. The gNB-CU-UP initiates the procedure by sending the MR-DC DATA USAGE REPORT message to the gNB-CU-CP.

During Rel17 study item, companies agreed that for standalone non-DC scenario, the predicted UE traffic will be used internally at the gNB, i.e., does not send to any neighbour gNB. Thus, RAN3 could at least discuss how to support UE traffic prediction within one gNB, especially between CU-CP and CU-UP considering the CP UP split architecture.

In legacy, usage data reporting procedure (i.e., containing measured data volume) over E1/Xn interface is supported for charging purpose in dual connectivity scenario as instructed by core network.

RAN3 discusses how to support UE data volume prediction between CU-CP and CU-UP over E1 interface.

To understand possible E1 interface impact, RAN3 needs to first discuss which node will perform the AI inference and generate the predicted UE data volume, i.e., by CU-CP or CU-UP. Both are possible with different implications in our view:

- Option 1: CU-CP could predict UE data volume based on measured data volume report received from CU-UP

- Option 2: CU-UP could predict UE data volume based on its own measurement and send the predicted UE data volume to CU-CP upon request

Option 1 does not require any AI capability for CU-UP and allows CU-CP to predict the future UE data volume using information collected from other sources other than CU-UP, e.g., traffic characteristics from UE or core network. On the other hand, Option 1 would require CU-UP to provide the measured data volume to CU-CP as demanded for the sake of AI inference. Note that current Data Usage reporting procedure over E1 interface follows instruction from OAM. Thus, to support option 1 enhancement is needed for CU-UP to provide measured data volume as demanded by CU-CP, e.g., upon request from CU-CP.

Comparatively, option 2 would require CU-UP to perform AI inference, and CU-UP has better knowledge of actual UE data volume than CU-CP, which can be used for the prediction. To support option 2 enhancement is needed for CU-UP to send the predicted UE data volume to CU-CP upon request.

No matter which option is supported, it seems some request/response procedure (similar as the resource status request/response procedure) is necessary for the CU-UP to provide the UE data volume measurement or prediction as CU-CP requested. And to distinguish with the legacy Data Usage Report procedure for DC scenario specially, RAN3 can introduce a new procedure, e.g., UE Traffic Report procedure, for the CU-UP to provide UE traffic related measurements/predictions to CU-CP as requested.

Figure 1: Example of CU-UP providing data volume measurement/prediction as CU-CP requested

RAN3 discusses if UE data volume can be predicted at CU-CP or CU-UP, or both.

RAN3 introduces new E1AP procedure for the CU-UP to provide UE data volume measurement/prediction to CU-CP as requested by CU-CP, e.g., UE Traffic Report procedure.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	New QoS requirement and characteristics of future UE traffic depends on UE application layer behavior, for which NG-RAN node doesn’t have the best knowledge.

Observation 2	In legacy, usage data reporting procedure (i.e., containing measured data volume) over E1/Xn interface is supported for charging purpose in dual connectivity scenario as instructed by core network.

Based on the discussion above, we propose:

Proposal 1	For UE traffic prediction, as a start point, RAN3 is suggested to discuss the data volume prediction for a certain UE.

Proposal 2	RAN3 at least discusses how to support UE data volume prediction between CU-CP and CU-UP over E1 interface.

Proposal 3	RAN3 discusses if UE data volume can be predicted at CU-CP or CU-UP, or both.

Proposal 4	RAN3 introduces new E1AP procedure for the CU-UP to provide UE data volume measurement/prediction to CU-CP as requested by CU-CP, e.g., UE Traffic Report procedure.

# Reference