3GPP TSG-RAN WG3 #117-e	R3-224427

# Online, 15th – 24th August 2022

Agenda Item:	12.3

Source:	Lenovo

Title: 	        Discussion on AI for NR DC scenario

Document for:	Discussion and Approval

# 1	Introduction

As captured in TR37.817, companies agreed that NG-RAN standalone is prioritized for the AI assisted use cases, while EN-DC and MR-DC is down prioritized but not precluded in Rel18 discussion. In this paper, we further discuss the applicability of AI capability in DC scenario.

TR 37.817 ## 4.1 High-level Principles The following high-level principles should be applied for AI-enabled RAN intelligence: - The detailed AI/ML algorithms and models for use cases are implementation specific and out of RAN3 scope. - The study focuses on AI/ML functionality and corresponding types of inputs/outputs. - The input/output and the location of the Model Training and Model Inference function should be studied case by case. - The study focuses on the analysis of data needed at the Model Training function from Data Collection, while the aspects of how the Model Training function uses inputs to train a model are out of RAN3 scope. - The study focuses on the analysis of data needed at the Model Inference function from Data Collection, while the aspects of how the Model Inference function uses inputs to derive outputs are out of RAN3 scope. - Where AI/ML functionality resides within the current RAN architecture, depends on deployment and on the specific use cases. - The Model Training and Model Inference functions should be able to request, if needed, specific information to be used to train or execute the AI/ML algorithm and to avoid reception of unnecessary information. The nature of such information depends on the use case and on the AI/ML algorithm. - The Model Inference function should signal the outputs of the model only to nodes that have explicitly requested them (e.g., via subscription), or nodes that take actions based on the output from Model Inference. - An AI/ML model used in a Model Inference function has to be initially trained, validated and tested by the Model Training function before deployment. - NG-RAN SA is prioritized; EN-DC and MR-DC are down-prioritized, but not precluded from Rel.18. - Functional framework and high-level procedures defined in this TR should not prevent from “thinking beyond” them during normative phase if a use case requires so. - User data privacy and anonymisation should be respected during AI/ML operation.

# 2	Discussion

In DC scenarios, the MN or SN could be either LTE or NR RAN node, however, it is questionable to us if companies really want 4G LTE base station to support AI training and inference capability, since 4G base station has been widely deployed and there might be backward compatibility risk to adopt AI/ML scheme for network operation. Besides, most of the conclusions from the NG-RAN SA scenario discussion can be inherited in the NR DC scenario, such as the support of resource status prediction, UE traffic prediction, and UE trajectory prediction etc. Therefore, RAN3 should at least support the AI assisted use cases (i.e., network energy saving, load balancing, mobility optimization) in NR-DC scenario, which does not require much extra work in our view. In parallel to the NG-RAN SA discussion, RAN3 could also discuss issues which are NR-DC specific.

The motivation to support AI training/inference capability in 4G LTE base station is questionable.

Most of the conclusions from NG-RAN node stand alone scenario can be inherited for the NR-DC scenario.

RAN3 is suggested to support AI assisted use cases in NR-DC scenario in Rel18.

In terms of NR-DC specific issues, we believe there are two aspects which are different from NG-RAN SA scenario and should be discussed. First, in NR-DC scenario, the MN and SN are serving the same UE at the same time, thus, there could be a need for MN and SN to share some UE related prediction result such as predicted UE trajectory and UE traffic. For example:

- The SN could make prediction about future PSCells that UE will move to and provide to MN upon request. The prediction result can be used by the MN for possible SN change decision.

- Similarly, the SN could make prediction about UE traffic over SN terminated QoS flows and provide to MN upon request. The prediction result can be used by the MN for possible SN modification, e.g., change the SN terminated QoS flow to MN terminated QoS flow.

The exact Xn signalling to support such UE related prediction request and provision can be further discussed. It can be based on existing Xn signalling such as SN Modification Request/Required, or new signalling.

Figure 1: SN provides UE related prediction upon MN request

In NR-DC scenario, the MN/SN can make UE related prediction (e.g., UE trajectory and UE traffic) and send to the peer SN/MN upon request.  FFS the Xn signalling to be used.

In paper [1], we explained why we believe it is beneficial for the source NG-RAN node to provide predicted UE trajectory or UE traffic to the target NG-RAN node in a handover scenario. Similarly, in NR-DC mobility scenario, such as SN addition and SN change, it is also considered beneficial to provide the predicted UE trajectory or UE traffic to the target SN for future target SN operation. In particular, during SN addition or SN change, the MN can include the predicted UE trajectory or UE traffic in the SN ADDITION REQUEST Xn message to the target SN. The predicted UE traffic could be the predicted data volume over the QoS flows that MN wants the target SN to support. The predicted UE trajectory could be the predicted future PSCells that UE will move to.

Figure 2: MN provides UE related prediction during SN addition

Figure 2: MN provides UE related prediction during SN change

During SN addition or SN change, the MN can provide UE related prediction (e.g., UE trajectory and UE traffic) to the target SN in the SN addition request message.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	The motivation to support AI training/inference capability in 4G LTE base station is questionable.

Observation 2	Most of the conclusions from NG-RAN node stand alone scenario can be inherited for the NR-DC scenario.

Based on the discussion above, we propose:

Proposal 1	RAN3 is suggested to support AI assisted use cases in NR-DC scenario in Rel18.

Proposal 2	In NR-DC scenario, the MN/SN can make UE related prediction (e.g., UE trajectory and UE traffic) and send to the peer SN/MN upon request.  FFS the Xn signalling to be used.

Proposal 3	During SN addition or SN change, the MN can provide UE related prediction (e.g., UE trajectory and UE traffic) to the target SN in the SN addition request message.

# Reference

[1] R3-224425	Discussion on prediction and feedback transfer in mobility scenario	Lenovo