3GPP TSG-RAN WG3 Meeting #117-e	R3-224458

E-meeting, 15 – 24 August, 2022

Agenda item:	11.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On support for QMC in NR-DC

Document for:	Discussion and Decision

# 1	Introduction

In this paper we provide initial discussion on support for QMC in NR-DC. The discussion takes into account the objectives of the Rel-18 QMC enhancements work item (WID in RP-221803) and earlier conclusions from Rel-17 study item captured in TR 38.890.

# 2	Discussion

The following objectives are captured in the WID, on top of which we've added a number O1-O4 to identify each of the sub-objectives:

- Specify to support for QoE in NR-DC, e.g. enable QoE reporting via SN [RAN3, RAN2].

- O1: Specify the QoE configuration, and measurement reporting over MN/SN for NR-DC architecture, and specify the QoE measurement reporting over the other DC leg in order to maintain the reporting continuity.

Note 1: The QoE measurements are not performed separately for each leg.

- O2: Support RAN-visible QoE and radio related measurement configuration and reporting in NR-DC scenarios.

- O3: Specify the QoE measurement continuity in mobility scenarios in NR-DC.

- O4: Specify the alignment of QoE measurements (including legacy QoE and RAN visible QoE measurements) and radio related measurement in NR-DC.

These objectives are covered as follows in the justification section of the WID: "As NR-DC is one of the important commercial deployment scenarios for 5G networks, it is critical to support it in the NR QoE framework. Therefore, Rel-18 QoE should address the support for QoE measurement configuration, measurement reporting over MN/SN for NR-DC scenario, as well as support for RAN visible QoE report over MN/SN, mobility continuity, the alignment of QoE report and MDT, etc.".

O1: encapsulated QMC configuration and reporting

O1 relates to QoE configuration and reporting encapsulated in SA4-defined QMC containers whose contents are not visible to the RAN or to the AS layer in the UE. Encapsulated QMC configuration is today supported over the MN leg only (SRB1), and it would be up to RAN2 decision to support encapsulated QMC configuration over the SN leg (SRB3). Still, absence of support of QMC configuration on SRB3 doesn't mean that NR-DC is not supported in the NR QoE framework, considering that DC (in the AS layer) and QMC (in the application layer) are orthogonal features. Also Note 1 hints that the UE application layer should remain unaware of DC operation. Therefore, any support of QMC configuration over SRB3 would need a more detailed justification. E.g., when it comes to QMC configuration mechanisms, it is clear that current standard doesn't support m-based QMC activation in the SN, on the other side we don't believe QoE reporting could be limited to DC operation without DC awareness in the application layer. And we don't believe it would be worthwhile to support mechanisms for QoE measurement continuity in case of SN-triggered QMC, which would probably also end up in triggering QMC in non-DC scenarios. So in our view, an operator who needs to check obtained QoE specifically in DC operation will configure the relevant MN nodes for QMC. However, as discussed during Rel-17 study item phase, monitoring of DC impact can still be solved by the RAN reporting radio-related information and hence informing the MCE about start and stop of DC operation (TR 38.890: "Besides radio-related measurement results, radio-related information may also be reported."). We also assume that CA operation could be reported via such mechanism.

Proposal 1: M-based QMC activation in the SN will not provide sufficient added value to be supported by the standard.

Proposal 2: RAN should inform the MCE about DC and CA operation as part of reported radio-related information.

If these proposals can constitute the basis for RAN3's work on O1, introduction of encapsulated QMC configuration on SRB3 would probably not have any other justification than any potential need for load balancing of signalling over MN/SN legs. It is in our view questionable whether load balancing of configuration information is worthwhile, but this should be up to RAN2 and we believe that RAN3 would not have any particular recommendation in this domain.

Observation: Load balancing of MN/SN legs would be the only reason to support encapsulated QMC configuration on SRB3.

Proposal 3: It is up to RAN2 whether to introduce encapsulated QMC configuration on SRB3.

Similarly, when it comes to encapsulated QoE reporting, it is today supported over the MN leg only (SRB4) so the MN leg today therefore needs to support QoE reporting also for services which the UE receives via the SN leg. Some services could also be provided via split DRBs, hence over both MN and SN legs. From a higher-layer (MCE) point of view, encapsulated QMC would work in the same way regardless of whether encapsulated QoE reports are sent via the MN leg or the SN leg, however in the latter case RAN3 would need to decide whether the SN forwards the QoE reports directly to the MCE, or whether they are sent via the MN (over Xn). Our preference would be to reduce the impact on the RAN by sending encapsulated QoE reports received on the SN-leg directly from the SN to the MCE.

Observation: Load balancing of MN/SN legs would be the only reason to support encapsulated QMC reporting on SRB3.

Proposal 4: It is up to RAN2 whether to introduce encapsulated QMC reporting over the SN leg.

O2: RVQoE configuration and reporting

O2 relates to QoE configuration and reporting via RRC-encoded IEs (aka RVQoE), hence visible to both the UE access stratum and the RAN (gNB). In the UE in DC operation, neither the application layer nor the access stratum would be able to map which leg (MN or SN) that provides service for which application. We therefore believe that RAN3's approach on O2 can follow the same approach as for O1, i.e. RAN3 lets RAN2 decide whether load balancing reasons would justify signalling of RVQoE configuration or reporting over the SN leg.

Proposal 5: It is up to RAN2 whether to introduce RVQoE configuration on SRB3.

Proposal 6: It is up to RAN2 whether to introduce RVQoE reporting over the SN leg.

Depending on RAN2 decisions, RAN3 may need to discuss network signalling support, e.g.:

- the SN informs the MN about desired RVQoE configuration (for further forwarding to the UE), e.g. in case QMC configuration is decided not to be supported on SRB3.

- mechanism to determine whether the received RVQoE report info is relevant for the SN and corresponding procedures for forwarding over Xn and/or F1. The simplest would be to always forward the RVQoE info to the DU (in the MN or in the SN) that is responsible for scheduling of the DRB, or to the configured DUs (in the MN, SN or in both MN and SN) in case of DC. (Assumption that the UE reports the DRB id(s) together with RVQoE report).

O3: QoE measurement continuity in mobility scenarios in NR-DC

Rel-17 measurement continuity is sufficient, no enhancement needed for NR-DC.

Proposal 7: Rel-17 measurement continuity is sufficient, no enhancement needed for NR-DC.

O4: Alignment of radio related measurement and QMC in NR-DC

In our understanding, the term "radio related measurement" under this sub-objective includes both legacy MDT measurements and information about DC activation/deactivation as discussed under O1 / proposal 2 above.

For discussion on QMC/MDT alignment in NR-DC we propose to use as starting points the principles agreed in earlier releases for QMC and MDT, i.e.:

- TS 38.300: "NG-RAN node can activate the MDT measurements that are to be aligned with the QoE measurements performed by the UE upon/after receiving the Session Start Indication from the UE.".

- TS 37.320: "In signalling based immediate MDT, MME provides MDT configuration for both MN and SN towards MN […]".

- TS 37.320: "In management-based immediate MDT, OAM provides the MDT configuration to both MN and SN independently."

The following combinations are supported:

- s-based QMC / s-based MDT: We believe the most important aspect is to provide observability of QoE changes that could be linked to DC activation/deactivation (see our discussion under O1), and impacts for this could be in the MN only. However also classical QMC/SN-MDT alignment seems straightforward, i.e. legacy SN-MDT activation / deactivation can be sent on Xn upon QMC session start / end.

- m-based QMC / m-based MDT: Also here we believe the most important aspect is to provide observability of QoE changes that could be linked to DC activation/deactivation, and impacts for this will be in the MN only. Then, whether and how to support alignment of legacy m-based SN-MDT with QMC can be discussed with lower priority.

Proposal 8: For s-based QMC / s-based SN-MDT alignment, SN-MDT activation / deactivation can be sent on Xn upon QMC session start / end.

Proposal 9: M-based QMC / m-based SN-MDT alignment can be handled with lower priority.

# 3	Conclusion

We have made the following proposals:

Proposal 1: M-based QMC activation in the SN will not provide sufficient added value to be supported by the standard.

Proposal 2: RAN should inform the MCE about DC and CA operation as part of reported radio-related information.

Proposal 3: It is up to RAN2 whether to introduce encapsulated QMC configuration on SRB3.

Proposal 4: It is up to RAN2 whether to introduce encapsulated QMC reporting over the SN leg.

Proposal 5: It is up to RAN2 whether to introduce RVQoE configuration on SRB3.

Proposal 6: It is up to RAN2 whether to introduce RVQoE reporting over the SN leg.

Proposal 7: Rel-17 measurement continuity is sufficient, no enhancement needed for NR-DC.

Proposal 8: For s-based QMC / s-based SN-MDT alignment scenario, SN-MDT activation / deactivation can be sent on Xn upon QMC session start / end.

Proposal 9: M-based QMC / m-based SN-MDT alignment scenario can be handled with lower priority.