3GPP TSG-RAN WG3 Meeting #117-e	R3-224461

E-meeting, 15 – 24 August 2022

Agenda item:	10.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion related to RACH Report retrieval methods

Document for:	Discussion and Decision

# 1	Introduction

SON/MDT for Rel-18 WI, agreed in RP-221825 defines the following objectives:

- Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PScell change report

- Successful Handover Report (e.g. inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

In RAN3 #112-e the following agreement related to RACH Report retrieval was made:

Postpone enhancements for RACH Report retrieval to Rel.18

Next, we discuss our views on RACH Report retrieval aspects.

# 2	Discussion

A UE may log up to 8 RACH Report entries, attempted on the same or different cell of one or more gNBs. Therefore, it is very difficult, if not impossible, for the network to have a clear view on the number of entries a UE has logged into its RACH Report. This information is available at the UE.

Observation 1: Network cannot have a clear view of the number of entries a UE has logged in the RACH Report.

Observation 2: RACH Report is an internal variable in the UE and thus it is the UE that can determine the exact number of entries logged.

Signalling can be optimized if a trigger for retrieval of a RACH Report by the network is sent when the report is full (all 8 entries have been logged). Otherwise, unnecessary availability indication needs to be sent to trigger this retrieval. Since UE is the one that knows exactly how many entries it has logged, we think that RACH Report retrieval is a topic that needs to be handled first in RAN2.

Proposal 1: RACH Report retrieval shall be handled first by RAN2.

Proposal 2: Network-based solutions may be further considered in a second step, if needed.

# 3	Conclusion

We make the following observations and proposals:

Observation 1: Network cannot have a clear view of the number of entries a UE has logged in the RACH Report.

Observation 2: RACH Report is an internal variable in the UE and thus it is the UE that can determine the exact number of entries logged.

Proposal 1: RACH Report retrieval shall be handled first by RAN2.

Proposal 2: Network-based solutions may be further considered in a second step, if needed.